3GPP TSG RAN WG4 Meeting #116-bis	R4-2513410

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			6.11.2

Source:	vivo

Title:	Discussion on general aspects for AI/ML for NR air interface

Document for:	Discussion

- Introduction

Regarding the general aspects of AI/ML, it mainly includes content on BM and CSI prediction. In this contribution, we provide our views for the issue on monitoring report delay for BM and CSI prediction and the issue on activation delay requirements. The latest WF was captured in [1][2].

- Discussion

  - Performance Monitoring verification for BM

LCM for beam management: Agreement: - Do not define performance monitoring delay requirements for LCM purpose - If the monitoring delay is identificated as critical in the future, this issue can be revisited. - No agreements on performance monitoring accuracy.

Regarding the performance monitoring mechanism, it is designed to monitor the prediction performance of the model. A key function it serves is to determine whether changes in factors affecting the model's prediction performance (such as environmental/signal quality variations) can be reflected in the monitoring metrics reported by the UE. If relevant tests for performance monitoring are not defined, there is no guarantee of the mechanism's robustness in real-world environments. Based on the above considerations, we believe a method to evaluate UE performance monitoring behavior is as follows:

Configure two different environmental conditions during testing and verify: under two sets of different testing conditions (e.g., one set of condition specifically those causing a rapid decline in prediction performance), whether UE will notice this change and the changes of the prediction performance will be reflected through the monitoring metric calculated by UE.

Proposal 1: For Performance Monitoring verification, one way is to design a test to verify

- Under two sets of different testing conditions, whether UE will notice this change and the changes of the prediction performance will be reflected through the monitoring metric calculated by UE.

  - Monitoring reporting delay and accuracy requirements for CSI prediction

Issue 1-5: Scheduling delay Agreement: - The delay from when the UE prediction is send until the time the TE applies is n+4 (slots) for FDD Issue 1-1: Performance monitoring Agreement: RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction FSS which requirements are to be introduced. Issue 1-2: Requirement baseline for monitoring Agreement: Introduce the following requirements for CSI performance monitoring: - Reporting delay - FFS how the delay is defined - Reporting accuracy - FFS on whether accuracy requirement can be defined/checked - Mapping table – To be checked whether RAN1 captures or RAN4 - If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this Issue 1-4: Reporting delay requirement Agreement: - wait for RAN1 decision

Currently, RAN1 has finished the R19 WI of CSI prediction. Some RAN1 LCM agreements have been achieved as following.

RAN1#120bis Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric. Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation. RAN1#121 Agreement For CSI prediction using UE-side model, for UE assisted performance monitoring, - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - For monitoring report type, semi-persistent and aperiodic are supported - The following combination for inference report type and monitoring report type are supported Monitoring report type Inference report type; SP report; AP report. AP report; Not support; Support. SP report; Support; Support. - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported. Agreement - For CSI prediction using UE-side model, for UE assisted performance monitoring, - If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report; - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report - If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, if any, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

Currently, RAN1 has agreed that UE assisted performance monitoring is based on Type 3 performance monitoring with SGCS reporting.

In Type 3 performance monitoring, SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. Thus, the delay for performance monitoring is only for one instance.

The SGCS is reported in wideband frequency granularity, only for one prediction instance configured by NW, and per layer, and the total number of layers is the same as the reported RI in the associated inference report.

RAN4 has agreed to introduce requirements for Type 3 performance monitoring for UE assisted performance monitoring of CSI prediction. To test the reporting accuracy of SGCS 1 and SGCS 2, extra non-prediction CSI report is needed. For SGCS 1, the NW would obtain the predicted CSI in current CSI-RS prediction report, and then the ground truth CSI in the CSI-RS non-prediction report for next CSI-RS transmission. For SGCS 2, the NW would obtain the CSI (non-predicted) in CSI-RS non-prediction report for current CSI-RS transmission, and then the ground truth CSI in the CSI-RS non-prediction report for next CSI-RS transmission.

Currently, the reporting accuracy of PMI has not been defined in RAN4 RRM, while RAN4 Demodulation only has performance test of PMI. RAN4 would need to further discuss this SGCS reporting accuracy based on non-prediction CSI report.

Proposal 2: For Type 3 performance monitoring for CSI prediction, RAN4 to discuss reporting accuracy requirements for SGCS 1 and SGCS 2.

- For accuracy metrics for SGCS 1 and SGCS 2, extra non-prediction CSI report would be needed for TE to obtain the ground truth CSI, and CSI (non-predicted).

- SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI.

- SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

RAN4 does not define CSI reporting RRM requirements for PMI/CQI/RI etc. For the reporting delay and measurement requirement, since CSI prediction reporting and Beam spatial/temporal prediction reporting are both belong to CSI report, similar reporting framework would be reused for these use cases. Thus, reporting delay requirements and measurement requirements of CSI prediction monitoring could largely reuse the approach of Beam management monitoring.

Proposal 3: For Type 3 performance monitoring for CSI prediction, RAN4 to define reporting delay requirements and measurement requirements, which could largely reuse the approach of Beam management monitoring.

  - Activation delay requirement

Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

In the previous meeting regarding the activation delay requirement, RAN4 reached the above conclusion based on the feedback in the reply LS[3] from RAN2. The content of RAN2's reply LS is as follows.

RAN2 LS RAN2 thanks RAN4 for the LS on AI/ML functionality activation. RAN2 discussed the two interpretations provided by RAN4 and concluded that it is necessary to clarify the following RAN2 understanding: Agreements - RRC processing delay shouldn’t be impacted by the model loading delay - If the UE is ready for inference (i.e. the model is ready to generate inference result based on configuration) by end of RRC processing delay, it reports configuration applicable. If not, it reports configuration inapplicable and doesn’t set the release flag. The network is not expected to release inference configuration. - Once the configuration is applicable (i.e. the model is ready to generate inference result based on configuration), UE reports applicability to network via UAI (applicable to all AI/ML based CSI reporting). - On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation from RAN2 point of view and no further RAN2 work. - RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration, from RAN2 point of view.

According to RAN2's conclusion, two scenarios will be addressed:

- Scenario 1: The UE is ready for inference at the end of the RRC processing delay and reports the configuration as applicable. In other words, by the time the UE sends the RRCreconfigurationcomplete message, model loading has already been completed.

- Scenario 2: The UE is not ready for inference at the end of the RRC processing delay (possibly because model loading has not been completed). In this case, the UE will first report the configuration as inapplicable when sending the RRC reconfigurationcomplete message. Later, once the configuration becomes applicable, the UE will report its applicability to the network via UAI.

In our understanding, the conclusion reached by RAN4 is only based on Scenario 1. For Scenario 2, when sending the RRCreconfigurationcomplete message, the UE may not be ready to start measurements, which will introduce additional time—from the time point when the UE first reports the configuration as inapplicable in the RRCreconfigurationcomplete message to the subsequent time point when the UE reports its applicability to the network via UAI once the configuration becomes applicable.

Meanwhile, in the last meeting, RAN2 also sent an LS[4] to RAN1 regarding the issue of when the RRC layer submits the periodic CSI inference configuration (i.e., CSI-ReportConfig) to the lower layer (i.e., the PHY layer). The main content is as follows:

RAN2 LS In more details, RAN2 identified the following two options on when RRC layer submits periodic CSI inference configuration (i.e. CSI-ReportConfig) to lower layer (i.e. PHY layer): - Option 1: Upon reception of RRC Reconfiguration message, UE’s RRC layer immediately submits inference configuration of periodic CSI to lower layer, regardless of whether the configuration is applicable/inapplicable. - Option 2: Upon reception of RRC Reconfiguration message, UE’s RRC layer submits inference configuration of periodic CSI to lower layer only if it is reported as applicable in RRCReconfigurationComplete. From RAN2 point of view, the consequence of the two options can be described as follows: - For Option 1, companies in RAN2 think there may be two different potential approaches in PHY layer how to handle non-applicable periodic CSI-ReportConfig. It is for RAN1 information. - Approach 1: The UE’s PHY layer will immediately perform inference of periodic CSI, even if the inference configuration is non-applicable. Consequently, the UE may report invalid periodic CSI before the corresponding CSI-ReportConfig becomes applicable. - Approach 2: The UE’s PHY layer will ignore the inference configuration of periodic CSI if it is non-applicable. Consequently, the UE will not report periodic CSI before the corresponding CSI-ReportConfig becomes applicable. - For Option 2, as RRC layer holds on submitting the inference configuration to lower layer until reporting as applicable, the UE’s PHY layer will perform inference of periodic CSI and report only after sending RRCReconfigurationComplete with the corresponding CSI-ReportConfig setting to “applicable”. From RAN2 point of view, this issue can be solved by option 2 but needs to check with RAN1. RAN2 also discussed option 1 and couldn’t conclude as its consequence is outside scope of RAN2. RAN2 would like to ask RAN1 which option (i.e. option 1 or option 2) is best.

For the two options and two approaches mentioned above, the main differences lie in when the RRC layer submits the periodic CSI inference configuration to the lower layer and whether the UE's PHY layer will immediately perform inference upon receiving the inference configuration or wait until the CSI-ReportConfig is set to "applicable". However, regardless of the option, for scenario 2, there will be a time gap when the first received inference configuration is not applicable. During this time, the UE continues to prepare for model activation-related task until it reports via UAI that the configuration is applicable. This time gap should be considered part of the functionality activation delay. RAN4 needs to define the requirement for this delay; otherwise, it cannot be guaranteed how long the UE will take to prepare for inference after receiving the RRC Reconfiguration message. According to our understanding, this activation delay can be defined as:

- From the reception of the RRC Reconfiguration message to the UE reporting that the configuration is applicable.

Observation 1: For the following scenario mentioned in RAN2, when sending the RRCreconfigurationcomplete message, the UE may not be ready to start measurements, which will introduce additional time gap—from the time point when the UE first reports the configuration as inapplicable in the RRCreconfigurationcomplete message to the subsequent time point when the UE reports its applicability to the network via UAI once the configuration becomes applicable.

- If the UE is not ready for inference by end of RRC processing delay, it reports configuration inapplicable and once the configuration is applicable (i.e. the model is ready to generate inference result based on configuration), UE reports applicability to network via UAI (applicable to all AI/ML based CSI reporting)

Proposal 4: RAN4 to define the activation delay to verify how long it takes for the UE to be ready for inference after receiving the RRC Reconfiguration message. The starting point and ending point of this delay are as follows:

- Upon the reception of the RRC Reconfiguration message to the UE reporting that the configuration is applicable (via RRCReconfigurationComplete or via UAI).

- Summary

Proposal 1: For Performance Monitoring verification, one way is to design a test to verify

- Under two sets of different testing conditions, whether UE will notice this change and the changes of the prediction performance will be reflected through the monitoring metric calculated by UE.

Proposal 2: For Type 3 performance monitoring for CSI prediction, RAN4 to discuss reporting accuracy requirements for SGCS 1 and SGCS 2.

- For accuracy metrics for SGCS 1 and SGCS 2, extra non-prediction CSI report would be needed for TE to obtain the ground truth CSI, and CSI (non-predicted).

- SGCS 1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI.

- SGCS 2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

Proposal 3: For Type 3 performance monitoring for CSI prediction, RAN4 to define reporting delay requirements and measurement requirements, which could largely reuse the approach of Beam management monitoring.

Observation 1: For the following scenario mentioned in RAN2, when sending the RRCreconfigurationcomplete message, the UE may not be ready to start measurements, which will introduce additional time gap—from the time point when the UE first reports the configuration as inapplicable in the RRCreconfigurationcomplete message to the subsequent time point when the UE reports its applicability to the network via UAI once the configuration becomes applicable.

- If the UE is not ready for inference by end of RRC processing delay, it reports configuration inapplicable and once the configuration is applicable (i.e. the model is ready to generate inference result based on configuration), UE reports applicability to network via UAI (applicable to all AI/ML based CSI reporting)

Proposal 4: RAN4 to define the activation delay to verify how long it takes for the UE to be ready for inference after receiving the RRC Reconfiguration message. The starting point and ending point of this delay are as follows:

- Upon the reception of the RRC Reconfiguration message to the UE reporting that the configuration is applicable (via RRCReconfigurationComplete or via UAI).

- References

- R4-2511849, WF on NR_AIML_air_part2, Ericsson

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R2-2506520, Reply LS on AI/ML functionality activation, Apple

- R2-2506519, LS on when RRC layer submits periodic CSI inference configuration to lower layer, Apple