3GPP TSG-RAN WG4 Meeting #116bis	R4-2513422

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	6.11.5

Source:	Huawei, HiSilicon

Title:	Discussion on RRM core requirement and testing framework for AIML positioning

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #115[1], we discuss the remaining issues related to the RRM core requirement for AIML positioning.

- Reporting related requirements

- Measurement reporting as model input

RAN1 #116 Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

According to RAN1 progress, timing information, paired timing information and power information are supported for case 3b. Whether to support phase information as model input is still under discussion. If there is no other information than time and power introduced as model input, RAN4 can discuss whether to reuse legacy requirement for reporting of timing information or timing and power information from gNB to LMF.

Proposal 1: For Case 3b, RAN4 to reuse the legacy requirement for reporting of timing information or timing and power information from gNB to LMF.

- Model output for assisted positioning

For case 1, the model output does not need to be specified as it is direct positioning, and the positioning report is the calculated location.

For case 3a, if timing information is reported from gNB, LMF may need to know whether the timing information is obtained by legacy measurement-based method or by R19 AIML nonmeasurement-based method. Otherwise, LMF will discard this timing information if it is reported with NLoS indicator. To solve this problem, several options are under discussion in RAN1. One is to explicitly add an indicator to distinguish whether the timing information is obtained by legacy method or by Rel-19 AI/ML. Another is to reuse the timing quality in legacy to implicitly indicate LMF whether to discard or not.

Proposal 2: For Case 3a, RAN4 to discuss whether to reuse the legacy requirement for reporting of timing information from gNB to LMF, if there is no explicit indicator introduced to distinguish whether the timing information is obtained by legacy method or by Rel-19 AI/ML.

Proposal 3: RAN4 to discuss the requirement for Rel-19 AI/ML based timing information reporting if introduced by other WGs.

- LCM related requirements

In RAN1#118 the following agreements are reached related to performance monitoring.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

In LCM procedure, the functionality/model activation/deactivation/fallback to non-AI function are to be considered. If the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., triggered by LMF via LPP signaling, the delay of activating/deactivating one function can be defined. Otherwise if these functionality/model changes are autonomously performed by the assisting node (UE or gNB) with AL/ML model, the assisting node would need to monitor the function performance, then the requirements would be related with performance monitoring requirements.

If performance monitoring is mainly executed at the assisting node, some metrics are to be defined so that UE itself can make decision of activation, deactivation, or fallback to non-AI. Then from RRM requirement perspective, the requirements of monitoring periodicity or monitoring delay when certain criterion is met may need to be specified.

Based on RAN1 progress so far, there are many open schemes for performance monitoring. Similar to other use cases, whether and how to define requirements for performance monitoring depends on the selected schemes which will be defined separately for each positioning sub-case. Also, it may depend on what kind of requirements are defined for inference for each positioning sub-case.

Proposal 4: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For Case 3b, reuse the legacy requirement for reporting of timing information or timing and power information from gNB to LMF.

Proposal 2: For Case 3a, RAN4 to discuss whether to reuse the legacy requirement for reporting of timing information from gNB to LMF, if there is no explicit indicator introduced to distinguish whether the timing information is obtained by legacy method or by Rel-19 AI/ML.

Proposal 3: RAN4 to discuss the requirement for Rel-19 AI/ML based timing information reporting if introduced by other WGs.

Proposal 4: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference.

References

- R4-2508080, “WF on AI ML for air interface study”, 3GPP RAN4 #115, 2025.