3GPP TSG-RAN WG4 Meeting #116bis				                              R4-2513442

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.6.3.2

Title: 	Discussion NGSO NTN RRM testing

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In Rel19 WI [1], RAN4 has also to introduce a channel model considering varying Doppler and timing shifts for NR-NTN and IoT-NTN devices [1]. A subset of tasks related to RRM requirements was highlighted below [1].

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In the last meeting, it was agreed that there is few impacts on the current RRM requirements due to the new TE channel model [2]. Therefore in this paper, how to define the corresponding tests of uplink timing for NGSO under the TE-emulated channel model was presented.

- Discussion

  - Test cases under TE channel model

In current TS38.133 [2], the following NTN test cases for timing were introduced in which AWGN channel was used.

Table A.14.3.1.1.1-2: Cell Specific Test Parameters for UL Transmit Timing test [2]

| Parameter | Unit | Config | Test1 | Test2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1,2 | 1 | 1 |
| Serving satellite configuration |  | 1 | SSC.1 |  |
|  |  | 2 | SSC.2 |  |
| BWchannel | MHz | 1,2 | 10: NRB,c = 52 |  |
| Initial BWP Configuration |  | 1,2 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP Configuration |  | 1,2 | DLBWP.1.1 ULBWP.1.1 |  |
| DRX Cycle | ms | 1,2 | N/A | DRX.8Note5 |
| PDSCH Reference measurement channel |  | 1,2 | SR.1.1 FDD |  |
| RMSI CORESET Reference Channel |  | 1,2 | CR.1.1 FDD |  |
| Dedicated CORESET Reference Channel |  | 1,2 | CCR.1.1 FDD |  |
| OCNG Patterns |  | 1,2 | OP.1 |  |
| SSB configuration |  | 1,2 | SSB.1 FR1 |  |
| SMTC Configuration |  | 1,2 | SMTC.1 |  |
| TRS configuration |  | 1,2 | TRS.1.1 FDD |  |
| EPRE ratio of PSS to SSS | dB | 1,2 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2 | -98 | -98 |
| Note2 | dBm/SCS | 1,2 | -98 | -98 |
|  |  | 1,2 | 3 | 3 |
|  |  | 1,2 | 3 | 3 |
| SS-RSRPNote3 | dBm/SCS | 1,2 | -95 | -95 |
| IoNote3 | dBm/9.36MHz | 1,2 | -65.2 | -65.2 |
| Propagation condition |  | 1,2 | AWGN |  |
| SRS Config |  | 1,2 | SRSConf.1Note6 | SRSConf.2Note6 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: DRX related parameters are given in Table A.3.3.8-1 Note 6: SRS configs are given in Table A.14.3.1.1.1-3 |  |  |  |  |

Observation 1: The NTN UL timing was tested under the legacy AWGN channel model.

Therefore, in order to verify the performance of NGSO UL timing under the new TE channel model, the additional test configuration for NTN NGSO can be introduced.

And in the last RAN4 meeting, for the other existing test cases whether the new TE channel model shall be applicable was debated but without consensus. Technically, if the AWGN channel replaced by the new TE channel whether the updated testing requirements or additional requirements margin is needed is still open in RAN4. If such updates needs for the other NTN testing cases beside the UL timing related, the overall additional work loading will be very huge.

Observation 2: The over loading because of the other NTN testing cases beside the UL timing related needs to be considered.

In the current WID, it was also implied that the working scope for NGSO testing is limited to UL timing test.

Observation 3: The other testing case beyond NGSO UL timing test applied the new TE channel model was not explicitly in the WID scope.

Proposal 1: The new test configuration with NTN NGSO TE channel model shall be introduced for NTN NGSO UL timing tests ONLY.

- Summary

This paper mainly proposes the consideration on the new NGSO TE channel impact to the current NTN UL timing core requirements and test cases.

Observation 1: The NTN UL timing was tested under the legacy AWGN channel model.

Observation 2: The over loading because of the other NTN testing cases beside the UL timing related needs to be considered.

Observation 3: The other testing case beyond NGSO UL timing test applied the new TE channel model was not explicitly in the WID scope.

Proposal 1: The new test configuration with NTN NGSO TE channel model shall be introduced for NTN NGSO UL timing tests ONLY.

- References

- RP-240857

- R4-2508261

- 3GPP TS 38.133 v18.7.0

- R4-2415870