3GPP TSG-RAN WG4 Meeting 116-bis	R4-2513455

# Prague, Czech Republic, October 13th – 17th, 2025

Source: 	Xiaomi

Title: 	Discussion on impacts on RAN4 requirement for RRM measurement prediction in AI mobility

Agenda Item:	7.12.3

Document for:	Discussion

# 1 Introduction

The AI mobility study item is now transitioning into the normative phase in Release 20. It is imperative for RAN4 to define the core performance requirements that will enable the promised gains of AI/ML while ensuring network reliability and performance. This contribution delves into the core requirements for RRM measurement prediction, covering both intra-frequency and inter-frequency scenarios. We identify key areas for requirement definition, leveraging agreements and discussions from RAN2, and propose the way forward for RAN4.

- Discussion

The WID agreed is as below:

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions For both RRM measurement prediction and measurement event prediction [RAN4]: - Specify RRM performance requirements and test cases for the corresponding use cases - Specify necessary performance requirements and test cases for LCM procedures, if any

Based on the WID and the current stage of the work plan, this contribution focuses exclusively on the foundational aspects of the core requirements for AI-based RRM measurement prediction. Topics pertaining to detailed performance validation, such as the specific definition of ground truth for accuracy calculation and mechanisms to ensure UE compliance (e.g., prevention of cheating), are considered integral to the "Performance Requirements and Test Cases" work item. We propose that these important but more detailed topics be addressed in the subsequent phase dedicated to performance specification.

The core requirements we aim to define in this early normative stage include:

- The performance metric of the prediction accuracy requirement (e.g., for multiple instances in a report).

- The impact of AI configuration (like Observation Window) on basic measurement period.

- The delay reduction impact on CSSF for inter-frequency prediction

This approach allows RAN4 to first establish a solid and agreed-upon conceptual framework, upon which the more detailed performance and test case requirements can be reliably built.

2.1 Performance Metric for Multi-Instance Prediction in Temporal Domain Case A

In legacy RRM measurement, a single measurement report contains one measurement result per cell, and the accuracy requirement is straightforwardly applied to that single result. However, for temporal domain Case A, it was agreed at RAN2#131 that the Prediction Window (PW) length is part of the inference configuration, and subsequently, it was agreed that one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message.

Agreements in RAN2#131

- For the interpretation of “skipping pattern” in temporal domain Case B, RAN2 confirm that it refers to SSB configuration to indicate the timing of NW's SSB transmission—not timing of UE's SSB measurement/skipping.

- For temporal domain Case A, NW can indicate the list of target cells for which it expects results (if available) as part of inference configuration.  This list is optional.

- For temporal domain case A, one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message

- For temporal domain Case B, the inference report can include the latest measurement results (regardless of actual results or predicted results).

- For frequency domain prediction, the inference report can include the latest measurement results of the predicted cell. (No change to the existing measurement report)

Observation 1: In RAN4, it’s agreed that one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message.

This new reporting mechanism raises a fundamental question on how to define the prediction accuracy requirement. If a single report contains M predicted results for a cell, and N such reports are transmitted, how do we define that the "prediction is accurate"?

- Option 1 (Overall Accuracy): The accuracy is evaluated over the total population of all predicted instances from all reports (i.e., M instances/report × N reports). The requirement (e.g., 90%) is applied to this total population.

- Option 2 (Per-Report Accuracy): The accuracy is first evaluated per report. A report is considered "accurate" only if all M predicted instances within it meet the accuracy criterion. Then, the requirement (e.g., 90% of reports are accurate) is applied to the set of N reports.

Option 1 is less strict but might mask poor performance in reports where most predictions are bad. Option 2 ensures the reliability of each holistic prediction report but is a more stringent requirement. Furthermore, it is recognized that prediction accuracy naturally degrades for instances further into the future (i.e., later in the PW). The choice of metric will significantly impact UE implementation complexity and testability.

Proposal 1: RAN4 shall discuss on the definition of prediction accuracy for multi-instance reports for temporal domain Case A.

      - 2.2 Measurement Time and Observation Window for Intra-frequency Prediction

Two scenarios are related to intra-frequency intra-cell prediction:

- FR1 to FR1 intra-frequency temporal domain case B

- FR2 to FR2 intra-frequency temporal domain case A

The introduction of an Observation Window (OW) and Prediction Window (PW) in AI-based prediction, as defined in TR 38.744, aims to reduce UE measurement burden. For temporal domain Case B, the core objective is to reduce measurements in the temporal domain by allowing the skipping of some measurement occasions.

the OW will have a direct impact on the measurement duration and report delay. Legacy L3 RSRP measurement periods are defined assuming periodic measurements without skipping. With AI, the reduced number of actual samples within a given time may necessitate a re-evaluation of the required measurement time to achieve reliable L3 filtering and prediction.

Proposal 2: For case B, Observation window length will have impact on measurement duration and report delay

The characteristics of the OW and PW are not uniform and are highly dependent on several factors agreed upon in RAN2: Observation window and prediction length is dependent on different sub-cases, different pattern and different L1/L3 filtering method.

Impact of L1/L3 sliding filtering

RAN2 defines L1/L3 sliding filtering and non-sliding filtering. Therefore, the final observation window length for the two cases is different for L3 RSRP to L3 RSRP prediction case.

Observation 2: Observation window and prediction length is dependent on different L1/L3 filtering method.

Impact of different sub-cases

Currently, RAN2 defined 3 subcases for RSRP prediction.

3 sub-use cases are considered for cell-level RRM measurement prediction: Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s) Sub-use case 3: Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) Editor Note 1: The RRM measurement refers to L3 cell/beam level measurement and/or L1 beam level measurement

- Sub-case 1: RSRP is predicted at L1

For sub-case 1, the prediction happens at L1. In legacy, 5 L1 samples are needed to be measured in one measurement period, which is shown in Fig.1(a). While when AI is applied, for example, only 3 samples are needed to be measured during observation window. Then UE can predict other samples with grey colour, as shown in Fig.1(b). Then measurement period can be reduced. Here observation window is similar as legacy measurement period with reduced samples. Observation window and prediction window length are based on number of L1 samples.

Fig.1(a) measurement period for non-AI L1 filtered RSRP for FR1

Fig.1(b) observation window and prediction window for AI based L1 filtered RSRP

- Sub-case 2: RSRP is predicted at L3

For sub-case 2, we give examples for non-sliding L1/L3 filtering case.

For sub-case 2, filtered L3 RSRP will be predicted by L3 RSRP. Suppose that every 2 L3 filtered RSRP can predict one L3 filtered RSRP. As shown in Fig.2, UE will measure for 10 samples to get 2 L3 results and skip the following L1 5 samples. For this case, the observation window and prediction window length are based on number of L1 measurement period.

Fig.2 observation window and prediction window for AI based L3 filtered RSRP

The observation window will be different for initial stage and later stage. At initial stage, new L1 filtered RSRP will be obtained very L1 sample.

- Sub-case 3: RSRP is predicted at L3

For sub-case 3, RSRP is also predicted at L3. Then observation window and prediction window are similar as sub-case 2.

Impact of pattern

RAN2 also defines different patterns for prediction window and observation window as below, it will also have impact on the prediction window length and observation window length

Fig.3 Example 1 of intra-frequency temporal domain case B

Fig.4 Example 2 of intra-frequency temporal domain case B

Observation 3: The effective measurement time for AI-based RRM is contingent on the configured OW pattern, the RRM sub-use case, and the L1/L3 filtering method, which differs from the legacy fixed measurement period.

Proposal 3: For intra-frequency intra-cell prediction, RAN4 to select a set of typical OW/PW patterns and configurations as the basis for defining new measurement time requirements.

      - 2.3 Impact on Inter-frequency / Inter-cell Prediction and CSSF

For inter-frequency inter-cell prediction, the key benefit is the potential to skip measurements on other frequencies by predicting them, thus reducing the need for measurement gaps. RAN2 has agreed that for FR1 to FR1 inter-frequency inter-cell prediction, the focus is on cases where cells are co-located, and an inter-frequency correlation model can be assumed .

- The current legacy requirements define a Cell-Specific Search Frequency (CSSF) as part of the measurement period, which accounts for the time needed to measure inter-frequency cells.

- If a UE can accurately predict the RSRP of an inter-frequency cell without physically measuring it, the actual demand for measurement gaps is reduced. This has a direct impact on the feasibility and extent of measurement period reduction.

Observation 4: Successful inter-frequency prediction can reduce the effective measurement load on other frequencies, which should be reflected in the requirements governing inter-frequency measurement scheduling, such as the CSSF.

Proposal 4: For inter-frequency inter-cell RSRP prediction, RAN4 should study the impact on CSSF reduction in measurement period and define new requirements accordingly.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: In RAN4, it’s agreed that one or more instances of predicted measurement results in PW per one cell are reported in one measurementReport message.

Proposal 1: RAN4 shall discuss on the definition of prediction accuracy for multi-instance reports for temporal domain Case A.

Proposal 2: For case B, Observation window length will have impact on measurement duration and report delay

Observation 2: Observation window and prediction length is dependent on different L1/L3 filtering method.

Observation 3: The effective measurement time for AI-based RRM is contingent on the configured OW pattern, the RRM sub-use case, and the L1/L3 filtering method, which differs from the legacy fixed measurement period.

Proposal 3: For intra-frequency intra-cell prediction, RAN4 to select a set of typical OW/PW patterns and configurations as the basis for defining new measurement time requirements.

Observation 4: Successful inter-frequency prediction can reduce the effective measurement load on other frequencies, which should be reflected in the requirements governing inter-frequency measurement scheduling, such as the CSSF.

Proposal 4: For inter-frequency inter-cell RSRP prediction, RAN4 should study the impact on CSSF reduction in measurement period and define new requirements accordingly.

- Reference