# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2513459

# Prague, CZ, 13-17 Oct, 2025

Source: 	Xiaomi

Title: 	(NR_NTN_Ph3-Core) Discussion on remaining issues on RRM core requirements of NTN for NR Phase 3

Agenda Item:	4.25.3

Document for:	Discussion

# 1Introduction

In the last RAN4#116 meeting, the discussion on RRM impact on the SMTC enhancement worked out fine and the agreements were captured in the approved WF [1]. During last meeting, RAN2 reached out further agreements on the SMTC enhancement scheme, an LS [2] was sent to RAN4 as captured below:

For downlink coverage enhancement, RAN2 further made the following agreements related to SMTC enhancement: - We introduce a capability to support STMC enhancements to FR1 (FFS on FR2 depending on RAN4 progress). - The maximum number configured SMTCs for idle/inactive is 7 and it also includes the SMTC of the serving cell (This updates a previous decision to have a maximum of 6 STMCs). - RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. - In connected mode the NW will only configure up to 4 STMC. - RAN2 supports to configure two different SMTC periodicities (with different offsets) for SMTCs per frequency layer for idle/inactive/connected mode, and UE capability will be introduced for this purpose (FFS if per UE or per band). If RAN4 cannot finish the corresponding RRM requirement design, we can introduce some statement in the specs in the future to limit the configuration to one periodicity only. RAN2 kindly request RAN4 to take above agreement into account when designing RAN4 RRM requirement for SMTC enhancement and provide feedback if needed.

In this contribution, we would like to continue the discussion on the remaining open issues.

- Discussion

Based on last meeting discussion, the requirement impact on the SMTC enhancement was mainly analyzed for RRC_CONNECTED mode. For RRC_IDLE mode, the remaining open issues are summarized below:

## [Online # 1] Issue 0: SMTC enhancements Moderator’s WF: - Applicability of 4 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN? - Option 1: Yes - Option 2: No (All NR NTN bands, including any that may be introduced in future releases)

The first issue we want to discuss is whether to limit the applicability of RAN4 requirement for 6 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode to FR1-NTN. From our perspective, the answer is yes. Requirements for FR2-NTN were initially discussed in Rel-18 for VSAT UE. Due to time limitation, RAN4 defined the FR2-NTN requirements based on some assumptions, and one of the assumptions is no Rx beam sweeping is expected for the mobility measurement. However, we understand the motivation of SMTC enhancement is to facilitate UE measurement due to beam hopping. For beam hopping, in our understanding, UE in FR2-NTN need more time to adjust the Rx beam, which is not aligned with the previous assumption. Considering the Rel-19 timeline, we prefer not to consider the STMC enhancements to FR2-NTN.

Proposal 1: Applicability of 7 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN.

###### [Online # 2] Issue 0-A: More than 4 SMTCs per frequency (i.e. 6) RAN4 to further discuss the following in RAN4#116-bis: - For RRC Idle/Inactive mode RRM requirements, no update is needed except for the following addition to the specification where relevant: - The SMTCs referenced in the subsequent requirements refer to the SMTCs selected by the UE. - Note: The SMTC selection rule may be further discussed and specified or left to UE implementation.

Based on the latest RAN2 LS, it has been agreed to support configuring 7 SMTCs per frequency for idle/inactive UEs. Based on current UE capability, UE can at most support 4 SMTCs per frequency. Going back to R17 discussion, RAN4 has already considered the case that the number of configured SMTCs per frequency layer is beyond UE capability, and concluded that no requirement for such case.

RAN2 agreed to introduction location based SMTC selection for RRC idle/inactive mode to help UE do the SMTC selection when configured with multiple SMTCs. Also, RAN2 agreed that it is up to UE implementation on how to utilize the assistance information. Based on RAN2 progress, the bellow several UE features were agreed to be introduced for SMTC enhancements:

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| x. NR_NTN_Ph3-Core | x-3 | SMTC enhancement with two periodicities | Indicates whether the UE supports measurement on target cells belonging to two SMTC periodicities on a single frequency carrier. | 34-1 |
| x. NR_NTN_Ph3-Core | x-4 | UE reporting closest reference location(s) | Indicates whether the UE supports reporting closest reference location(s) based on reference location configurations. | 34-1 |
| x. NR_NTN_Ph3-Core | x-5 | Location based SMTC selection | Indicates whether the UE supports SMTC(s) selection based on UE’s position and associated reference location to each SMTC in RRC idle/inactive. | 34-1 |

RAN2 introduced UE capability on indicating support of SMTC(s) selection based on UE’s position and associated reference location to each SMTC in RRC idle/inactive. From our perspective, this optional capability to some extent differentiates different UE implementation. For UE support this capability, we are fine to introduce related measurement requirements.

Proposal 2: If the UE supports Location based SMTC selection and related NW configuration is present, requirements in section 4.2C.2.3 and 4.2C.2.4 apply and the SMTCs referenced in the requirements refer to the SMTCs selected by the UE.

## [Online # 3] Issue 0-B: Two different SMTC periodicities (with different offsets) in one frequency layer Moderator’s WF: - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For M1/M2/M3/M4 value in Cell re-selection requirement: - CMCC: M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC 6, - Huawei: No further relaxation on M1 - CMCC, Apple: The SMTC periodicity in this note of M2/M3/M4 is the one used by the cell being identified - ZTE: - M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC ≤ 6 upon more than 1 SMTC configured at the UE - M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE - Xiaomi: The SMTC in PSS/SSS detection corresponds to the shortest SMTC periodicity

In most of RRM requirements for RRC_IDLE/INACTIVE mode, DRX cycle is used to define the measurement delay. The SMTC periodicity is only used under some cases. Based on RAN2 agreements, we can see that the configuration of two different SMTC periodicities (with different offsets) in one frequency layer is only for neighbor cell measurements. So, we go through the requirements for RRC_IDLE mode, and capture the related text below:

#### 4.2C.2.3 Measurements of intra-frequency NR cells Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1 and FR2-NTN. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh DRX cycle length [s]; Tdetect,NR_Intra_enh [s] (number of DRX cycles); Tmeasure,NR_Intra_enh [s] (number of DRX cycles); Tevaluate,NR_Intra_enh [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2)Note 1; 0.32 x M3 (1 x M3) Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5.

#### 4.2C.2.4 Measurements of inter-frequency NR cells The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in table 4.2C.2.4-1 under the following conditions: - TSMTC_intra = TSMTC_inter = 160 ms; where - TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. - TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected. - SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and - SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS 38.304 [1].

As highlighted in the text, the UE behavior needs to be clarified when it is configured with different SMTC periodicities during PSS/SSS detection.

Proposal 3: RAN4 to clarify that the SMTC periodicity used during PSS/SSS detection is the shortest SMTC periodicity among all the configured SMTCs, for intra-frequency measurement and inter-frequency measurement requirements in RRC_IDLE/INACTIVE mode.

- Conclusion

Proposal 1: Applicability of 7 SMTCs for RRC Idle/Inactive UEs and 2 different SMTC periodicities for RRC Idle/Inactive/Connected mode UEs is limited to FR1-NTN.

Proposal 2: If the UE supports Location based SMTC selection and related NW configuration is present, requirements in section 4.2C.2.3 and 4.2C.2.4 apply and the SMTCs referenced in the requirements refer to the SMTCs selected by the UE.

Proposal 3: RAN4 to clarify that the SMTC periodicity used during PSS/SSS detection is the shortest SMTC periodicity among all the configured SMTCs, for intra-frequency measurement and inter-frequency measurement requirements in RRC_IDLE/INACTIVE mode.

- Reference

- R4-2512337, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm

- R2-2503068, LS on SMTC enhancements, RAN2