3GPP TSG-RAN WG4 Meeting #116bis                   	R4-2513464

Agenda item:			7.6.3

Source:	Xiaomi

Title:	Discussion on R20 HPUE NTN RX requirement

Document for:	Discussion

Background

In t he RAN#109 meeting, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2 by the meeting aims to complete missing requirements for NR-NTN (Non-Terrestrial Networks) and IoT-NTN that were not able to be concluded in Rel-19.

HPUE for NTN (PC1.5)

The first set of requirements to be completed relate to higher UE uplink power classes. Specifically, the objective for high power UE (HPUE) for NTN from the Rel-19 work item titled “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” originally included power class 2 (PC2 26 dBm) and power class 1.5 (PC1.5 29 dBm for TDD only) and power class 1 (PC1 31 dBm) but was downscoped to include specifications for only PC2 and PC1 due to lack of time.  This work item therefore seeks to include specifications for PC1.5.  It is noted that the coexistence analysis conducted in Rel-19 considered up to PC1 for both NR NTN and IoT NTN, so can also be applicable to PC1.5.

Until Rel-18, the 3GPP specifications limit maximum non-terrestrial network (NTN) UE transmission (Tx) power to the level of 23dBm, which may not allow to use the full potential of satellite communication. On the other hand, power class 2 (PC2 26dBm) and power class 1.5 (PC1.5 29dBm for TDD only) are widely supported for terrestrial network (TN), and power class 1 (PC1 31dBm) is also supported for particular device for public safety.

When compared to the existing TN deployments, NTN becomes critical in adverse conditions such as severe weather events or natural disasters, where or when the terrestrial deployments are not present or temporally unavailable. However, satellite communication involves transmitting signals over long distances, which results in larger path loss. Thus increasing the UE maximum transmit power will significantly improve UL performance in terms of coverage, availability and throughput, which would be needed to meet the above market demands associated with fixed wireless, public safety and automotive usage.

HPUE for NTN is feasible. For a number of NTN use cases, including automotive, maritime, logistics, outdoor industrial monitoring and other scenarios not reliant on battery power, such that the higher power requirements of PC2, PC1.5 and PC1 are not with a battery power issue. Regarding the regulation, the existing regulation and specifications used by the current satellite services (primarily MSS), such as ECC/CEPT and FCC, typically allow significantly higher UE transmit powers even for handheld (e.g. satellite phones and trackers, which are different from smartphones) and automotive UE, and operate in narrow-band spectrum. Therefore, regulation is not expected to be a barrier.

Both NR-NTN and IoT-NTN need be included since the techniques share the commonality for them.

In this document, we would like to discuss the Rx requriements for HPUE for IoT-NTN.

Disucussion

In the WID of Rel-20 NTN HPUE, the major propose is to determine the RSD requirement for PC1.5 UE.

In the Rel-19, the agreement for NB-IoT based NTN RSD for the PC2 HPUE is not needed, since Regarding PC2 IoT NTN IoT NB1 and NB2 UEs operated in half-duplex FDD mode, no RX RSD for bands 256 and 255. Similarly, PC1.5 NB-IoT based HPUE should not change the duplex type and the RSD may not necessary for the PC1.5 UE.

Propose 1: For NB-Iot based Iot-NTN, no RSD needed for PC1.5 HPUE.

The RX requirements including max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, spurious emissions. Even though RSD is not necessary for the PC1.5 NB-Iot based Iot-NTN, RAN4 still can review the other RX requirement to check whether those requirements should be considered or not.

Propose 2: RAN4 can discuss the necessity of the other RX requirement for PC1.5 NB-Iot based Iot-NTN.

Conclusion

Propose 1: For NB-Iot based Iot-NTN, no RSD needed for PC1.5 HPUE.

Propose 2: RAN4 can discuss the necessity of other RX requirement for PC1.5 NB-Iot based Iot-NTN