3GPP TSG-RAN WG4 Meeting # 116bis	 R4-2513471

Prague, Czech Republic, 13-17 October 2025

Agenda item:	6.12.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of UE-initiated/event-driven beam management

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. In this contribution, we provide our views regarding the RAN4 impacts on the performance part of UEIBM.

- Discussion

UE-initiated/event-driven beam management

During the last meeting, the WF [4] captured the test scope; this paper will provide our view on the way forward. The  content from the WF [4] are captured in the following:

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

For UEIBM, considering the progress on core requirements discussion so far, test cases to verify UE behaviour and requirements shall be introduced. In detail, the following aspects shall be considered for test case design.

- timeRestrictionForChannelMeasurement: Based on the discussion in the last meeting, as the last the UE behaviour when M=1 is clear. Whether to define TC for M=3 (i.e. timeRestrictionForChannelMeasurement needs FFS)

- SSB-based/CSI-RS-based: Both SSB-based and CSI-RS based UEIBR is supported. The remaining issue is the applicable rule of CSI-RS based UEIBR for inter-cell. As discussed for many meetings, it is the historical problem. Thus, at least test case for intra-cell shall be defined for both SSB-based and CSI-RS based UE IBR.

- Event 1/2/7: Based on RAN1 discussion, Event 1/2/7 are both supported. For RRM requirements and TC, RAN4 shall discuss whether we should introduce TC for both Event 1/2/7. One pathway is to define TC for all events and define applicability rules so that UE is not required to test all three events. Companies’ view of event 7 shows that there is one test issue on it; we share the same view. The TC design for event 7 needs further discussion.

Events in Rel-19 UEIEBM: Event-1: Quality of the current beam is worse than a certain threshold. Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

Proposal 1: For UEIBR, define TC as follows:

- SSB-based UEIBR for Event-2.

- CSI-RS based UEIBR for Event-2.

Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured.

Note 2: At least define TC for intra-cell.

Proposal 2: RAN4 to discuss whether to define TC for other Events, i.e. Event 1 and Event 7.

- Time window-based: RAN4 introduced the window-based requirement. The agreement on this issue is shown as follows:

Issue 1-1-a: How to specify requirements for “with time window based with eventInstanceCount-r19” if option 2 in Issue 1-1-1 is supported? Agreement: For the window-based solution: if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel - eventInstanceCount-r19 is configured by NW - L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19). - TL1-meas_basic is the L1 measurement period which agreed in without window-based soltuion, in details, it is the maximum of L1-RSRP measurement periods on the beams corresponding to the event The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic , and no requirement when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic. Send the LS capturing the RAN4 agreements to RAN1/2, and the action requested from RAN4 to RAN1/2 is to take RAN4 agreements into account.

There is no core requirement defined for the case where the window (eventDetectionTimeWindowLength-r19) is not provided and the counter (eventInstanceCount-r19) is larger than 1. Therefore, the test case for UEIEBM should not cover the case with no time window and eventInstanceCount-r19 > 1.

Proposal 3: RAN4 not to define the TC for the case that no time window and eventInstanceCount-r19 > 1.

Proposal 4: RAN4 to discuss whether to define TC for inter-cell UEIBR.

- Beam switching delay for TCI state switching: RAN4 has agreement that the TCI switching delay requirements do not need to be updated or specified. So, it is unnecessary to define the test cases for beam switching delay for TCI state switching. The content of agreements is captured from WF [4], shown as follows:

Issue 1-2: Whether to specify new unified TCI switching delay requirements? Agreement: - This issue is closed. Issue 1-3: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? Agreement: - No need to capture this scenario in RAN4 requirements.

Proposal 5: RAN4 not to define the TC for Beam switching delay for TCI states switching.

- Mode A and Mode B: These two types of beam reports are defined by RAN1. Mode A adapts to resource fluctuations through dynamic scheduling but increases signalling overhead. Mode B relies on pre-configured resources, reducing signalling but sacrificing flexibility. Mode A is a basic capability, while Mode B may be an optional feature requiring network configuration support. In our view, all of these two Mode need to be considered in the TC, but the UE only need to pass one of them.

Proposal 6: RAN4 defines TC for both Mode A and Mode B. The UE only need to pass one of them.

- Overall discussion: RAN4 to define the test cases of UEIEBM in FR2.

Proposal 7: RAN4 to define the test cases of UEIEBM in FR2.

Note: We update our views about the UEIEBM test scope in the table, which you can find in the Annex.

- Conclusions

Proposal 1: For UEIBR, define TC as follows:

- SSB-based UEIBR for Event-2.

- CSI-RS based UEIBR for Event-2.

Note 1: At least define TC when timeRestrictionForChannelMeasurement is configured.

Note 2: At least define TC for intra-cell.

Proposal 2: RAN4 to discuss whether to define TC for other Events, i.e. Event 1 and Event 7.

Proposal 3: RAN4 not to define the TC for the case that no time window and eventInstanceCount-r19 > 1.

Proposal 4: RAN4 to discuss whether to define TC for inter-cell UEIBR.

Proposal 5: RAN4 not to define the TC for Beam switching delay for TCI states switching.

Proposal 6: RAN4 defines TC for both Mode A and Mode B. The UE only need to pass one of them.

Proposal 7: RAN4 to define the test cases of UEIEBM in FR2.

Note: We update our views about the UEIEBM test scope in the Table 5 - 1, which you can find in the Annex.

4. References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2508272 WF on RRM requirements for NR_MIMO_Ph5_Part2

[4] R4-2512130 WF on RRM requirements for NR_MIMO_Ph5_Part1

5. Annex

Table 5-1 Views on UEIEBM test scope

|  | Huawei |
| --- | --- |
| High level: Define TCs to verify UEIBM | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2 FFS on others |
| Whether to cover Mode A and Mode B? | Both. UE only need to pass one of them. |
| Event-trigger for Intra-cell BM for SSB | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y |
| Event-trigger for Inter-cell BM for SSB | FFS |
| Event-trigger for Inter-cell BM for CSI-RS | FFS |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N |
| Frequency Range – FR2? | Y |
| Beam switching delay for TCI states switching | N |
| L1 measurement reporting delay |  |
| CMI |  |