3GPP TSG-RAN WG4 Meeting #116bis	R4-2513489

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on performance requirements of Event triggered L1 measurement reporting

Source: 	Huawei, HiSilicon

Agenda item:	6.17.2.1

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

The contribution provides preliminary discussion on the performance requirements for R19 mobility.

Discussion

There are SSB based and CSI-RS based event triggered report.

SSB based event triggered report

It is straight forward to define test cases for SSB based LTM Intra-frequency and inter-frequency event triggered reporting without gap:

- FR1 and FR2 SSB based LTM Intra-frequency event triggered reporting tests without gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests with gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests without gap

It shall be noted that from UE testing perspective, as the R19 SSB based event triggered tests are on top of existing R18 SSB based L1 -RSRP measurement tests, the difference is that event is to be configured and will trigger a UE reporting, UE which can pass R19 test cases can skip R18 test cases.

Proposal 1: The following test cases are specified for SSB based L1 measurement event trigger reporting:

- FR1 and FR2 SSB based LTM Intra-frequency event triggered reporting tests without gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests with gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests without gap

Proposal 2: UE which can pass the above R19 test cases can skip corresponding R18 SSB based L1 measurement test cases.

CSI-RS based event triggered report

In core part, it is agreed that NOT to define RAN4 requirements for L1 measurement with gap for CSI-RS based L1 measurement on neighbor cell. And as per the intra-frequency definition of CSI-RS based measurement, gap is not needed in intra-frequency measurement.

Proposal 3: The following test cases are specified for CSI-RS based L1 measurement event trigger reporting:

- FR1 CSI-RS based LTM Intra-frequency event triggered reporting tests without gap

- FR2 CSI-RS based LTM Intra-frequency event triggered reporting tests without gap

I shall be emphasized that the baseline CSI-RS L1 measurement on neighbor cell is “3-step” (i.e., L3 measurement, SSB based L1 measurement, CSI-RS based L1 measurement). Skipping step2 is an optional capability. To reduce test number and effort, we suggest to verify 3-step CSI-RS L1-RSRP measurement only.

Proposal 4:  For event triggered reporting tests for CSI-RS L1 RSRP measurement, “3-step” CSI-RS measurement procedure is to be verified.

As per UE capability agreed in RAN1 and RAN2, CSI-RS based L1 measurement and LTM event trigger reporting are sperate UE capability. For UE which supports both capabilities, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

Proposal 5: For UE which supports both capabilities of CSI-RS based L1 measurement and LTM event trigger reporting, it is only required to pass CSI-RS based L1 measurement event trigger test cases, and skip the test sets of CSI-RS based L1 measurement without reporting.

Conclusion

The contribution provides analysis on performance part. The following proposals are proposed:

Proposal 1: The following test cases are specified for SSB based L1 measurement event trigger reporting:

- FR1 and FR2 SSB based LTM Intra-frequency event triggered reporting tests without gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests with gap

- FR1 and FR2 SSB based LTM Inter-frequency event triggered reporting tests without gap

Proposal 2: UE which can pass the above R19 test cases can skip corresponding R18 SSB based L1 measurement test cases.

Proposal 3: The following test cases are specified for CSI-RS based L1 measurement event trigger reporting:

- FR1 CSI-RS based LTM Intra-frequency event triggered reporting tests without gap

- FR2 CSI-RS based LTM Intra-frequency event triggered reporting tests without gap

Proposal 4:  For event triggered reporting tests for CSI-RS L1 RSRP measurement, “3-step” CSI-RS measurement procedure is to be verified.

Proposal 5: UE which can pass CSI-RS based L1 measurement event trigger test cases can skip the test sets of CSI-RS based L1 measurement without reporting.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4