3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513495

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on enhancement of low NR band carrier aggregation via switching

Source: 	Huawei, HiSilicon

Agenda item:	7.8.4

Document for:	Discussion

Introduction

R20 WI on enhanced low band carrier aggregation via switching is approved in [1].

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

The contribution provides further analysis on enhanced low band carrier aggregation via switching from RRM perspective.

Discussion

Background

R20 LB-CA WI is an enhancement of R19 LB-CA especially for the case of switching between FDD and SDL bands with enabling dual downlink.

The R20 LB-CA allow switching between {case 1, case 3} which enables low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band:

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

RANP has made conclusion that semi-static RRC configured switching pattern defined in Rel-19 LB-CA is reused in R20.

Figure 1. case 1 and case 3

Figure 2. Switching in R20 enhanced LB-CA

It is observed in R20 enhanced LB-CA via switching:

- DL reception on FDD carrier is not confined to switching pattern. UE can perform DL reception on PCC during both FDD ON duration and “dual SDL+FDD DL” ON duration in accordance with switching pattern

- UL transmission on FDD carrier is confined to the switching pattern

- DL reception on SDL carrier is confined to the switching pattern

Note: In switching pattern, FDD UL ON duration responds to SDL DL OFF duration, vice versa. UE is not able to simultaneously receive on SDL and transmit on FDD UL.

PCC related RRM requirements

As illustrated in Figure 2, DL reception on FDD carrier is not confined to switching pattern. UE can perform DL reception on PCC during both FDD ON duration and “dual SDL+FDD DL” ON duration in accordance with switching pattern. This is quite different with the R19 LB-CA where PCell DL reception depends on PCell ON duration. Therefore the L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC defined for R19 LB-CA via switching are not applicable for R20.

Proposal 1: The L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC defined for R19 LB-CA via switching are not applicable for R20 eLB-CA.

It shall be noted that RF will discuss the switch period between case 1 and case 3. During the switch period/gap, UE does not transmit and receive any signals. If SSB/CSI-RS on PCC collides with the switching period/gap, the L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC would be affected. From specifying requirements perspective, it can clarify that the available RS is the RS not colliding with switching period, or it can simply to say that the corresponding delay would be extended if the to-be measured RS is colliding with switching period.

Proposal 2: If to-be-measured SSB/CSI-RS on PCC collides with the switching period/gap, the L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC would be affected. RAN4 to discuss how to specify the responding requirements, e.g., clarify that the available RS is the RS not colliding with switching period/gap, or simply to clarify the delay will be extended if the to-be-measured RS is colliding with switching period/gap.

Interruption due to L3 measurement on deactivated SDL

In R19 LB-CA via switching, RAN4 agreed that switching pattern is not applicable when SDL SCell is deactivated. We think the same principle can be inherited to R20 eLB-CA to reduce excessive interruptions on PCC.

Proposal 3: The principle can be inherited from R19: Switching pattern is NOT applicable when SDL SCell is deactivated in R20 eLB-CA.

In R20 enhanced low band switching scenario, UE is not able to simultaneously receive on SDL and transmit on FDD UL. In order to measure on deactivated SDL, UE shall switch to deactivated SDL carrier and switch back to PCC after L3 measurement. As UE can support dual DL reception on both PCell and SDL, there is no interruption on PCell DL during UE performing measurement on deactivated SCell. The interruption occurs at “switch from” and “switch to” instances. Therefore the interruption length on PCell DL is 2x retuning time (switch to and back).

As UE is not able to support simultaneous DL measurement on SDL and UL transmission on PCell, the interruption length on PCell UL shall comprise 2 x retuning time (switch to and back) and the time duration staying on SDL carrier (i.e., SMTC duration).

Proposal 4: In enhanced LB-CA via switching:

- the DL interruption length on PCell is 2x retuning time (switch to and back)

- the UL interruption length on PCell comprises 2 x retuning time (switch to and back) and the time duration staying on SDL carrier (i.e., SMTC duration)

Regarding the interruption percentage, the loss of ACK/NACK may be caused by the interruption of PDCCH and the interruption of the UL transmission which carrying ACK/NACK. To be precise, the ACK/NACK loss on PCell would be twice the interruption length if both PCell UL and DL are interrupted. The ACK/NACK loss on PCell would be equal to the interruption length if only PCell UL is interrupted. Therefore in R20 eLB-CA, the missed ACK/NACK percentage shall be represented as X=2xA+BmeasCycleSCell  where A = 2x retuning time (switch to and back), B= the time duration staying on SDL carrier (i.e., SMTC duration).

Proposal 5: In R20 eLB-CA, the missed ACK/NACK percentage shall be represented as X=(2xA+B)/(measCycleSCell ) where A = 2x retuning time (switch to and back), B= the time duration staying on SDL carrier (i.e., SMTC duration)

SDL SCell related requirements

SDL SCell activation

In RAN4#116 meeting, for R19 LB-CA it is agreed that switching pattern is also applied to SCell activation procedure. The motivation of the agreement is that some companies think if seme-statically configured switching pattern is activated by the DCI based aperiodic CQI reporting which is the ending point of SCell activation procedure, certain time margin will be needed for applying RRC switching pattern. The bottleneck is due to the lack of interaction between physical layer and higher layer. Based on the implementation restriction, RAN4 reverted the previous conclusion and agreed to use SCell activation command MAC CE to trigger RRC configured switching pattern in RAN4#116 meeting. In R20 eLB-CA, we think the similar implementation bottleneck still exists, so it is fine to follow the same principle, i.e., switching pattern is applied to SCell activation procedure in R20.

Proposal 6: The principle can be inherited from R19: switching pattern is applied to SCell activation procedure in R20 eLB-CA.

In R20 eLB-CA, as the available SDL SCell reference signals to be measured for SCell activation are also in accordance with the SDL SCell active period in switching pattern, the SCell activation delay defined in R19 LB-CA can be reused. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed, and we think it is just a wording clarification.

Proposal 7: SCell activation delay defined in R19 LB-CA can be reused. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed.

BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

It is straight forward that the switching pattern is applicable to activated carriers. In R20 eLB-CA via switching, the BFD/CBD, TCI state switch delay, PL-RS switch, L3-RSRP/L1-RSRP/SINR measurement on SDL are in accordance with the SDL SCell active period based on switching pattern. This is quite similar as R19 LB-CA. Therefore the BFD/CBD evaluation period, TCI state switch delay, PL-RS switch delay, L3-RSRP/L1-RSRP/SINR measurement period defined in R19 LB-CA can be reused. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed, and we think it is just a wording clarification.

Proposal 8: The BFD/CBD evaluation period, TCI state switch delay, PL-RS switch delay, L3-RSRP/L1-RSRP/SINR measurement period defined in R19 LB-CA can be reused in R20 eLB-CA. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed.

Conclusions

This contribution provides further analysis on enhanced low band carrier aggregation via switching from RRM perspective. The following proposals are provided:

PCC related RRM requirements

Proposal 1: The L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC defined for R19 LB-CA via switching are not applicable for R20 eLB-CA.

Proposal 2: If to-be-measured SSB/CSI-RS on PCC collides with the switching period/gap, the L3, L1 measurement, TCI state switching and RLM/BFD/CBD evaluation requirements on PCC would be affected. RAN4 to discuss how to specify the responding requirements, e.g., clarify that the available RS is the RS not colliding with switching period/gap, or simply to clarify the delay will be extended if the to-be-measured RS is colliding with switching period/gap.

Interruption due to L3 measurement on deactivated SDL

Proposal 3: The principle can be inherited from R19: Switching pattern is NOT applicable when SDL SCell is deactivated in R20 eLB-CA.

Proposal 4: In enhanced LB-CA via switching:

- the DL interruption length on PCell is 2x retuning time (switch to and back)

- the UL interruption length on PCell comprises 2 x retuning time (switch to and back) and the time duration staying on SDL carrier (i.e., SMTC duration)

Proposal 5: In R20 eLB-CA, the missed ACK/NACK percentage shall be represented as X=(2xA+B)/(measCycleSCell ) where A = 2x retuning time (switch to and back), B= the time duration staying on SDL carrier (i.e., SMTC duration)

SDL SCell activation

Proposal 6: The principle can be inherited from R19: switching pattern is applied to SCell activation procedure in R20 eLB-CA.

Proposal 7: SCell activation delay defined in R19 LB-CA can be reused. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed.

BFD/CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for SDL SCell

Proposal 8: The BFD/CBD evaluation period, TCI state switch delay, PL-RS switch delay, L3-RSRP/L1-RSRP/SINR measurement period defined in R19 LB-CA can be reused in R20 eLB-CA. Whether certain clarification, e.g., “dual SDL SCell and PCC DL active period”, is needed can be further discussed.

Reference

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching