3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513540

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.9.3

Source: 	National Taiwan University

Title: 	Reference Model Derivation and Selection for Inter-Vendor Training Collaboration and Interoperability for Two-Sided AI/ML Models

Document for:	Approval

Introduction

We present our view on partially specified decoder and reference decoder derivation procedure in this contribution.

Discussion

Reference model derivation procedure

In previous RAN4 discussion, many companies have proposed decoder derivation procedures. We summarize them in the following, by first list out the steps proposed in [1] [2] and then list the contents from related RAN4 contributions (from Apple[3], Vivo[4],Samsung[5],Oppo[6]).

- Step 1: Determine simulation setup and encoder input dataset generation procedure

Summary of company contributions:

Apple (R4-2407236): For generating data for training the pair of UE encoder/Test Decoder, use a system-level simulator. The proposed baseline system simulation assumptions are shown in Table 2. (partial quote for the relevant part) Build a database by agreeing channel model(s) and aggregate data from UE vendors.

Vivo (R4-2408294): align the dataset containing only channel information, by agreeing simulation assumptions and data sample format etc. Multiple dataset can be merged

Samsung (R4-2405653): standardization procedure start: consider a certain use case, e.g., CSI compression under certain configuration

Oppo (R4-2405187): determine test condition; determine test data, considering generation method, collect/merg data from different companies

  - Summary and suggestions based on RAN4 discussion:

    - Open issue for simulation setup:

- System level or link level simulation for determining test decoder

- Using setup in RAN4 TR as starting points, what are the parameters need to be updated

    - RAN4 concluded that aggregated encoder input dataset is recommended for reference decoder evaluation since the performance variation across encoder/decoder pairs from different companies when input dataset from only one company is used

- Step 2: Determine model type and parameter

Summary of company contributions:

Apple: identify model architectural parameters for test decoder, e.g., backbone type, model depth/width, layer type/size, quantization

Vivo: determine the model hyperparameters that need to be aligned

Samsung: identify necessary model architecture parameters

Oppo: determine the test model structure, how to setup and merge into one

  - Summary and suggestions based on RAN4 discussion

    - RAN4 has selected MLP and CNN in R19 discussion

- Step 3: Determine training hyper parameters

Summary of company contributions:

Apple: specify training model parameters (loss function, hyper-parameters, regularization 	etc). Build a dataset (from step 1) with encoder output

Samsung: identify necessary model training parameters (training procedure, loss function etc)

Qualcomm: remove training procedure from “the parameter needed to be agreed”

  - Summary and suggestions based on RAN4 discussion:

    - Open issue: whether to agree training related parameters, and if needed, what parameters to be agreed

- Step 4: Determine the test decoder(s) by first determine evaluation methodology and selection/merge criterion, and then decide the test decoder(s) based on companies’ evaluation

We will discuss this step in the next section.

We summarize the above discussions into the two proposals below:

Proposal 1: Consider the following steps for deriving the reference models (for encoder and decoder) for dataset generation:

- Step 1: Determine simulation setup and encoder input dataset generation procedure

- Step 2: Determine model type and parameter

- Step 3: Determine training hyper parameters

- Step 4: Determine the test decoder(s) by first determine evaluation methodology and selection/merge criterion, and then decide the test decoder(s) based on companies’ evaluation

Proposal 2: Consider the following guidelines based on R19 RAN4 discussions

- For simulation setup:

  - First decide whether to use system level or link level simulation for determining test decoder

  - Consider setup in RAN4 R19 TR as starting points, what are the parameters need to be updated

- RAN4 concluded that aggregated encoder input dataset is recommended for reference decoder evaluation since the performance variation across encoder/decoder pairs from different companies when input dataset from only one company is used

- RAN4 has selected MLP and CNN in R19 discussion, and these models can be the starting point for RAN1 discussion

- For training, RAN1 needs to decide whether to agree training related parameters, and if yes, what parameters to be agreed

Discussion for Reference Decoder Selection Criterion and Inter-Operability

As pointed out in [1], the inter-vendor collaboration procedure targets the best system performance with reasonable complexity to limit implementation cost, as well as a flexible design space to allow vendors to develop their own models based on the reference model(s). In RAN4#116 meeting, the following criteria for decoder selection are agreed:

“Encoder / Decoder selection criteria: Achievable performance, complexity and robustness in different conditions need to be taken into account in selecting the model.”

Based on the study done in [1], we propose further details of the selection criterion below.

Proposal 3: Consider the following as (test/reference) encoder/decoder selection criterion:

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

The evaluation of performance metrics and complexity is more straightforward. The robustness metric is for guaranteeing test repeatability and interoperability. We propose the following procedure to derive the robustness metric as part of selection criterion:

Proposal 4: For all the candidate decoders, evaluate the robustness metrics based on test repeatability and interoperability by the following procedure

- Companies contribute encoders trained with the candidate decoders (one encoder per candidate decoder)

- Connect a encoder to the corresponding decoder, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the encoder and decoder pairs, record all the SGCS

- For each candidate decoder, capture the variation of the SGCSs when connecting to different encoders, and evaluate the variation together with performance metrics and complexity

Test Decoder Verification Procedure

In addition, for option 4, none of the options are complete enough for TE vendors to derive the test decoders that meet the test repeatability requirements. For example, in 4a, there are different ways to train the test decoders and the achieved loss functions from the specified dataset can vary a lot, and we still don’t have any test repeatability guarantee across different test decoders. Similarly, the joint training with reference encoder and yield different test decoders without guaranteed test repeatability. Therefore, additional requirements/specifications are needed for dataset based and reference encoder-based approach. We provide the suggestions for further specifications for each option in the following:

Dataset based approach 4a: we can specify a loss/similarity function of the test decoder output and the specified dataset for a given input as follows

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset, and f is the chosen loss/similarity function to evaluate the difference between the decoder output from the dataset corresponding to the latent message z, gref(z), and the test decoder output from z, gtest(z).

The drawback of the approach is that both the test decoder (and implicitly the DUT encoder) are heavily restricted by the dataset due to the specified constraint, and we don’t expect that the DUT encoder can work in practice unless in an environment that is very closely resembled with the specified dataset. In fact, the purpose of the test is, as agreed, “verify whether the minimum performance of AI/ML functionality/feature can be achieved”. But we don’t expect such dataset shared between the UE vendors and infra vendors is available for all the application scenarios, and therefore the performance achieved here is definitely not the minimum performance we can expect in the field. Alternatively, if we consider the approach proposed in the previous meeting (and included also in section 2.2), we can derive a more flexible requirement that can enable both TE vendors and UE vendors to develop decoders and encoders that can be trained by different datasets than the specified one. Such a flexibility can get us closer to the agreed test goal, since we allow the encoders and decoders to be trained by other dataset as long as the specified requirement in the form of latent message/decoder output ordering is fulfilled. To be more specified, the alternative requirement is:

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space.

Similarly, we can add the requirement for option 4b as

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input during test decoder verification procedures. Therefore, we also need to specified the generation procedures of X.

Alternatively, based on the proposal in section 2.2, we can specify the requirements of the test decoder as

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

In summary, we have the following proposal:

Proposal 5: Additional steps for option 4a and 4b feasibility study:

- Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

- RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Conclusion

Proposal 1: Consider the following steps for deriving the reference models (for encoder and decoder) for dataset generation:

- Step 1: Determine simulation setup and encoder input dataset generation procedure

- Step 2: Determine model type and parameter

- Step 3: Determine training hyper parameters

- Step 4: Determine the test decoder(s) by first determine evaluation methodology and selection/merge criterion, and then decide the test decoder(s) based on companies’ evaluation

Proposal 2: Consider the following guidelines based on R19 RAN4 discussions

- For simulation setup:

  - First decide whether to use system level or link level simulation for determining test decoder

  - Consider setup in RAN4 R19 TR as starting points, what are the parameters need to be updated

- RAN4 concluded that aggregated encoder input dataset is recommended for reference decoder evaluation since the performance variation across encoder/decoder pairs from different companies when input dataset from only one company is used

- RAN4 has selected MLP and CNN in R19 discussion, and these models can be the starting point for RAN1 discussion

- For training, RAN1 needs to decide whether to agree training related parameters, and if yes, what parameters to be agreed

Proposal 3: Consider the following as (test/reference) encoder/decoder selection criterion:

- Achievable performance metrics: SGCS or NMSE

- Complexity: including flops or model storage size, can consider to set an upper bound

- Robustness (based on performance metrics): whether the decoder can achieve satisfactory performance when connecting to encoder with different structures with the decoder, but trained with the decoder’s input and output dataset

Proposal 4: For all the candidate decoders, evaluate the robustness metrics based on test repeatability and interoperability by the following procedure

- Companies contribute encoders trained with the candidate decoders (one encoder per candidate decoder)

- Connect a encoder to the corresponding decoder, run the test based on the agree test dataset and record the SGCS

- Repeat step 2 for all the encoder and decoder pairs, record all the SGCS

- For each candidate decoder, capture the variation of the SGCSs when connecting to different encoders, and evaluate the variation together with performance metrics and complexity

Proposal 5: Additional steps for option 4a and 4b feasibility study:

        - Define the decoder verification criterion from one of the following options (for 4a and 4b, respectively):

(Option 4a: specify dataset)

- 4a-1: RAN4 derive the threshold δ via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

z∈Zfgref(z),gtest(z)<δ,

where Z is the set of decoder inputs (latent messages) in the specified dataset (from step 2), and f is the chosen loss/similarity function gref(z) is the test decoder output and gtest(z) is from the dataset.

- 4a-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where the notations are the same as 4a-1, and Ytest is the test decoder output space.

(Option 4b: specify reference encoder)

- 4b-1: RAN4 derive the threshold δ and the encoder input X generation procedure via simulation study (setup following option 3 discussion), and check whether it’s feasible to derive a test decoder satisfying

x∈Xfgref(z(x)),gtest(z(x))<δ,

where z(x) is the reference encoder output (latent message) of encoder input x, and X is the sampled encoder input in the test decoder verification procedures. Note that RAN4 needs to specified the generation procedures of X and specifying

- 4b-2: RAN4 studies whether it is feasible to derive a test decoder satisfying

gref(z)-gtest(z)=miny∈Ytestgref(z)-y for all z∈Z

where Ytest is the test decoder output space and Z is the reference encoder output space.

        - RAN4 check if the decoders derived from the above procedure can satisfy test repeatability requirement, i.e., the loss function delta between two different decoders when connecting to the same encoder, is within a RAN4 agreed margin. If yes, we can conclude that option 4 (at least one sub-option) is feasible.

Reference

[1] C.-H. Huang, C.-K. Wen, G. Y.  Li, “AI/ML Life Cycle Management for Interoperable AI Native RAN”, submitted to IEEE Communication Standard Magazine, available at http://arxiv.org/abs/2507.18538

[2] R4-2412130

[3] R4-2407236

[4] R4-2408294

[5] R4-2405653

[6] R4-2405187