3GPP TSG-RAN WG4 #116bis	R4- 2513559

Prague, Czech Republic, October 13-17, 2025

Agenda item:	7.5.4	Performance requirements for 6Rx UE with interference

Source:	Qualcomm Incorporated

Title:	Views on PDSCH demodulation performance requirements for 6Rx devices with interference

Document for:	Discussion

# Introduction

According to [1], 6Rx UE (both handheld and FWA) performance requirements with interference shall be defined in Rel-20. In this paper, we share our high-level views on demodulation requirements.

# Discussion

In the following sections we share our view on demodulation requirements for 6Rx devices.

## PDSCH Requirements

From [1], performance requirements for the following aspects shall be defined.

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference

  - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

    - Use the Rel-17 interference model for 2/4Rx UE as a baseline

    - Up to 4 MIMO layers for target cell

  - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Use the existing interference model for 2/4Rx UE as a baseline

    - Up to 2 MIMO layers for each user

In the recent RAN4#116 meeting, 8Rx demodulation requirements for both inter-cell and intra-user inter-cell scenarios were agreed. We would like to reuse these scenarios and test settings for 6Rx devices. Adopting this approach, our observations and proposals are:

Proposal 1: RAN4 to consider scenarios of Rel-19 8Rx inter-cell and intra-cell inter-user interference as starting point.

Proposal 2: RAN4 to consider Rank2 and Rank 4 scenarios for PDSCH demodulation requirements with inter-cell interference.

Proposal 3: RAN4 to consider Rank2 for both target and interfering UE for PDSCH demodulation requirements with intra-cell inter-user interference.

Proposal 4: RAN4 to have only a single requirement for both handheld and FWA devices.

Observation 1: RAN4 reused Rel-17 interference models for demodulation and CSI requirements for 8Rx devices.

Proposal 5: RAN4 to reuse only Rel-17 interference models for 6Rx devices too.

# Conclusions

In this paper we provided our views on PDSCH demodulation requirements with inter-cell and inter-user intra-cell interference. Our observations and proposals are summarized as follows:

Proposal 1: RAN4 to consider scenarios of Rel-19 8Rx inter-cell and intra-cell inter-user interference requirements as starting point.

Proposal 2: RAN4 to consider Rank2 and Rank 4 scenarios for PDSCH demodulation requirements with inter-cell interference.

Proposal 3: RAN4 to consider Rank2 for both target and interfering UE for PDSCH demodulation requirements with intra-cell inter-user interference.

Proposal 4: RAN4 to have only a single requirement for both handheld and FWA devices.

Observation 1: RAN4 reused Rel-17 interference models for demodulation and CSI requirements for 8Rx devices.

Proposal 5: RAN4 to reuse only Rel-17 interference models for 6Rx devices too.

# References

- RP-252957, WID: NR demodulation performance: Phase 6, Beijing, September 2025.