3GPP TSG-RAN4 Meeting #116bis R4-2513578 Prague, Czech, Oct. 13-17, 2025 CR-Form-v12.3. CHANGE REQUEST. 38.774; CR; -; rev; -; Current version:; 19.0.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; draft CR to TR 38.774 on LP-WUS. Source to WG:; vivo. Source to TSG:; R4. Work item code:; NR_LPWUS-Core; Date:; 2025-09-30. Category:; F; Release:; Rel-19. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; Due to limited time last meeting, some of the FR2 analysis was not captured in the TR. This CR is to supplement some missing FR2 LP-WUR RF requirements analysis.. Summary of change:; Adding FR2 SNR simulation outcome summary and SNR decision. Adding agreed FR2 NF+IM value and basic principle on deriving FR2 REFSENS requirements based on offset from MR.. Consequences if not approved:; The spec is not completed for FR2 LP-WUR RF analysis.. Clauses affected:; 6.3.2, 7.1.3, 7.1.4, 7.1.10. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:.

<<< Skip Unchanged Sections >>>

## <<< START OF CHANGES >>>

### 6.3.2	SNR simulations

In this sub-clause, SNR simulation results from different companies are collected for analysis of target SNR for LP-WUR.

Table 6.3.2-1 SNR simulation summary for FR1 Envelop-detection LP-WUR (OOK-4 M=4)

| Companies Input | SNR | RM coding | Timing error | Note |
| --- | --- | --- | --- | --- |
| A | -6 to -2dB | 32/16/8 | 4us | -2.15 dB (8 bit), -4.63 dB (16 bit), -5.94 dB (32 bit) |
| B | -7 to -3dB |  | 0us | 5th order of lowpass Butterworth |
| C | -7.5 dB to -2.5dB | 32/16/8 | 0us | simulated SNR with 4bit ADC or 8bit ADC is nearly the same |
| D | -7.1 to -2.5dB | 32/16/8 |  | OOK SNR: RM coding, 8bit with -2.5dB, 16bit with -5.3dB, 32bit with -7.1dB. |
| F | -7.7dB to -3.2dB | 32/16/8 | 0/0.9us | 0.9us TE will introduce less than 1dB degradation |
| G | -9.2 to -5.9dB |  |  | 8bit with -5.9dB, 16bit with -7.5dB, 32bit with -9.2dB. |
| H | -8.0 dB | 32 bits |  | under 4us TE has no impact |
| I | -7.8 to -5.9 dB | 32/16/8 |  | Payload: 5 bits with RM, w/o MC |

Observations and Summary of SNR simulation outcome for envelop-detection receiver: most companies perform analysis based on RM coding of 8/16/32 bits, and majority views prefer to use 16bit RM coding as RMC and derive target SNR. The final target SNR is agreed as -4.5dB for LP-WUS envelop-detection.

Table 6.3.2-2 SNR simulation summary for FR1 OFDM-based LP-WUR

| Companies Input | SNR | Timing error | Note |
| --- | --- | --- | --- |
| A | -4dB |  | no big difference for the applicable SNR for both OOK based and OFDM based signals |
| B | -4dB |  |  |
| C | -5.5dB |  |  |
| D | -7.4dB |  |  |
| F | -8.0dB |  | OFDM-based receiver outperforms OOK-based receiver by 2.5~3dB SNR |
| G | -13.8 to -9.9dB |  |  |
| H | -3.5dB |  |  |

Observations and Summary of SNR simulation outcome for OFDM-based receiver: There is no repetition for OFDM-based receiver, the simulated SNR from companies is not much far from the simulated performance of LP-WUS envelop-detection. With consideration of other impacted aspects, the final target SNR of OFDM-based receiver is also specified as -4.5dB.

For FR2, there were simulation results provided by companies, the FR2 OFDM-based receiver SNR performance is a little bit better than FR1. For the sake of uniformity, RAN4 agree the same value of -4.5 dB for FR2 LP-WUS SNR.

<<< Skip Unchanged Sections >>>

7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas.

In RAN4, there had been no consensus on what IM includes, and no agreement was achieved on individual values of NF and IM, so a joint number was agreed. There are two sets of NF + IM values agreed, and these will be used for REFSENS calculations. The values are,

- Set 1: 18 dB,

- Set 2: 13.5 dB.

The above mentioned numbers are for FR1.

For FR2, RAN4 also agreed a band-agnostic NF+IM assumption that NF + IM value is 12.0 dB for set 2.

7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + 10log10(BWLR)+ (NFLR + IMLR) + SNRLR.

Here, BWLR corresponds to the bandwidth of the LP-WUS signal in hertz. RAN1 has decided to have 11 RBs for LP-WUS irrespective of SCS, thus the LP-WUS bandwidth will be 1.98 MHz and 3.96 MHz, for 15 and 30 KHz SCS, respectively. Further, there are two sets of values for NFLR + IMLR, i.e. 18.0dB and 13.5dB for Type 1 and Type 2 LP-WURs, respectively. There is only one Rx antenna port for LP-WUR so there is no diversity gain included in the REFSENS.

In the REFSENS test, only the RBs allocated for LP-WUS are populated with the LP-WUS and the rest of the RBs for a given channel bandwidth are left empty. This configuration, however, does not correspond to the real-life case where NR signals in other RBs are generally present and thus can create a coverage shortfall. Separately, the ASCS requirement allows a maximum degradation of 0.5 dB and the side conditions incorporate populated adjacent RBs.

Given the small value of the ASCS degradation factor (0.5 dB), it was agreed not to include this factor in the REFSENS.

For FR1, a SNR value of -4.5 dB for both Type 1 and Type 2 LP-WURs was selected by RAN4 to be used for the REFSENS equation.

To keep the specifications simpler and given the frequency and NF assumptions made, it was agreed to keep the REFSENS same for majority of bands, irrespective of the band to band variation. For some specific bands with FDL_low higher than 2400 MHz and higher noise figure, there can be additional relaxation and that relaxation will be recorded in the specification.

For FR2, the SNR value is also -4.5dB for deriving REFSENS. For FR2, the same general principle is retained while recognizing that the basic expression for REFSENS also contains aspects of antenna gain and dual pol. combining. The LR REFSENS can be calculated from the legacy MR sensitivity requirements, agreements in RAN4 and basic principles:

-  NF+IM is 12 dB (1dB degraded from low FR2 band conn. mode PDSCH reception)

-  Diversity gain is -3 dB (3dB degraded from conn. mode PDSCH reception)

-  Antenna gain assumption is based on single element rather a 4-element array (6 dB degraded from conn. mode PDSCH reception)

-  The SNR proposal for FR2 LPWUS is -4.5 dB (3.5 dB improvement from MCS4)

-  BW adjustment (degraded by 10log(NRB/11))

The net degradation of LPWUS sensitivity (PC3 as example) from connected mode PDSCH requirement is therefore 6.5 + 10log(NRB/11) dB, i.e. (1+3 + 6 -3.5 +10log(NRB/11)) dB.

PREFSENS,LPWUS= PREFSENS,MR+6.5+10log10NRBNRB, LPWUS

<<< Skip Unchanged Sections >>>

7.1.10	Max Input Level

It is agreed that a single maximum input power level is specified for all channel bandwidths. It is also agreed to introduce two test cases. One test case without the NR signal and the other one NR signal with the same PSD. This is to make sure the LR operate in the presence of the NR signal near the cell centre.

Above basic principle applies to both FR1 and FR2 LP-WUR.

## <<< END OF CHANGES >>>