# 3GPP TSG-RAN WG4 Meeting #116bis                                                       R4-2513583

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Apple

Title:	CSI reporting requirement and testing framework for CSI prediction

Agenda item:	6.11.3

Document for:	Discussion

Introduction

This paper discusses the CSI reporting requirement framework for CSI prediction. In this paper, we address the performance monitoring for AI/ML-based CSI prediction use cases.

Discussion

Performance monitoring

The WI on AI/ML-based CSI prediction use case is specifying the reporting of performance. First, the delay and accuracy of performance monitoring metric is discussed in RAN4#115 [1].

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

In RAN4 agreement, monitoring reporting is under RAN1’s scope. RAN1#120-bis and RAN1#121 have made the agreements as follows.

RAN1#120-bis Agreement For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

In RAN1 agreement, SGCS is agreed the metric of CSI prediction using UE-side model for performance metric type 3. In Type 3, the UE computes the performance metrics, reports them to the network, which then determines whether to initiate functionality fallback to legacy CSI reporting.

RAN1#121 Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, - one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCS is reported - wideband frequency granularity - only for one prediction instance configured by NW - per layer, and the total number of layers is the same as the reported RI in the associated inference report - Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’ - Each SGCS value is quantized with 4-bit - 15 codepoints are used to uniformly quantize in range [0.3 1] in linear scale. - One codepoint is used to indicate the value range of (0 0.3] - Only one monitoring resource set is configured in the CSI-ReportConfig - The one configured time instance (i.e. f-th doppler domain unit in one inference report) for SGCS calculation is configured in the CSI-ReportConfig for monitoring, for N4>1 Agreement For CSI prediction using UE-side model, support single UCI part with the following report format (i.e., CSI field mapping order) among a set of SGCSs. SGCS1 #1. SGCS1 #2. …. SGCS1 #v. SGCS2 #1. SGCS2 #2. …. SGCS2 #v. - SGCS1 is calculated based on predicted CSI for one inference reporting, and ground truth CSI, - SGCS2 is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance - SGCSi #k is the SGCS of k-th layer of i-th SGCS where k ={1, …., v}, and i ={1, 2}. - v is the value of the reported RI in the associated inference report.

During RAN4#116 the following agrrements were reached:

Agreement:

RAN4 will introduce requirements for Type 3 performance monitoring for CSI prediction

FSS which requirements are to be introduced.

Issue 1-2: Requirement baseline for monitoring

Agreement:

Introduce the following requirements for CSI performance monitoring:

- Reporting delay

  - FFS how the delay is defined

- Reporting accuracy

  - FFS on whether accuracy requirement can be defined/checked

- Mapping table (38.1.3.3 section 10)– To be checked whether RAN1 captures or RAN4.

- If it is not feasible to define reporting accuracy requirement or test it, RAN4 will send an LS to RAN1 to inform RAN1 about this

Based on the RAN1#121 agreement, the requirements for CSI prediction using UE-side models and reporting contents of UE-assisted performance monitoring are defined.

For CSI prediction using UE-side models, the agreement specifies that two types of SGCS values are calculated and reported. The first type, SGCS1, is based on the predicted CSI and the ground truth CSI. The second type, SGCS2, is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference reporting instance.

However, for the network to rely on these predictions, it must continuously monitor the prediction quality and decide when to trust the reported CSI or revert to legacy CSI reporting. Today there is no standardized, verifiable way to ensure that a UE-reported similarity metric truly reflects the accuracy of its prediction. Without clearly defined accuracy requirements and corresponding test procedures, a malfunctioning or mis-implemented predictor can silently degrade link adaptation, scheduling, or mobility performance while still presenting apparently healthy CSI reports to the network.

Issues with Only SGCS1
SGCS1 measures the similarity between the UE’s predicted CSI and the UE’s own ground-truth CSI, i.e., CSI derived from standard CSI-RS measurements on the configured reference resource. Although SGCS1 provides a direct view of prediction quality in principle, the network cannot directly observe how the UE determines its ground truth or performs the similarity computation. This creates room for errors or even manipulation, for example, the UE could inadvertently use outdated channel samples, or, in a worst case, use an internally simulated (“genie”) channel, producing a high SGCS1 even when the actual prediction is inaccurate. If RAN4 were to rely only on SGCS1, it would leave operators exposed to silent failures, mis-implementation, or intentional misreporting that could compromise overall system performance.

Dual-Metric Safeguard with SGCS1 and SGCS2
The standardized introduction of SGCS2, defined as the similarity between the same ground truth and the most recent non-predicted CSI measurement, provides a critical second layer of protection. Under realistic, dynamic channel conditions, a functioning AI/ML predictor should consistently yield SGCS1 values significantly higher than SGCS2, reflecting the genuine performance gain of prediction over legacy CSI. Conformance and network monitoring can therefore look not only at absolute SGCS1 thresholds but also at the relationship between SGCS1 and SGCS2. Any large inconsistency or unexpected correlation (e.g., SGCS1 not appreciably exceeding SGCS2) flags potential mis-reporting or degraded predictor performance. Because SGCS1 and SGCS2 are both quantized per layer and computed over a single configured prediction instance, they can be independently recomputed by test equipment using known channel realizations, enabling objective validation without requiring insight into the UE’s internal ML model.

RAN4 can strengthen UE-side CSI prediction monitoring by pairing independent verification with explicit performance thresholds. During conformance testing, the test equipment (TE) already controls and knows the true radio channel. This enables the TE to recompute both SGCS1 (predicted CSI vs. true channel) and SGCS2 (most recent non-predicted CSI vs. true channel) using the UE’s reported data and the known channel realizations. By comparing these recomputed values with those reported by the UE, the TE verifies that the SGCS metrics faithfully represent the physical channel without requiring access to the UE’s internal ML model.

Building on this verification capability, RAN4 should define two complementary performance requirements. First, an absolute SGCS1 floor ensures that the predictor’s overall accuracy remains above a minimum acceptable level. For example, within a sliding window of 20–40 reports, the 5th–10th percentile of SGCS1 should exceed a target T_abs (value for further study). Second, a dual-metric margin ensures that prediction provides measurable gain over legacy CSI: within the same window at least 95 % of reports should satisfy SGCS1 – SGCS2 ≥ Δ_min (about one to two quantization codepoints or roughly 0.03–0.06 in linear SGCS). These requirements capture both absolute accuracy and relative improvement.

To provide operational stability, RAN4 can further require persistence and hysteresis before triggering lifecycle management (LCM) actions such as model fallback. For example, the UE would only be deemed non-compliant if either the absolute floor or dual-metric margin is violated for two consecutive windows, and the alarm would clear only after two good windows. Additional refinements: per-layer checks tied to the reported rank indicator, and mobility-specific threshold tables: would adapt the requirements to realistic Doppler conditions. Together, TE-based SGCS verification and these layered performance thresholds give operators a trusted, standardized basis for deciding when to continue, switch, or retrain AI/ML CSI prediction models.

Dual-Metric Requirements for UE-Side CSI Prediction

Ensure network trust in UE-reported SGCS and define LCM criteria by (i) enabling the test equipment (TE) to independently recompute SGCS1/SGCS2 and (ii) defining absolute and relative SGCS performance thresholds as triggers for LCM actions.

- (Accuracy of reported KPI) Feasibility study of TE-based ground truth derivation: TE emulates known channels, derives SGCS1 and SGCS2 as reference values, and validates UE-reported results against this recomputed ground truth.

- (LCM) Absolute SGCS1 floor: Require that, over a defined monitoring window, a specified lower percentile of SGCS1 remains above an agreed minimum accuracy level. Scenario specific threshold

- (LCM )Dual-metric margin: Require that, within the same window, a high proportion of reports show SGCS1 exceeding SGCS2 by at least an agreed minimum margin.

- (LCM) Persistence and hysteresis: Trigger LCM fallback only when either requirement is violated consistently across consecutive monitoring windows and clear the condition after sustained compliance.

- (LCM) Per-layer and RI coupling: Apply checks per reported layer and adjust thresholds if the rank indicator changes.

- (LCM) Mobility/Doppler profiles: Provide profile-specific thresholds to reflect differences in channel dynamics.

For AI/ML-based CSI prediction, the network relies on UE-reported SGCS metrics to judge whether predicted CSI is trustworthy and when to trigger fallback to legacy CSI reporting. The first SGCS report after the network activates prediction is critical, because only after receiving it can the network confirm that the predictor is running correctly. However, current specifications do not define how quickly the UE must deliver this initial performance-monitoring report once the AIML functionality and monitoring resources are configured. Without a standardized delay requirement, some UEs could take an excessively long time to initialize their models, gather measurements, or schedule the first report, leaving the network blind to early prediction failures and potentially degrading link adaptation or mobility performance.

RAN4 should define a reporting-delay requirement as the elapsed time from completion of the RRC (re)configuration message, including AIML functionality activation and monitoring-resource setup, to the transmission of the first SGCS-based performance-monitoring report. This definition aligns with the beam-management precedent and captures all key delay contributors: configuration processing, model activation, and UE scheduling of the first monitoring instance.

Conclusion

In this contribution, we have discussed the RRM impact of AI/ML-based CSI prediction. The proposals are presented as follows.

- Dual-Metric Requirements for UE-Side CSI Prediction

Ensure network trust in UE-reported SGCS and define LCM criteria by (i) enabling the test equipment (TE) to independently recompute SGCS1/SGCS2 and (ii) defining absolute and relative SGCS performance thresholds as triggers for LCM actions.

- (Accuracy of reported KPI) Feasibility study of TE-based ground truth derivation: TE emulates known channels, derives SGCS1 and SGCS2 as reference values, and validates UE-reported results against this recomputed ground truth.

- (LCM) Absolute SGCS1 floor: Require that, over a defined monitoring window, a specified lower percentile of SGCS1 remains above an agreed minimum accuracy level. Scenario specific threshold

- (LCM )Dual-metric margin: Require that, within the same window, a high proportion of reports show SGCS1 exceeding SGCS2 by at least an agreed minimum margin.

- (LCM) Persistence and hysteresis: Trigger LCM fallback only when either requirement is violated consistently across consecutive monitoring windows and clear the condition after sustained compliance.

- (LCM) Per-layer and RI coupling: Apply checks per reported layer and adjust thresholds if the rank indicator changes.

- (LCM) Mobility/Doppler profiles: Provide profile-specific thresholds to reflect differences in channel dynamics.

RAN4 should define a reporting-delay requirement as the elapsed time from completion of the RRC (re)configuration message, including AIML functionality activation and monitoring-resource setup, to the transmission of the first SGCS-based performance-monitoring report. This definition aligns with the beam-management precedent and captures all key delay contributors: configuration processing, model activation, and UE scheduling of the first monitoring instance.

- References

- 3GPP TSG-RAN WG4 Meeting #116, R4-2511890, “WF on AI/ML for air interface for [116][131],” Qualcomm Incorporated, Bengaluru, India, 25 – 29 Aug 2025, Agenda item 7.17.1.