# 3GPP TSG-RAN WG4 Meeting #116bis		R4-2513591

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Apple

Title:	General aspects on AI/ML for mobility in NR

Agenda item:	7.12.2

Document for:	Discussion

Introduction

During the RAN4#112bis, the discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) will start in RAN4. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In this discussion paper we overview the main objectives and general aspects of AI/ML for mobility in NR.

Discussion

Main objectives with new WID and RAN2 status

The FS_NR_AIML_Mob focuses on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made at the network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

In addition to the RAN4 objectives already presented in the Introduction section above, the following RAN2 objectives and use-cases are listed in the SID [1]:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2]

During TSG RAN #109 the updated WID [RP-252899] captured the objectives of SI or Core part WI or Testing part WI:

The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline.

The objectives are as the following:

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]:

- Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported.

- L3 Cell level predication is supported

Note 1: For UE sided model, spatial domain prediction is not supported.

Note 2: For UE sided model, L3 beam level prediction is not supported.

- For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]:

- UE capability request and response procedure

- Applicability report for both full and partial configuration approach

- Inference configuration and inference report

- Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality

- UE sided data collection

- For measurement event prediction:

  - Measurement event A1~A6 are in the scope in this release

Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation.

- For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]:

- Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction

- Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction

- Network sided data collection for all scenarios and  all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least

    - UE is configured to log RRM measurement results.

    - UE supports periodical logging and L3 event triggered logging.

    - UE can indicate logged data availability.

Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction.

Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]:

- Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction

  - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction

- For direct event prediction, study feasibility of core requirement in Q2 2026.

- Specify LCM-related core requirements to support the LCM functionality management , if any

Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

We would like to outline some directions for the future working plan.

Frequency Domain Correlation Model

In inter-frequency RSRP prediction, the degree of frequency-domain channel correlation fundamentally determines how well information from a measured layer can predict another layer’s RSRP. The correlation is shaped by large-scale propagation parameters (delay spread, angle spread, LOS probability) and small-scale multipath evolution across frequencies. 3GPP 38.901 notes that while some parameters (e.g., cluster delays/angles) may be partly shared across frequencies, others, such as per-cluster shadowing, frequency-dependent cluster powers, and independent small-scale phases, are generated separately, weakening true physical correlation. Antenna pattern differences and bandwidth variations further distort frequency-domain coherence, so a simple single-number “correlation coefficient” is unlikely to capture all relevant physics.

Among these influences, the structure of the power-delay profile (PDP) and how power clusters evolve with frequency is the most critical. PDP determines how multipath energy is distributed over delays; any change in delay spread, cluster power scaling, or blockage with frequency directly alters the wideband RSRP that AI/ML models must predict. Dynamic EPRE differences add bias but can be corrected if slowly varying. Thus, realistic multi-frequency PDP correlation, beyond a single aggregate coefficient, is essential for both link-level accuracy and system-level simulation.

Based on the above observations, we have the following proposal:

Spatial- RAN4 should revisit the frequency-correlation modeling across power-delay profiles and RSRP statistics for inter-frequency prediction. Instead of relying on a single scalar correlation coefficient, the model should describe how each delay bin or cluster correlates across frequencies, incorporating effects such as antenna pattern mismatch, frequency-scaled delay/angle spreads, and frequency-dependent shadowing and blockage. This richer representation will enable simulation setups that preserve the true joint statistics of multi-frequency channels, ensuring that AI/ML inter-frequency prediction requirements remain realistic

Comprehensive Measurement-Error and Multi-Cell Clustering Framework

RAN4 has already acknowledged the need for a basic measurement-error model, but real deployments show far more complexity. Environmental and network-side filtering add additional variability: multi-cell interference and neighbor-cell timing offsets distort the UE’s instantaneous RSRP, while L1 and L3 filtering configurations can either smooth out transient noise or introduce systematic biases that shift RSRP averages. These effects cascade into AI/ML models, influencing both absolute and relative RSRP predictions and ultimately altering key event triggers such as A3.

At the prediction stage, errors interact with AI/ML methods in even more intricate ways. A mismatch between training and live error levels can erode model generalization; cluster-based multi-cell prediction may either amplify or average out per-cell errors; and inter-frequency scenarios introduce unique frequency-dependent error characteristics. Measurement errors also affect lifecycle management (LCM), skewing online performance indicators and potentially causing unnecessary model switches or delayed fallbacks. Together, these factors show that a single static error term is inadequate for capturing the true dynamics of measurement errors in practical networks

AI/ML model performance can be further influenced when measurements from a group of cells in one carrier frequency are used as input, since diverse inputs enrich the model’s spatial and frequency fingerprints. Clustering is particularly valuable for inter-frequency measurement prediction, where a single serving cell provides only a narrow view of the radio environment. By combining data from multiple serving and neighbor cells, the model can average out transient anomalies—such as short-term beam misalignment or rapid fading—and extract stable long-term correlations between frequency layers. This approach not only increases robustness to momentary disturbances but also improves generalization across heterogeneous deployments with varying topologies, power settings, and beamforming patterns, allowing more reliable RSRP prediction even in regions with sparse direct measurements.

Comprehensive Measurement-Error and Multi-Cell Clustering Framework for Inter-Frequency Prediction

RAN4 shall expand beyond a single static error model and develop a multi-factor measurement-error and clustering framework to ensure AI/ML-based RRM prediction reflects real network conditions.
The framework shall:

- Capture RF/baseband noise, and dataset misalignment between training and live operation if possible

- Include environmental drivers such as multi-cell interference, neighbor-cell timing offsets, and dynamic EPRE changes.

- Model filtering influences (L1 and L3) that can smooth or bias RSRP measurements.

- Evaluate how errors propagate through AI/ML methods, covering temporal patterns, cluster-based multi-cell prediction, and inter-frequency dependencies.

- Integrate multi-cell clustering to provide richer, more diverse spatial fingerprints, improving model robustness and generalization.

Conclusion

In this paper we listed the objectives and the RAN2 progress on the FS_NR_AIML_Mob [4]. We have additionally outlined some directions for the future working plan. The Proposals were made:

- Spatial- RAN4 should revisit the frequency-correlation modeling across power-delay profiles and RSRP statistics for inter-frequency prediction. Instead of relying on a single scalar correlation coefficient, the model should describe how each delay bin or cluster correlates across frequencies, incorporating effects such as antenna pattern mismatch, frequency-scaled delay/angle spreads, and frequency-dependent shadowing and blockage. This richer representation will enable simulation setups that preserve the true joint statistics of multi-frequency channels, ensuring that AI/ML inter-frequency prediction requirements remain realistic

Comprehensive Measurement-Error and Multi-Cell Clustering Framework for Inter-Frequency Prediction

RAN4 shall expand beyond a single static error model and develop a multi-factor measurement-error and clustering framework to ensure AI/ML-based RRM prediction reflects real network conditions.
The framework shall:

- Capture RF/baseband noise, and dataset misalignment between training and live operation if possible

- Include environmental drivers such as multi-cell interference, neighbor-cell timing offsets, and dynamic EPRE changes.

- Model filtering influences (L1 and L3) that can smooth or bias RSRP measurements.

- Evaluate how errors propagate through AI/ML methods, covering temporal patterns, cluster-based multi-cell prediction, and inter-frequency dependencies.

- Integrate multi-cell clustering to provide richer, more diverse spatial fingerprints, improving model robustness and generalization.

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.

- R2-24xxxxx, Discussion on work plan of AI mobility, RAN2#127bis, Hefei, China, Oct 14th – 18th, 2024.

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Hefei, China, Oct 14th – 18th, 2024.

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, Source: OPPO, InterDigital, Beijing, China, Sept. 15–18, 2025, Document for: Approval, Agenda Item: 9.3.2.1.