3GPP TSG-RAN WG4 Meeting #116bis                                  	                        R4-2513598

# Prague, Czech Republic, October 13 – October 17, 2025

Agenda item:	7.5.4

Source:	Apple Inc.

Title:	On Demodulation Performance Requirements for 6Rx UEs with Inter-Cell and Intra-Cell Inter-User Interference

Release:	Rel-20

Document for:	Discussion

# Introduction

A new WID on NR demodulation performance has been created for 5G Advanced in Release 20. The WID covers two main areas: First, specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1, and second, to define performance requirements for 6Rx UE (both handheld and FWA) in interference scenarios. In this contribution we focus on the second objective, and the relevant objectives in the WID are captured below:

- Define the following performance requirements for 6Rx UE (both handheld and FWA) with interference

  - PDSCH demodulation with inter-cell interference with MMSE-IRC receiver

    - Use the Rel-17 interference model for 2/4Rx UE as a baseline

    - Up to 4 MIMO layers for target cell

  - Specify CQI reporting requirements with inter-cell interference with MMSE-IRC receiver

    - Note 3: The start of the work is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

  - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver

    - Use the existing interference model for 2/4Rx UE as a baseline

    - Up to 2 MIMO layers for each user

# General Aspects

We start by discussing the general aspects and considerations for demodulation performance requirements for 6Rx UEs, taking previous work in Rel-17 and Rel-19 as a baseline. As per the current WID [1], the focus should be on Fixed Wireless Access (FWA) devices and on Handheld UEs. In the past Rel-18 8Rx discussion there were further scenarios that we don’t think are relevant at this point.

Observation #1: RAN4 defined requirements for inter-cell and intra-cell inter-user interference cancellation with 2Rx and 4Rx in R17, and later for 8Rx in Rel-19.

Proposal #1: Define requirements for FWA and Handheld UEs, but do not consider additional scenarios such as Vehicular and Industrial UEs.

During Rel-19 discussion there was a long debate on receiver architecture assumptions, i.e. about the difference between a baseline 8Rx receiver and the simplified 8Rx receiver. This also went into several discussions about having different set of requirements, performance differences between receiver assumptions, etc. In addition, during Release 18, 2Rx/4Rx requirements with the assumption of an R-ML receiver were defined. During 6Rx Release 20 discussion, none of the above is in scope, but only the assumption of a MMSE-IRC receiver.

Observation #2: In contrast to Relase 18 and Release 19 requirements, the only receiver assumption for Release 20 scope is the baseline MMSE-IRC receiver.

Proposal #2: Confirm assumption that a single type of receiver, i.e. only a MMSE-IRC is considered.

On the other hand, there are several parameters and assumptions that should be discussed and clarified in general, so the new requirements discussion focuses on critical aspects. For instance, most of the general test parameters can be reused from the previous Rel-19 8Rx discussion.

Observation #3: There are many test configuration parameters that can be reused from the requirements of previous releases.

One clear example is duplex modes and channel bandwidth.

Proposal #3: Reuse previous channel bandwidth and subcarrier spacing assumptions. Consider 15kHz/10MHz for FDD and 30kHz/40MHz with 7D1S2U for TDD.

Another common debate is the PDSCH Mapping Type. We believe, as discussed in Release 19, we should only consider Mapping Type A. Mapping Type B requirements have been defined in previous releases, and assuming inter-cell and intra-cell interference doesn’t affect UE processing only specific to Mapping Type B.

Observation #4: There’s no meaningful UE signal processing difference between Mapping Type A and B when considering inter-cell and intra-cell interference.

Proposal #4: Focus PDSCH requirements on Mapping Type A only.

# PDSCH Requirements with Inter-Cell Interference

In this section we focus on the aspects specific to PDSCH requirements with inter-cell interference. The first aspect to consider is the interference model and how many interfering cells are deployed. We believe that there’s no need to innovate here, and same assumptions as in Rel-19 should apply.

Observation #5: Previous inter-cell interference requirements asume the case of 1 and 2 interfering cells, considering HomNet and HetNet scenarios.

Proposal #5: Reuse the inter-cell interference model specified in Clause B.6.2 of TS 38.101-4, reusing the same interference profiles, random 16QAM modulated symbols, and interference-to-noise (INR) values previously defined in Release 17 for 2Rx/4Rx and in Release 19 for 8Rx.

In addition to the interference model, another key discussion is the transmission rank assumption. Here we also believe no major innovation is needed.

Observation #6: At the serving cell, existing Release 17 2Rx/4Rx requirements for inter-cell interference consider Rank 1 at the serving cell, while Release 19 8Rx requirements for inter-cell interference consider Rank 2 and Rank 4. At the interference cells, both Release 17 and Release 19 consider random rank with 70% and 30% probability for Rank 1 and Rank 2.

During Release 20, we will define requirements for both FWA and Handheld UEs. During Release 17. requirements were defined for any kind of UE, while during Release 19 the requirements focused on FWA devices only.

Proposal #6: For 6Rx UEs, consider Rank 1 and Rank 2 transmission at the serving cell.

Finally, with respect to propagation conditions, we believe that both TDLC300-100 and TDLA30-10 can be considered as a starting point for handheld devices. For FWA devices, TDLA30-10 is sufficient.

Observation #7: Typical propagation conditions for FWA and handhel devices differ in both frequency and time selectivity.

Proposal #7: For inter-cell interference requirements for 6Rx handheld UEs, consider TDLC300-100 and TDLA30-10 as a starting point. For FWA devices, consider TDLA30-10 only.

# PDSCH Requirements with Intra-Cell Inter-User Interference

In this section we focus on the aspects specific to PDSCH requirements with intra-cell inter-user interference.

The first topic that needs to be address is the interference modelling to be considered, which we should maintain as the Annex B.4.2 Beamforming for MU-MIMO in TS 38.101-4. This has been used in the past, including in recent discussions in Release 19 for 8Rx.

Observation #8: Since Release 17 2Rx/4Rx and Release 19 8Rx discusions, same MU-MIMO beamforming model has been used, i.e. Annex B.4.2 in TS 38.101-4.

Proposal #8: Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2 for 6Rx UE intra-cell inter-user requirements, considering one target UE plus one co-scheduled UE.

Another point to discuss for these requirements is the transmission ranks for the target UE and the co-scheduled UE. As per previous discussions, we believe that Rank 2+2 is the most balanced approach. Finally, we also propose to consider the same precoding configurations for target and co-scheduled UE as in Release 19 discussion.

Proposal #9: For 6Rx with intra-cell inter-user interference UEs, consider Rank 2+2 for target and co-scheduled UEs, with a corresponding 2T4R antenna configuration.

Proposal #10: For 6Rx UEs with intra-cell inter-user interference, reuse same precoding configurations as in Release 19 8Rx requirements, i.e. randomized orthogonal precoder.

Finally, with respect to propagation conditions, again, we believe that both TDLC300-100 and TDLA30-10 can be considered as a starting point for handheld devices, while for FWA devices the TDLA30-10 model is sufficient.

Proposal #11: For intra-cell inter-user interference requirements for 6Rx handheld UEs, consider TDLC300-100 and TDLA30-10 as a starting point. For FWA devices, consider TDLA30-10 only.

# CQI Reporting with Inter-Cell Interference

Observation #9: As per current WID, the start of the discussion on CQI reporting with interference is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: RAN4 defined requirements for inter-cell and intra-cell inter-user interference cancellation with 2Rx and 4Rx in R17, and later for 8Rx in Rel-19.

Proposal #1: Define requirements for FWA and Handheld UEs, but do not consider additional scenarios such as Vehicular and Industrial UEs.

Observation #2: In contrast to Relase 18 and Release 19 requirements, the only receiver assumption for Release 20 scope is the baseline MMSE-IRC receiver.

Proposal #2: Confirm assumption that a single type of receiver, i.e. only a MMSE-IRC is considered.

Observation #3: There are many test configuration parameters that can be reused from the requirements of previous releases.

Proposal #3: Reuse previous channel bandwidth and subcarrier spacing assumptions. Consider 15kHz/10MHz for FDD and 30kHz/40MHz with 7D1S2U for TDD.

Observation #4: There’s no meaningful UE signal processing difference between Mapping Type A and B when considering inter-cell and intra-cell interference.

Proposal #4: Focus PDSCH requirements on Mapping Type A only.

Observation #5: Previous inter-cell interference requirements asume the case of 1 and 2 interfering cells, considering HomNet and HetNet scenarios.

Proposal #5: Reuse the inter-cell interference model specified in Clause B.6.2 of TS 38.101-4, reusing the same interference profiles, random 16QAM modulated symbols, and interference-to-noise (INR) values previously defined in Release 17 for 2Rx/4Rx and in Release 19 for 8Rx.

Observation #6: At the serving cell, existing Release 17 2Rx/4Rx requirements for inter-cell interference consider Rank 1 at the serving cell, while Release 19 8Rx requirements for inter-cell interference consider Rank 2 and Rank 4. At the interference cells, both Release 17 and Release 19 consider random rank with 70% and 30% probability for Rank 1 and Rank 2.

Proposal #6: For 6Rx UEs, consider Rank 1 and Rank 2 transmission at the serving cell.

Observation #7: Typical propagation conditions for FWA and handhel devices differ in both frequency and time selectivity.

Proposal #7: For inter-cell interference requirements for 6Rx handheld UEs, consider TDLC300-100 and TDLA30-10 as a starting point. For FWA devices, consider TDLA30-10 only.

Observation #8: Since Release 17 2Rx/4Rx and Release 19 8Rx discusions, same MU-MIMO beamforming model has been used, i.e. Annex B.4.2 in TS 38.101-4.

Proposal #8: Reuse MU-MIMO Beamforming in TS 38.101-4 Annex B.4.2 for 6Rx UE intra-cell inter-user requirements, considering one target UE plus one co-scheduled UE.

Proposal #9: For 6Rx with intra-cell inter-user interference UEs, consider Rank 2+2 for target and co-scheduled UEs, with a corresponding 2T4R antenna configuration.

Proposal #10: For 6Rx UEs with intra-cell inter-user interference, reuse same precoding configurations as in Release 19 8Rx requirements, i.e. randomized orthogonal precoder.

Proposal #11: For intra-cell inter-user interference requirements for 6Rx handheld UEs, consider TDLC300-100 and TDLA30-10 as a starting point. For FWA devices, consider TDLA30-10 only.

Observation #9: As per current WID, the start of the discussion on CQI reporting with interference is postponed until when the decision to define the R19 CQI requirements without interference is made, and will only start when there is R19 CQI requirements without interference.

# References

- RP-252957 “New WID: NR Demodulation Performance, Phase 6”, by RAN4 chair (Apple), 3GPP TSG-RAN Meeting #109, Beijing, China, September 15-18, 2025