# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513607

# Prague, Czech Republic, October 13 – 17, 2025

Agenda item:	6.3.2

Source:	Apple

Title:	On RRM performance for R19 LB CA

Document for:	Discussion

# Introduction

The WF on RRM requirements for NR_LBCA_Sw [1] have been agreed in RAN4#116, and the open issues have been summarized in the topic summary for [116][206] NR_LBCA_Sw [2]. The work plan for the performance part has been agreed in [3].

3GPP RAN4 #116bis meeting (October, 2025, 0.25TU, Performance part) - Discuss and agree on the test case list - Subject to the discussion, the list may have a work splitting among companies - Continue discussion on the details of test cases

In this contribution, we continue the discussion for RRM performance part of LB CA via switching.

# Test case list and configurations

##### In previous RAN4 meeting, the key requirements for PCell and SCell in LB CA were summarized as in WF R4-2508259, as follows,

| Index | Requirement | note |
| --- | --- | --- |
| 1 | Activated SDL SCell L3 measurement | Activated and deactivated SCell measurement delay requirement can be in the same section. All details are depending on the discussion. |
|  | Deactivated SDL SCell measurement |  |
| 2 | Interruption for deactivated SDL SCell measurement |  |
| 3 | SDL SCell activation/deactivation and corresponding interruption | All scope and details are depending on the discussion. |
| 4 | BFD/CBD for SDL SCell and PCell | (1) BFD/CBD requirement for SDL SCell. (2) The impact of switching pattern to PCell BFD/CBD requirement |
| 5 | TCI state switch delay for SDL SCell and PCell | (1) TCI state switch delay requirement for SDL SCell. (2) The impact of switching pattern to PCell TCI state switch delay requirement |
| 6 | PL-RS switch delay for PCell | The impact of switching pattern to PCell PL-RS switching requirement |
| 7 | L1-RSRP/SINR for SDL SCell and PCell | (1)L1-RSRP/SINR measurement requirement for SDL SCell. (2) The impact of switching pattern to PCell L1-RSRP/SINR measurement requirement |
| 8 | PCC L3 measurement delay requirement | (1) PCC L3 requirement impact can also be captured if needed. (2) PCell RLM requirement impact can also be captured if needed. |
|  | RLM requirement for PCell |  |

Even though the switching pattern will impact both PCell and SDL SCell operations, we don’t think it’s very necessary to test both PCell and SDL SCell for the same RRM operations, from the functionality perspective. In order to simplify the testing and control the test case number, we propose to have one test case for each requirement. The test case shall be specified for corresponding requirement on SDL SCell, unless only PCell has such requirement.

Proposal 1: introduce one test case for each newly specified requirement of LB-CA.

Proposal 2: The test case shall be specified for corresponding requirement on SDL SCell, unless only PCell has such requirement.

Based on the proposal 1, the following test case list is suggested:

Proposal 3: to agree the following test case list for LB-CA RRM.

| TC Index | Requirement | RS used for test | Target Cell for testing |
| --- | --- | --- | --- |
| 1 | Activated SDL SCell L3 measurement | SSB | SDL SCell |
| 2 | Deactivated SDL SCell measurement and interruption requirement | SSB | SDL SCell |
| 3 | SDL SCell activation/deactivation and corresponding interruption | SSB and CSI-RS | SDL SCell |
| 4 | BFD/CBD for SDL SCell and PCell | SSB | SDL SCell |
| 5 | TCI state switch delay for SDL SCell and PCell | SSB | SDL SCell |
| 6 | PL-RS switch delay for PCell | SSB | FDD PCell |
| 7 | L1-RSRP/SINR for SDL SCell and PCell | SSB | SDL SCell |
| 8 | RLM requirement for PCell | SSB | FDD PCell |

Regarding the test case configuration, one key aspect is the switching pattern configuration in the test cases. In previous RAN4 agreement, one principle for requirement design is as following:

Issue 1-1-1: RRM requirement applicability for R19 LB CA via switching - Agreement in online session: - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

The periodicity is 40ms for switching pattern as defined in RAN1 so far, and SSB/SMTC periodicity in RRM requirement can be configured 20ms or 40ms in most typical test cases. In our view, only one scenario is sufficient for testing among between scenario a and scenario b above. In order to test the worst case, we propose to only test scenario b, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern.

Proposal 4: only test partially overlapped scenario for LB CA RRM, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern.

- For example, SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms

# Conclusion

In this contribution, we discuss the scope of corresponding RRM requirement for this R19 LB CA scenario.

Proposal 1: introduce one test case for each newly specified requirement of LB-CA.

Proposal 2: The test case shall be specified for corresponding requirement on SDL SCell, unless only PCell has such requirement.

Proposal 3: to agree the following test case list for LB-CA RRM.

| TC Index | Requirement | RS used for test | Target Cell for testing |
| --- | --- | --- | --- |
| 1 | Activated SDL SCell L3 measurement | SSB | SDL SCell |
| 2 | Deactivated SDL SCell measurement and interruption requirement | SSB | SDL SCell |
| 3 | SDL SCell activation/deactivation and corresponding interruption | SSB and CSI-RS | SDL SCell |
| 4 | BFD/CBD for SDL SCell and PCell | SSB | SDL SCell |
| 5 | TCI state switch delay for SDL SCell and PCell | SSB | SDL SCell |
| 6 | PL-RS switch delay for PCell | SSB | FDD PCell |
| 7 | L1-RSRP/SINR for SDL SCell and PCell | SSB | SDL SCell |
| 8 | RLM requirement for PCell | SSB | FDD PCell |

Proposal 4: only test partially overlapped scenario for LB CA RRM, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern.

- For example, SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms

# References

[1] R4-2512119, WF on RRM requirements for NR_LBCA_Sw, Apple, RAN4 #116

[2] R4-2509049, Topic summary for [116][206] NR_LBCA_Sw, Apple, RAN4#116

[3] R4-2509990, Work plan of performance part for R19 NR_LBCA_Sw RRM, Apple, RAN4#116