3GPP TSG-RAN WG4 Meeting #116bis	 					     						     R4-2513611

# Prague, Czech Republic, 13th – 17th October 2025

Agenda item:			4.7.1

Source:	Apple

Title:	Alignment of RAN4 requirement with RAN2 signalling

Document for:	Discussion

# Introduction

Big CRs for supporting non-collocated NR CA and EN-DN got approved in RAN 4#116 meeting [1, 2]. While the requirement seems not fully align with RAN2 design. This paper will review RAN2 signaling design at first and then propose further modification for RAN4 spec.

# Discussion

## RAN2 signalling design

RAN2 signaling on UE capability and network indicator for R19 are summarized as below:

- UE capability

      - Introduced per-BC UE capability interBandMRDC-WithOverlapDL-Bands-r19 into MRDC-Parameters.

      - Introduced per-BC UE capability intraBandNR-CA-non-collocated-r19 into CA-ParametersNR.

      - If a UE supports Type 4 UE capabilities, it also reports support on corresponding Type 2 UE capabilities.

- BS signaling

        - Introduced Rel-19 nonCollocatedTypeNR-CA/MRDC-r1900 with two code points {Type 1, Type 4} into CellGroupConfig, applicable when MaxMIMO-layers is configured as 4.

        - If Rel-19 nonCollocatedTypeNR-CA/MRDC-r1900 is absent, Rel-19 UE(s) apply Rel-18 principle accordingly.

Below table gives the R19 UE behavior with regard to MIMO configuration and Collocated/Non-collocated indicator in network signaling.

Table 2.1-1 R19 type 4 UE behavior

| MIMO configuration | Collocated/Non-collocated Indicator | R19 UE operation |
| --- | --- | --- |
| R18 gNB (Configuration <=2 MIMO layers) | nonCollocatedTypeNR-CA/MRDC-r18 = present (Type 1: collocated) | Collocated |
|  | nonCollocatedTypeNR-CA/MRDC-r18 = absent (Type 2: non-collocated) | Non-Collocated |
| R18 gNB (Configuration =4 MIMO layers) | N/A | Collocated (Rel-18 Type 1) |
| R19 gNB (Configuration =4MIMO layers) | nonCollocatedTypeNR-CA/MRDC-r19 = 1 (Type 1: collocated) | Collocated (Rel-19 Type 1) |
|  | nonCollocatedTypeNR-CA/MRDC-r19 = 0 (Type 4: non-collocated) | Non-collocated R19 (Type 4) |

## RAN4 requirement

In the agreed big CR [1, 2], it is described as below, where 6dB power spectrum density is only mentioned for the case when the UE indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is provided with value type1.

Maximum 6 dB power spectral density imbalance between downlink carriers is assumed when the UE indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is provided with value type1. Clause 7.10A.2 power imbalance requirement is applied, if the UE indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v19xy is provided with value type4 and UE is configured with maxMIMO-Layers equal to 4. For these UEs, the power spectral density imbalance condition is also applied for their carriers when applicable intra-band non-contiguous NR CA configuration is a subset of a higher order NR CA configuration.

While according to the UE behavior summary in Table 2.1-1, the UE behavior is also collocated when it indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is absent.

Proposal 1: It is proposed to update the requirement to cover the case that “6dB power imbalance apply for type 4 UE when it indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is absent.”

# Conclusion

This contribution discussed the missing UE behavior description for type 4 UE.

Proposal 1: It is proposed to update the requirement to cover the case that “6dB power imbalance apply for type 4 UE when it indicates intraBandNR-CA-non-collocated-r19 and nonCollocatedTypeNR-CA-v1900 is absent.”

# Reference

[1] RP-2509642, Big CR on 38.101-1 for Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2, KDDI, RAN4#116

[2] R4-2509643, Big CR on 38.101-3 for Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2, KDDI, RAN4#116