# 3GPP TSG-RAN WG4 Meeting #116bis	                                                R4-2513622

# Prague, CZ, 13 – 17 October, 2025

Agenda Item:	6.17.2.1

Source: 	Apple

Title:	Discussion on RRM performance requirements for Event triggered L1 measurement reporting

Document for:	Discussion

# Introduction

According to TU assignment for this WID, performance part starts from RAN4#115. In this contribution, we provide initial discussion on performance part of event triggered L1 measurement reporting. After discussion our proposals are provided.

# Discussion

#### Issue 4-1-1: test case list for event-triggered L1 reporting

Candidate options:

- Option 1: Nokia

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case for event-triggered periodic reporting.

- Option 2: CATT

  - SSB based test cases:

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

    - TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

  - CSI-RS based test case:

    - Only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

- Option 3: Apple, [vivo?]

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

- Option 4: CTC

  - LTM Intra-frequency SSB/CSI-RS based event triggered L1-RSRP measurement reporting tests without gap in FR1/FR2.

  - LTM Inter-frequency SSB based event triggered L1-RSRP measurement reporting tests with/without gap in FR1/FR2.

  - LTM Inter-frequency CSI-RS based event triggered L1-RSRP measurement reporting tests with gap in FR1/FR2.

- Option 5: OPPO

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

- Option 6: MTK

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Option 7: CMCC

  - Both FR1 and FR2-1 are considered.

  - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

  - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

- Option 8: ZTE

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

- Option 9: HW

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- Option 10: E///

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

Recommended WF:

- Further discuss based on the following directions

  - For SSB based L1 RSRP measurement reporting:

    - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple)

    - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW)

  - For CSI-RS based L1 RSRP measurement reporting:

    - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

    - For UE supporting event triggered reporting, UE only needs to pass 2)

    - For UE not supporting event triggered reporting, UE only needs to pass 1)

As we discussed in core part, this objective doesn’t introduce any new measurement behavior. Only a new reporting mechanism is introduced. For measurement, RAN4 already has core requirements and corresponding test cases introduced in R18, including intra-frequency/inter-frequency, measurement without gap and within gap and so on. Therefore, in this objective RAN4 only needs to introduce some essential test cases.

For SSB based L1 RSRP measurement reporting, there are two approaches on the table:

- Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple)

- Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW)

Generally, we are fine with either approach. Approach 1 is slightly preferred with consideration that total test cases UE need to pass is a bit less than that of approach 2. Some company mentioned that approach 1 may require more standardization work. However, we believe somehow it is manageable. Because the additional reporting procedure could be more or less same for all the test cases. Thus RAN4 only needs to discuss one test case and then duplicate the agreement to others.

Proposal 1: For SSB based L1 RSRP measurement reporting, the following approach 1 is slightly preferred:

    - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases

    - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

Regarding CSI-RS, we believe the recommended WF is reasonable.

Proposal 2: For CSI-RS based L1 RSRP measurement reporting:

    - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

    - For UE supporting event triggered reporting, UE only needs to pass 2)

    - For UE not supporting event triggered reporting, UE only needs to pass 1)

#### Issue 4-1-2: test configuration for event-triggered L1 reporting

Candidate options:

- Proposal 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia)

  - Use LTM3 as the reporting event in the test cases.

- Proposal 2: it is proposed to define test cases for LTM event 2/3/4/5. (CMCC)

- Proposal 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW)

- Proposal 4: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

Agreement:

- On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

Although we had agreement on high level procedure, there are still some details need to be discussed. Regarding the event type, we don’t think they would make any difference from RAN4 point of view. The fundamental issue that needs to be verified is the latency caused by UE measurement, rather than event type. Therefore, we support to only focus on the typical case, i.e. LTM event 3. However, since there are multiple test cases, we are also fine with using other event types as long as it won’t result in extra testing time.

Proposal 3: testing LTM event 3 is enough. However, we are also fine with using other event types in different test cases as long as it won’t result in extra testing time.

# Conclusion

In this contribution, we provide discussion on performance requirements for event triggered L1 measurement reporting. After discussion, the following conclusions are provided:

Proposal 1: For SSB based L1 RSRP measurement reporting, the following approach 1 is slightly preferred:

    - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases

    - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

Proposal 2: For CSI-RS based L1 RSRP measurement reporting:

    - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

    - For UE supporting event triggered reporting, UE only needs to pass 2)

    - For UE not supporting event triggered reporting, UE only needs to pass 1)

Proposal 3: testing LTM event 3 is enough. However, we are also fine with using other event types in different test cases as long as it won’t result in extra testing time.

# Reference

- TS38.133