3GPP TSG-RAN WG4 Meeting #116BIS	R4-2513634

# Prague, Czech Republic, 13th – 17th October, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for Ku-band NTN

Agenda item:	4.2.7.4

Document for:	Discussion

Introduction

In this contribution, we provide our views on open issues for Ku-band NTN based on approved WF [1].

Discussion

Satellite switch requirements

In the last RAN4 meeting, RAN4 agreed on detailed requirements for both hard and soft Ku-band satellite switching with re-sync for FR1 and FR2 numerology.

Hard satellite switching with re-sync Agreement: - The components in hard satellite switching delay requirements refer to legacy hard satellite switching delay and inter satellite handover delay. Soft satellite switching with re-sync Agreement: - Agree to introduce RRM requirements for soft satellite switching with re-sync for the VSAT UE with electronic beam steering: Tsoft_switch = max(t-service-t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing - The requirements in this clause do not apply if the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-ServiceStart-r18 [and ssb-TimeOffset-r18] is acquired by the UE for the first time to the first slot corresponding to t-ServiceStart-r18 is less than [1] s. - Further refinement on the wording is not precluced. - Further discussion on the value of [1] is not precluded. - Extend the period of the scheduling restriction by two sided TRX_switch, TRX_switch = 1 symbol.

For soft satellite switching with re-sync, an exception requirement was introduced for soft satellite switching with re-sync to handle the case where the UE receives the updated t-serviceStart-r18 value too late, i.e., shortly before the t-serviceStart. The current clause states that requirements do not apply if the updated value is acquired less than 1 second before t-ServiceStart.

While the intention is clear—to avoid forcing the UE to execute full switching procedures when there is insufficient preparation time— the one-second threshold appears unnecessarily generous when compared with the actual time a UE requires to process after receiving an SI.

In practice, the UE must carry out a series of low-level preparations—such as RF retuning and stabilization, baseband re-synchronization, and buffer or decoder setup. Each of these steps typically completes within a few tens to a few hundred milliseconds, and together they are unlikely to exceed a few hundred milliseconds of total processing time.

This shows that the UE is in fact able to complete the necessary preparations well within a one-second window. Keeping the current fixed one-second exception effectively disregards situations where the UE could have been ready in time, which reduces the practical usefulness of soft satellite switching. To better reflect actual UE capabilities, we propose redefining the exception margin to be less than 1 second (e.g., 0.5 seconds) as a reasonable approximation based on the expected processing time.

- Proposal 1: Refine the exception time limit less than 1 second (e.g., 0.5 seconds).

Conclusion

In this contribution, we provide our views on open issues for Ku-band NTN, and we propose

- Proposal 1: Refine the exception time limit less than 1 second (e.g., 0.5 seconds).

Reference

[1] R4-2512121, “WF on RRM requirements for NR_NTN_Ku_bands,” Apple