# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513651

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.13.3.1

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for UE-to-UE CLI handling

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on RRM performance requirements for UE-to-UE CLI handling, and a WF was approved [2]. This contribution provides discussion on this topic.

- Discussion

## 2.1 Measurement accuracy

Sub-topic 1-2: L1-SRS-RSRP Issue 1-2-1: Side conditions - Agreements (from online session) - For SRS configuration, reuse R16 assumptions on SRS configuration for L1 SRS-RSRP measurements - Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition. - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB

According to RAN1 reply LS [6], L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both intra-cell scenario and inter-cell scenario. We notice that RAN1 reply LS also mentioned that from RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement, considering no specification impact for SRS configuration information exchange between gNBs. However, similar situation exsits for Rel-16 L3 CLI measurement that even though how SRS configuration information are exchanged between gNBs are not specified in Rel-16, but RAN4 requirements for Rel-16 CLI measurement are specified to cover inter-cell scenario with the assumption that SRS configuration information exchange between gNBs is left to network implementation. Similar approach can be used for RAN4 requirements definition for Rel-19 L1 CLI measurement. What’s more, from deployment point of view, only consider intra-cell is too limited. Based on above consideration, it is proposed to define RAN4 requirements for L1 CLI for both intra-cell scenario and inter-cell scenario.

- Case 1, the residual timing error is 4.67us for FR1 and 3.67us for FR2, same as the Rel-16.

- Case 2, remove the cell phase error, in detail, the residual timing error is 1.67us for FR1 and 0.67us for FR2.

One argument of the proponents which support only consider intra-cell scenario is that it is expected that the measurement accuracy will be improved without considering the residual timing error. In our view, it depends on the SRS SCS.

Table 1 is the simulation results summary based on companies’ feedback.

Table 1 performance comparison between case 1 and case 2 based on companies’ simulation results,

(SNR=1dB, SRS symbol number = 1)

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | CATT (R4-2417696) |  |  |  | HW (R4-2419148) |  |  |  | Samsung (R4-2419620) |  |  |  | Ericsson (R4-2419442) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  | case 1 |  | case 2 |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.98 | 0.41 | -0.61 | 0.71 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -0.77 | 0.2 | -0.41 | 0.54 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -0.71 | 0.11 | -0.35 | 0.48 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -0.83 | 0.13 | -0.5 | 0.48 | -0.98 | 0.07 | -0.54 | 0.42 | 1.2 | 1.2 | 0.7 |  | -0.14 | 0.74 | 0.03 | 0.76 |
|  |  |  | 2 | -0.7 | -0.01 | -0.36 | 0.33 |  |  |  |  | 1.1 | 1.1 | 0.6 |  |  |  |  |  |
|  |  |  | 3 | -0.63 | -0.06 | -0.29 | 0.26 | -0.84 | -0.23 | -0.39 | 0.15 | 1 | 1 | 0.5 |  |  |  |  |  |
|  | 30 | 24 | 1 | -1.58 | -0.07 | -0.81 | 0.57 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -1.38 | -0.26 | -0.61 | 0.34 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -1.27 | -0.37 | -0.55 | 0.3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -1.46 | -0.35 | -0.67 | 0.3 | -1.66 | -0.5 | -0.78 | 0.19 | 2.1 | 2.1 | 1 |  | -0.49 | 0.67 | -0.06 | 0.74 |
|  |  |  | 2 | -1.29 | -0.52 | -0.54 | 0.16 |  |  |  |  | 1.9 | 1.9 | 0.8 |  |  |  |  |  |
|  |  |  | 3 | -1.23 | -0.59 | -0.48 | 0.1 | -1.54 | -0.84 | -0.64 | -0.08 | 1.7 | 1.7 | 0.8 |  |  |  |  |  |
|  | 60 | 24 | 1 | -3.12 | -1.27 | -1.28 | 0.21 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -2.82 | -1.53 | -1.03 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -2.66 | -1.62 | -0.93 | -0.1 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -2.9 | -1.58 | -1.09 | -0.09 | -3.28 | -1.76 | -1.25 | -0.16 | 3.9 | 3.9 | 1.5 |  | -1.32 | 0.58 | -0.28 | 0.71 |
|  |  |  | 2 | -2.68 | -1.77 | -0.96 | -0.23 |  |  |  |  | 3.6 | 3.6 | 1.4 |  |  |  |  |  |
|  |  |  | 3 | -2.61 | -1.85 | -0.89 | -0.3 | -3.05 | -2.18 | -1.13 | -0.5 | 3.4 | 3.4 | 1.3 |  |  |  |  |  |
| FR2 | 60 | 24 | 1 | -2.36 | -0.66 | -0.71 | 0.63 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -2.07 | -0.93 | -0.5 | 0.43 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -1.98 | -1.04 | -0.44 | 0.36 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -2.21 | -1.04 | -0.6 | 0.38 | -2.59 | -1.22 | -0.7 | 0.3 | 3.1 | 3.1 | 0.9 |  | -0.93 | 0.62 | -0.02 | 0.74 |
|  |  |  | 2 | -2.04 | -1.2 | -0.47 | 0.23 |  |  |  |  | 2.7 | 2.7 | 0.7 |  |  |  |  |  |
|  |  |  | 3 | -1.97 | -1.27 | -0.39 | 0.18 | -2.33 | -1.56 | -0.56 | 0.02 | 2.6 | 2.6 | 0.6 |  |  |  |  |  |
|  | 120 | 24 | 1 | -5.58 | -3.12 | -1.07 | 0.35 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 2 | -5.22 | -3.48 | -0.86 | 0.16 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 3 | -5.06 | -3.62 | -0.76 | 0.07 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | 1 | -5.37 | -3.56 | -0.93 | 0.09 | -6.01 | -3.6 | -1.05 | -0.05 | 6.8 | 6.8 | 1.3 |  | -2.71 | 0.52 | -0.2 | 0.73 |
|  |  |  | 2 | -5.07 | -3.82 | -0.77 | -0.07 |  |  |  |  | 6.2 | 6.2 | 1.1 |  |  |  |  |  |
|  |  |  | 3 | -4.96 | -3.92 | -0.73 | -0.15 | -5.56 | -4.18 | -0.92 | -0.3 | 6 | 6 | 1 |  |  |  |  |  |

It can be seen that whether there is performance gap between case 1 and case 2 is related with SCS. For 15KHz SCS and 30KHz SCS, the performance gap between case 1 and case 2 is small, less than 1dB. While with larger SCS, e.g. 120KHz, the performance gap between case 1 and case 2 is large.

Observation 1: L1-SRS-SRSP measurement performance gap between case 1 and case 2 is related with SCS

- For 15KHz SCS and 30KHz SCS, the performance gap between case 1 and case 2 is small, less than 1dB.

- For larger SCS, e.g. 120KHz, the performance gap between case 1 and case 2 is large.

Legacy L3-SRS-RSRP accuracy requirements are SCS specific, as dulicated as following.

Table 10.1.22.1.1-1:  SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  | Extreme condition |  |  | SRS Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB |  |  |  |  |  | dB |  | dBm / SCSSRS |  |  | dBm/BW Channel | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 | 30 | 60 | 15 | 30 | 60 |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3 | 3.5 | 5 | 7.5 | 8 | 9.5 | 1 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  |  |  |  |  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 6.5 | 7 | 8.5 | 9.5 | 10 | 11.5 | 1 | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E | N/A | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |

Taking above into account, it is proposed to define accuracy requirements with different side conditions for different SCS. In detail, for 15KHz SCS and 30KHz SCS, the accuracy requirements are defined to cover both intra- and inter-cell scenarios. For SCS larger than or equal to 60KHz, the accuracy requirements are defined only cover intra-cell scenario.

Proposal 1: it is proposed to define accuracy requirements with different side conditions for different SCS

- For 15KHz SCS and 30KHz SCS, the L1-SRS-SRSP accuracy requirements are defined to cover both intra-cell scenario and inter-cell scenario

- For SCS larger than or equal to 60KHz, the L1-SRS-SRSP accuracy requirements are defined only cover intra-cell scenario

## 2.2 Test cases on CLI measurement

In core part discussion, it was agreed that RAN4 not to define RRM requirements for gNB to gNB CLI handling. For UE to UE CLI handling, RAN4 define requirements for L1-SRS-RSRP and L1-CLI-RSSI. Measurement period requirements for L1-CLI-RSSI are defined based on 1 sample. For L1-SRS-RSRP, for periodic and semi-persistent SRS resource, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. For aperiodic SRS resources M=1. And RAN4 to define one set of accuracy requirements based 1 sample. It is proposed to define test cases on measurement period to cover both L1-SRS-RSRP and L1-CLI-RSSI.

Proposal 2: it is proposed to define test cases on measurement period for both L1-SRS-RSRP and L1-CLI-RSSI.

In total, there are three methods. As shown in following, different scheduling restrictions are defined for different method. It is necessary to define test cases for three methods.

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

Agreements on scheduling restriction for L1-SRS-RSRP (Method#2) - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for SRS resource and Y symbols before, where Y is re-using same number as R16 requirements. - For the symbols after the SRS resource, not define scheduling restriction on symbols after symbols for SRS resource in TS 38.133, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211. - For DL, - if UE is able to support FDM-ed DL reception and SRS measurement, scheduling restriction does not apply except when SRS resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, otherwise (if UE does not support the capability), scheduling restriction applies. - The restricted symbols include symbols for SRS resource and Y symbols before symbols for SRS resource, where Y is re-using same number as R16 requirements. - FFS whether restricted symbol(s) is needed for the symbol(s) after SRS resource to consider the switching between UL subband and DL subband. Agreements on scheduling restriction for L1-CLI-RSSI - Method #1: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource - Method #3: - For UL, - Scheduling restrictions always apply, for both FR1 and FR2. - The restricted symbols include symbols for RSSI resource. - Do not define scheduling restriction on symbols after symbols for RSSI resource, with the understanding that the UL/DL switching time is covered by RAN1 specification TS 38.211 clause 4.3.2 - For DL, - Scheduling restriction does not apply except when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH in FR2, the restricted symbols include symbols for RSSI resource. - Note: the common understanding in RAN4 if the RSSI in UL suband is too strong compare to the wanted signals in DL suband(s), UE may fail to decode the PDCCH or PDSCH. - FFS whether restricted symbol(s) is needed for the symbol(s) after RSSI resource to consider the switching between UL subband and DL subband.

Proposal 3: it is proposed to define test cases on scheduling restriction for following methods

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Conclusion

This contribution provides discussion on RRM performance requirements for UE-to-UE CLI handling. The proposals are:

Observation 1: L1-SRS-SRSP measurement performance gap between case 1 and case 2 is related with SCS

- For 15KHz SCS and 30KHz SCS, the performance gap between case 1 and case 2 is small, less than 1dB.

- For larger SCS, e.g. 120KHz, the performance gap between case 1 and case 2 is large.

Proposal 1: it is proposed to define accuracy requirements with different side conditions for different SCS

- For 15KHz SCS and 30KHz SCS, the L1-SRS-SRSP accuracy requirements are defined to cover both intra-cell scenario and inter-cell scenario

- For SCS larger than or equal to 60KHz, the L1-SRS-SRSP accuracy requirements are defined only cover intra-cell scenario

Proposal 2: it is proposed to define test cases on measurement period for both L1-SRS-RSRP and L1-CLI-RSSI.

Proposal 3: it is proposed to define test cases on scheduling restriction for following methods

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2504904, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2502699, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, Apple

- R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2416919, LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2502633, Updated summary of simulation results for L1-SRS-RSRP, Huawei, HiSilicon

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R1-2501620, LS on L1 based UE-to-UE CLI measurement and reporting