# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513658

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	7.12.2

Source: 	CMCC

Title: 			 Discussion on general aspects for AI/ML for mobility

Document for:	Discussion

- Introduction

In RAN #108 meeting, the new WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [1]. In RAN #109 meeting, the revised WID on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR was approved [2]. The objectives for RAN4 are duplicated as following.

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any

This contribution provides discussion on general aspects.

- Discussion

For AI/ML for mobility, there are 6 scenarios, scenario 2, 3, 4 are prioritized. Scenario 4 is FR2 intra-frequency temporal domain case A (i.e., prediction of measurements in a prediction window by using historical actual measurements in an observation window). Scenario 2 is FR1 intra-frequency temporal domain case B (similar to case A, but predicted measurement result(s) in previous prediction window are skipped). And scenario 3 is FR1 inter-frequency prediction. The study on temporal domain case A is to verify performance improvement, i.e. how much handover performance can be improved based on predicted measurement results/event in future. The motivation of study on temporal domain case B is about measurement reduction, i.e. whether the degradation of handover performance is acceptable while partial measurement is skipped.

Table 1 Evaluation scenarios for RRM measurement prediction

| scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

During study phase, there is discussion on whether to consider other spatial domain prediction. The conclusion, is that except scenario 2, 3, 4, discussion on other scenarios are not precluded in work item.

TR 38.744 section 6.2.1 The impact on RRM requirements for L3 cell-level RSRP (i.e., Point C as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has been studied for scenarios 2, 3, and 4 as defined in Table 5.2.1.1-1 and the sub-use cases 1, 2, and 3 as defined in section 4.2. Editor Note: The impact of L3 beam level (Point E as defined in Figure 9.2.4-1 in TS 38.300) measurement prediction has not been studied. However, it doesn’t preclude the discussion on L3 beam level measurement prediction in the work item phase. Editor Note: From RAN4 perspective, it is not precluded that other scenarios in Table 5.2.1.1-1 will be discussed in work item.

According to the WID [2], temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases are considered. For RAN4 work, we can follow the WID and focus on these three scenarios. No need to consider other scenarios.

Proposal 1: for scenario, it is proposed that RAN4 focus on temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases.

One issue is about L3 beam-level measurement prediction. As duplicated below, the conclusion in study phase that RAN4 can further discuss this issue pending on the RAN2 progress.

Agreements in RAN4#115 meeting (R4-2508268) Issue 2-1: Relative RSRP accuracy definition Agreement: - For intra-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 1 and cell 2 are on the same frequency - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy - Relative RSRP accuracy for Beam level measurements is FFS during WI phase depending upon RAN2 progress. - For inter-frequency L3-RSRP, - Relative accuracy of predicted L3-RSRP = (reported predicted L3-RSRP of cell 1 – reported RSRP of cell 2) – (ground truth of RSRP of cell 1 – ground truth of RSRP of cell 2), - cell 2 is on a different frequency than cell 1 but in the same FR as cell 1 - the reported RSRP of cell2 can be measured or predicted. - FFS on whether and how to consider time difference between the two reported RSRPs when applying the definition of the relative RSRP accuracy Issue 2-5: Impacts on beam/cell level measurements Agreement: Consider L3 beam-level (Point E) measurement prediction depending upon the progress in RAN2. Note: RAN4 didn’t study impact of L3 beam level measurements. However, it doesn’t preclude the discussion on L3 beam level measurements in the work item phase.

However, according to the revised WID [2], it is determined that L3 beam level prediction is not supported for UE sided model. RAN4 no need to discuss this issue and focus on cell level prediction.

Revised WID (RP-252899) The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported.

Proposal 2: it is proposed that RAN4 work focus on L3 Cell level predication. No need to consider L3 beam level prediction for UE sided model.

- Conclusion

This contribution provides discussion on general aspects. The proposals are:

Proposal 1: for scenario, it is proposed that RAN4 focus on temporal domain prediction case A, temporal domain prediction case B, and frequency domain prediction for co-located cases.

Proposal 2: it is proposed that RAN4 work focus on L3 Cell level predication. No need to consider L3 beam level prediction for UE sided model.

- Reference

- RP-251864, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR