# 3GPP TSG-RAN WG4 Meeting #116bis                              R4-2513661

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.11.6

Source: 	CMCC

Title: 			 Discussion on RRM performance requirements for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM performance requirements for beam management.

- Discussion

For the target for AI/ML based beam management, our view is that AI/ML based performance requirement should not be worse than the legacy. According to the discussion in previous meetings, some companies mentioned that the AI/ML based performance will be worse than the legacy, since the gain come from overhead reduction. From operator point of view, we do not think that this is a good way to go. If the predicted results are worse than the legacy measured results, it will have impact on the already deployed network, which is not preferred.

Proposal 1: For RSRP accuracy for AI/ML based beam management, it is proposed that AI/ML based performance requirement should be no worse than the legacy measurement accuracy requirements.

RAN1 agreements in RAN1#120 meeting Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value. Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. Agreement For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

For beam management, RAN1 agreed the report content of inference results for UE-sided model. The largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

The legacy RSRP measurement report mapping table is defined in RAN4, which are duplicated as following. In legacy, the reporting range of SS-RSRP for L1 reporting is defined from -140 to -44 dBm with 1 dB resolution. And the reporting range of differential SS-RSRP for L1 reporting is defined from 0 dB to -30 dB with 2 dB resolution. It can be seen that RAN1 design on the report content of inference results is alligned with legacy L1 measurement report, which means that legacy L1 report mapping table can be reused for the report of inference results. However, the lagacy report mapping is for measured RSRP, some wording update to cover predicted RSRP is necessary.

Table 10.1.6.1-1: SS-RSRP and CSI-RSRP measurement report mapping (TS 38.133)

| Reported value | Measured quantity value (L3 SS-RSRP) and CSI-RSRP | Measured quantity value (L1 SS-RSRP and CSI-RSRP) | Unit |
| --- | --- | --- | --- |
| RSRP_0 | SS-RSRP<-156 | Not valid | dBm |
| RSRP_1 | -156 SS-RSRP<-155 | Not valid | dBm |
| RSRP_2 | -155 SS-RSRP<-154 | Not valid | dBm |
| RSRP_3 | -154 SS-RSRP<-153 | Not valid | dBm |
| RSRP_4 | -153 SS-RSRP<-152 | Not valid | dBm |
| RSRP_5 | -152 SS-RSRP<-151 | Not valid | dBm |
| RSRP_6 | -151 SS-RSRP<-150 | Not valid | dBm |
| RSRP_7 | -150 SS-RSRP<-149 | Not valid | dBm |
| RSRP_8 | -149 SS-RSRP<-148 | Not valid | dBm |
| RSRP_9 | -148 SS-RSRP<-147 | Not valid | dBm |
| RSRP_10 | -147 SS-RSRP<-146 | Not valid | dBm |
| RSRP_11 | -146 SS-RSRP<-145 | Not valid | dBm |
| RSRP_12 | -145 SS-RSRP<-144 | Not valid | dBm |
| RSRP_13 | -144 SS-RSRP<-143 | Not valid | dBm |
| RSRP_14 | -143 SS-RSRP<-142 | Not valid | dBm |
| RSRP_15 | -142 SS-RSRP<-141 | Not valid | dBm |
| RSRP_16 | -141 SS-RSRP<-140 | RSRP<-140 | dBm |
| RSRP_17 | -140 SS-RSRP<-139 | -140RSRP<-139 | dBm |
| RSRP_18 | -139 SS-RSRP<-138 | -139 RSRP<-138 | dBm |
| … | … |  | … |
| RSRP_111 | -46≤ SS-RSRP<-45 | -46 RSRP<-45 | dBm |
| RSRP_112 | -45 SS-RSRP<-44 | -45 RSRP<-44 | dBm |
| RSRP_113 | -44 SS-RSRP<-43 | -44 RSRP | dBm |
| RSRP_114 | -43 SS-RSRP<-42 | Not valid | dBm |
| RSRP_115 | -42 SS-RSRP<-41 | Not valid | dBm |
| RSRP_116 | -41 SS-RSRP<-40 | Not valid | dBm |
| RSRP_117 | -40 SS-RSRP<-39 | Not valid | dBm |
| RSRP_118 | -39 SS-RSRP<-38 | Not valid | dBm |
| RSRP_119 | -38 SS-RSRP<-37 | Not valid | dBm |
| RSRP_120 | -37 SS-RSRP<-36 | Not valid | dBm |
| RSRP_121 | -36 SS-RSRP<-35 | Not valid | dBm |
| RSRP_122 | -35 SS-RSRP<-34 | Not valid | dBm |
| RSRP_123 | -34 SS-RSRP<-33 | Not valid | dBm |
| RSRP_124 | -33 SS-RSRP<-32 | Not valid | dBm |
| RSRP_125 | -32 SS-RSRP<-31 | Not valid | dBm |
| RSRP_126 | -31 SS-RSRP | Not valid | dBm |
| RSRP_127 (Note) | Infinity | Infinity | dBm |
| NOTE: The value of RSRP_127 is applicable for RSRP threshold configured by the network as defined in TS 38.331 [2], but not for the purpose of measurement reporting. |  |  |  |

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting) report mapping (TS 38.133)

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

Proposal 2: based on RAN1 agreements, the legacy L1 report mapping, i.e. L1 part of Table 10.1.6.1-1 and Table 10.1.6.1-2 in TS38.133 can be reused for the report mapping of predicted L1-RSRP, but some wording update is needed.

- Conclusion

This contribution provides discussion on RRM performance requirements for beam management. The proposals are:

Proposal 1: For RSRP accuracy for AI/ML based beam management, it is proposed that AI/ML based performance requirement should be no worse than the legacy measurement accuracy requirements.

Proposal 2: based on RAN1 agreements, the legacy L1 report mapping, i.e. L1 part of Table 10.1.6.1-1 and Table 10.1.6.1-2 in TS38.133 can be reused for the report mapping of predicted L1-RSRP, but some wording update is needed.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2505105, WF on requirements for AI/ML air interface normative work, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface