# 3GPP TSG-RAN WG4 Meeting #116bis                             R4-2513663

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	6.11.7

Source: 	CMCC

Title: 			 Discussion on demodulation requirements for CSI prediction

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #106 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion core requirements for CSI prediction.

- Discussion

In last meeting, following agreements are reached for AI/ML based CSI predictions.

Issue 1-1: Requirement considerations Agreement: The performance of AI based CSI prediction should not be worse than R16 etypeII. - The exact test procedure and configurations are FFS - The impact of different timeline between AI and R16 etype II should be also carefully analysed Issue 1-3: Doppler handling Agreement: - De-prioritize 100Hz Doppler in R19 - Consider 100Hz Doppler in the future release if needed Issue 1-5: SNR levels for training data and generalization Agreement: No need to capture anything explicit in RAN4 about training data in the spec, - For simulation calibration, SNR range alignment for training can be discussed separately UE performance at more than one MCS should be tested - Model generalization can be discussed separately - Each individual test is expected to involve in single MCS

In previous meeting, it was agreed to use relative throughput as requirement metric for CSI prediction. Reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook. Rel-18 eType II Doppler codebook is used for AI/ML based CSI predictions.

For testing conditions, it was agreed to use static conditions as baseline, FFS on whether to have non-static condition. Static condition is similar as we have for legacy test. Non-static condition is more suitable to verify the generalization and is benificial in reducing the number of tests. It is good if we could have tests for both static and non-static condition.

However, considering the limited timeline and there are so many issues to be solved, it seems impossible to have parallel discussion on both cases. Especially, non-static conditions are new for RAN4 which will take a long time to discuss the detail, for example, how the channel model and SNR settings are changed over the test. It is preferred to firstly focus on static condition, and non-static condition can be further discussed if time allowed.

With static conditions, different scenarios, e.g. antenna configuration, UE speed, nee to be considered. In legacy PMI test, the test cases are defined without considering different scenarios for a certain precoder type, since it is assumed same PMI reporting algorithm is used. However, AI/ML performance is impacted by training dataset, which is related with scenarios. According to RAN1 evaluation, CSI prediction performance is impacted by the observation window length, prediction window length, and UE speed. These factors which have impact on performance need to be considered in define requirements. To guarantee the performance, it is necessary to define test cases with different test configuration.

Proposal 1: for CSI prediction test with static condition, it is proposed to define test cases with different configuration/parameters, e.g. antenna configuration, observation window length, prediction window length, and UE speed .

For channel model for CSI prediction tests, it was agreed that TDL channel models will be used, FFS on whether CDL channel model is needed for generalization test. It is beneficial to use CDL channel model. However, legacy PMI tests are not based on CDL. And the feasibility for demodulation performance requirements with CDL channel model is under discussion in Rel-19 SI. Considering current situation, it is not feasible to consider CDL channel model in Rel-19. Using CDL channel model for CSI prediction test can be further dicsussed in later release based on the outcome of Rel-19 SI on SCM.

Proposal 2: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further discussed in later release based on the progress on SCM.

- Conclusion

This contribution provides discussion on CSI prediction. The proposals are:

Proposal 1: for CSI prediction test with static condition, it is proposed to define test cases with different configuration/parameters, e.g. antenna configuration, observation window length, prediction window length, and UE speed .

Proposal 2: For channel model for CSI prediction tests, it is proposed to focus on TDL channel models in Rel-19. Using CDL channel model for CSI prediction test can be further discussed in later release based on the progress on SCM.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2508080, WF on NR_AIML_air_part1, Qualcomm Incorporated

- R4-2502856, WF on the requirements for AI/ML air interfance, Qualcomm Incorporated

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

- 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- R4-2502970, Baseline Simulation assumptions for AIML based CSI prediction

- R4-2505229, Updated simulation assumptions for AIML CSI prediction