3GPP TSG-RAN WG4 Meeting # 116-bis		                                                                R4-2513672

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	7.11.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on co-existence evaluation for Rel-20 Ambient IoT outdoor SID

Document for:	Discussion

Introduction

In RAN#108 meeting, the SID [1] on Study on solutions for Ambient IoT in NR was approved as one of R20 RAN1-led items. In this contribution, we want to provide some initial views on the co-existence study for Ambient IoT and NR/LTE.

This SI is to study a second phase of RAN aspects for outdoor scenarios of Ambient IoT, a 3GPP IoT technology which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The study shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power, nor the features of 6G relevant to IoT. Note: This SI shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc., as well as the features of 6G relevant to IoT. The study shall not aim to replace existing 3GPP LPWA technologies. The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI and WI in RAN WGs are taken into this SI. A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

Discussion

Coexistence scenarios and cases

According to the SID, the deployment scenario 4 with topology 1with no external carrier wave to support A-IoT outdoor scenarios. FR1 licensed spectrum in FDD in-band to NR and in standalone bands, n8 as example band with R2D in DL spectrum and D2R in UL spectrum. A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs, we can assume that A-IoT BS don’t share same hardware with outdoor NR macro BS. BS tx power is different between standalone and in-band to NR mode in RAN1. In our understanding, in-band to NR and standalone deployment and co-existence with NR all need to be evaluated. In addition, according to the RAN4-led part, device 2b and device C are all should be studied. Based on the above conditions and TR 38.769, we can adopt the Table 1 as the proposed scenarios for in-band coexistence evaluation.

Proposal 1: In-band to NR and Standalone deployment and co-existence with NR all need to be evaluated.

Proposal 2: Adopt the Table 1 as the proposed co-existence scenarios and cases.

Table 1: co-existence scenarios and cases

| Case No | Deployment scenario and Topology | Spectrum | Aggressor | Victim | Note |
| --- | --- | --- | --- | --- | --- |
| a | D4T1-C NR UE outdoor | R2D: DL D2R: UL | Reader | NR DL | R2D |
| b | D4T1-C NR UE outdoor | R2D: DL D2R: UL | NR DL | device | R2D |
| c | D4T1-C NR UE outdoor | R2D: DL D2R: UL | Device | NR UL | D2R |
| d | D4T1-C NR UE outdoor | R2D: DL D2R: UL | NR UL | reader | D2R |

Co-existence evaluation assumptions

    - Evaluation assumptions analysis

◆ Network layout model

According to the RAN1#122 agreements, which are about the pathloss models and layout assumption as follow.

Agreement For Rel-20 study, the following path-loss models are used in the coverage evaluation: - UMa NLOS defined in TR38.901 - RMa NLOS defined in TR38.901 - Note: companies can also report to use RMa LOS defined in TR38.901 - UMi NLOS defined in TR38.901 - Note: this does not have implication on the coverage target (if any) Agreement For Rel-20 study, the antenna height for BS and Device used for evaluation purpose are as follows: - BS antenna height - UMa: 25m - RMa: 35m - UMi: 10m - Device antenna height - 1.5m Agreement For Rel-20 study, the following layout assumption is used for evaluation purpose: - Hexagonal grid, 19 macro sites, 3 sectors per site - Note: the same layout assumption is used for UMi

With regard to the assumption that A-IoT readers are deployed on the same sites as existing outdoor NR macro BSs, and in order to reduce the RAN4 workload, we can take Uma NLOS for A-IoT BS as the pathloss models as the starting point. In addition, for D4T1, we can take the urban macro network layout model for single operator in TR 38.803 as the starting point. In which, there are two point that need to be illustrated. The one point is active devices for one A-IoT BS reader, based on RAN1’s conclusion, D2R is supported for FDM(A) other than msg1 and msg3. Thus, we need to consider more active devices rather than one active device for one A-IoT BS reader drop. In our understanding, we can consider 2 active devices as baseline, 1 or 4 active devices as optional. The other point is whether NR UE is only outdoor or NR UE is outdoor and indoor. Based on the simulation results for TR 38.769, when the A-IoT BS readers and devices are indoor and NR UEs are only outdoor, negligible performance degradation for NR system is observed. Less than 1 dB SINR degradation due to inter-system interference from NR to ambient IoT system is observed. Similarly, it is obvious that when NR UE is indoor, the co-existence influence can be negligible. Therefore, we can assume the NR UE is only outdoor to evaluate the co-existence performance. If we only consider NR UE is outdoor, there is no need to consider penetration loss for NR UE and A-IoT devices.

Table 2: Single operator layout for urban macro for NR macro BS

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 750m |
| BS antenna height |  | 25 m |
| UE location | Outdoor/indoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS |
|  | UE antenna height | 1.5m |
| UE distribution (horizontal) |  | For NR UE only outdoor, uniformly distributed outdoor. NR UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| Minimum BS - UE distance (2D) |  | 35 m |
| Channel model |  | UMa |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |

Table 3: Single operator layout for urban macro for A-IoT BS

| Parameters |  | Values |
| --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | Reuse TR 38.769, 750m |
| BS antenna height |  | 25 m |
| Device location | Outdoor/indoor | Outdoor |
|  | LOS/NLOS | LOS and NLOS |
|  | Device antenna height | 1.5m |
| Device distribution (horizontal) |  | Reuse TR 38.768, Number of A-IoTs = Total area × activated density (1.5 A-IoT devices/m²) Active devices: 2 baseline/(1 or 4 optional ) under one reader at one drop Minimum distance between reader and device is 1m |
| Minimum BS - UE distance (2D) |  | 35 m |
| Channel model |  | UMa |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |

Proposal 3: We can consider 2 active devices as baseline, 1 or 4 active devices as optional for one A-IoT BS reader drop.

Proposal 4: Just NR UE only outdoor needs to be considered for co-existence study.

◆  Propagation model

Path loss

RAN1 has reached an agreement about path loss model as described above. There are four path need to be considered in the pathloss model. We can assume that NLOS is applicable to the signal path and LOS assessment is applicable to the interference path before the specific discussion. Based on this prerequisite, for NR macro BS to NR UE, we can use UMa NLOS as pathloss model; for A-IoT BS to A-IoT device, we can still use UMa NLOS as pathloss model; for NR UE to A-IoT device, with regard to the antenna height, we can consider UMi LOS as pathloss model; for NR macro BS to A-IoT device, we can still use UMa LOS as pathloss model; for A-IoT BS to NR UE, we can still use UMa as pathloss model.

Proposal 5: For D4T1, these pathloss models need to be consider as follow.

- For NR macro BS to NR UE, use UMa NLOS as pathloss model.

- For A-IoT BS to A-IoT device, use UMa NLOS as pathloss model.

- For NR UE to A-IoT device, use UMi LOS as pathloss model.

- For A-IoT BS to NR UE, use UMa LOS as pathloss model.

- For NR macro BS to A-IoT device, use UMa LOS as pathloss model.

◆ Antenna pattern

A-IoT/NR BS antenna configuration

2 or 4 antenna elements, with (M,N,P,Mg,Ng)=(1,1,2,1,1) or (2,1,2,1,1)

NR UE/A-IoT device antenna configuration

1 element (vertically polarized), Isotropic antenna gain pattern

- Noise Figure

According to the RAN1’s agreement, the A-IoT BS and NR macro BS NF are 5dB. Device 2b NF is 15dB and device C NF is 13dB. NR UE is 9dB.

Agreement For Rel-20 study, BS noise figure assumption in link budget for evaluation purpose is 5dB.

Agreement For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose: - Device 2b: 15 dB - FFS: 40dB as additional assumption for Device 2b with IF-ED - Device C: 13 dB

    - Evaluation assumptions summary

◆  Deployment

With regard to power control, some companies think that it is necessary to consider for UL and DL. However, there is no clear conclusion about this, in addition, for evaluating co-existence interference, to transmit maximum power maybe the worst case. In our understanding, there is no need to consider power control for UL and DL. Based on the above analysis and Section 2.2.1, we can adopt the Table 4 as the deployment parameters for D4T1.

Table 4: Deployment parameters for D4T1

| Parameter | D4T1 |
| --- | --- |
| Carrier frequency | 900 MHz (Band n8) |
| BW for NR | 10MHz with 15KHz SCS |
| BW for A-IOT system | 180kHz, for 15kHz SCS |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |
| Waveform (CW) | Unmodulated single tone |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |
| NR BS deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-NR BS distance: 750m NR BS height: 25m |
| A-IoT reader deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-A-IoT BS distance: 750m A-IoT BS height: 25m |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 2 baseline/(1 or 4 optional ) active A-IoT devices under one reader at one drop Minimum distance between reader and device is 1m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. NR UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| NR BS Inter-site distance | MCL of 70 dB |
| Pathloss model | PL : NLOS and LOS in TR38.901 NR BS – NR UE: UMa NLOS A-IoT BS – Device: UMa NLOS NR UE – A-IoT device: UMi LOS NR UE – A-IoT BS: UMa LOS A-IoT device – NR BS: UMa LOS |

◆ NR BS RF parameters

For NS BS RF parameters, we can reuse TR 38.769 as follow.

Table 5: NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:17 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc |

- A-IoT BS RF parameters

According to the RAN1’s LS[2], for in-band to NR mode, we can take the 38dBm A-IoT BS as a starting point.

Agreement For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 Note2: companies should report any PSD limitation they assume in their RAN1 evaluations Agreement For Rel-20 study, BS antenna gain assumption in link budget for evaluation purpose: - 17dBi - This assumes in the LLS table (as per TR38.769), the values in “[0h1] Number of antenna elements” and “[0h2] the Number of TXRUs” would be same (as Rel-19)

Table 6: A-IoT BS RF parameters

| A-IoT reader parameters | Values for evaluation |
| --- | --- |
| A-IoT reader total Tx power | 38 dBm |
| A-IoT reader receiver Noise Figure (dB) | 5 |
| A-IoT reader antenna gain (dBi) | 17dBi |
| ACLR | Same as legacy NR BS |
| ACS | Same as legacy NR BS |
| Antenna configuration | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:17 |

- A-IoT device RF parameters

According to the RAN1’s agreements, we can take the minimum power for device 2b and maximum power for device c as assumption parameters. It means that, we can take the -20dBm for device 2b and 5dBm for device C. In addition, device 2b and device c with different transmit power and receiver capability, they will cause different co-existence impacts for NR system, in our understanding, device 2b and device c are all needed to be evaluated.

Agreement For Rel-20 study, Device Tx power assumption for evaluation purpose is from the followings: - Device 2b: {-10, -20} dBm - Device C: {-3, 0, 5} dBm

Table 8: A-IoT device RF parameters

| A-IoT device parameters | Device 2b | Device C |
| --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device transmit power | -10 dBm | 5dBm |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | FFS | FFS |
| A-IoT device noise figure (dB) | 15dB | 13dB |
| Guard band | 0PRB | 0PRB |

Proposal 6: Device 2b and device C are all needed to be evaluated.

- NR UE RF parameters

For NR UE, we can reuse TR 38.769.

Table 9: NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Proposal 7: Adopt the Table from 4 to 9 as the evaluation assumptions.

- Performance metrics

For NR UE, if throughput loss should be less than 5%, it can be considered that inter-system interference is negligible. For A-IoT device, we can reuse TR 38.769, if SINR degradation is smaller than and equal to 1dB, it can be considered that inter-system interference is negligible.

Proposal 8: Throughput loss less than 5% for NR as the performance metric. For A-IoT, SINR degradation is smaller than and equal to 1dB as the performance metric.

    - Co-existence simulation methodology

- Co-existence simulation methodology

We can reuse the methodology in TR 38.769 is proposed as a starting point.

1)	Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader UE based on coupling.

NOTE: For A-IoT devices are associated to A-IoT reader, the way as follow:

The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other. The receiving power of device meets the device receiver sensitivity.

3)	Once association is done, round robin scheduling is used.

4)	For inter-system interference (between A-IoT and NR):

-	If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

-	Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

- SINR definition

For D4T1-C:

SINR definition for D2R:

-	SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference

-	SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

SINR definition for R2D:

-	Signal power of device to the noise and interference within 10MHz is baseline assumption

-	Consider 180KHz noise and interference bandwidth after BB LPF as optional

Proposal 9: Adopt the methodology in section 2.2.3 for co-existence.

Proposal 10: SINR definition can be proposed as follow.

- SINR definition for D2R:

-	SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference

-	SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

- SINR definition for R2D:

-	Signal power of device to the noise and interference within 10MHz is baseline assumption

-	Consider 180 KHz noise and interference bandwidth after BB LPF as optional

Conclusions

In this paper, we provide our views on co-existence between A-IoT and NR/UE, we have made the following proposals.

Proposal 1: Proposal 1: In-band to NR and Standalone deployment and co-existence with NR all need to be evaluated.

Proposal 2: Adopt the Table 1 as the proposed co-existence scenarios and cases.

Proposal 3: We can consider 2 active devices as baseline, 1 or 4 active devices as optional for one A-IoT BS reader drop.

Proposal 4: Just NR UE only outdoor needs to be considered for co-existence study.

Proposal 5: For D4T1, these pathloss models need to be consider as follow.

- For NR macro BS to NR UE, use UMa NLOS as pathloss model.

- For A-IoT BS to A-IoT device, use UMa NLOS as pathloss model.

- For NR UE to A-IoT device, use UMi LOS as pathloss model.

- For A-IoT BS to NR UE, use UMa LOS as pathloss model.

- For NR macro BS to A-IoT device, use UMa LOS as pathloss model.

Proposal 6: Device 2b and device C are all needed to be evaluated.

Proposal 7: Adopt the Table from 4 to 9 as the evaluation assumptions.

Proposal 8: Throughput loss less than 5% for NR as the performance metric. For A-IoT, SINR degradation is smaller than and equal to 1dB as the performance metric.

Proposal 9: Adopt the methodology in section 2.2.3 for co-existence.

Proposal 10: SINR definition can be proposed as follow.

- SINR definition for D2R:

-	SINR includes CW interference is used as the baseline reference for co-existence evaluation for CW reader.

baseline SINR=received wanted signal powernoise + intra_system interference

-	SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

- SINR definition for R2D:

-	Signal power of device to the noise and interference within 10MHz is baseline assumption

-	Consider 180 KHz noise and interference bandwidth after BB LPF as optional

Reference

- RP-251840, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices LGE, INC.TSG-RAN 108

- TR 38.769