3GPP TSG RAN4 Meeting #116bis		R4-2513717
Prague, Czech Republic, 13th – 18th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on core maintenance for SBFD operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Core |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clarification of the measurement period or evaluation period when DRX is configured and the measurement resource is either overlaping with an UL allocation, or with a symbol of different type than the one configured for the measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifying that the measurement period will only be extended if all the occasions of the measurement resource are unavailable during the DRX on duration timer. |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement period or the evaluation period might be unecessarily extended. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.2, 8.5.3.2, 8.5.6.2, 9.5.4.2, 9.8.4.1, 9.8.4.2, 9.8.4.3, 9.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

#### 8.1.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_CSI-RS period becomes worse than the threshold Qout_CSI-RS within TEvaluate_out_CSI-RS evaluation period.

UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_CSI-RS ms period becomes better than the threshold Qin_CSI-RS within TEvaluate_in_CSI-RS ms evaluation period.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-1 for FR1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-2 for FR2 with scaling factor N=1.

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-3 for FR1 (deactivated PSCell).

-	TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS are defined in table 8.1.3.2-4 for FR2 (deactivated PSCell) with scaling factor N=1.

The requirements of TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS apply provided that the CSI-RS for RLM is not in a resource set configured with repetition ON. The requirements do not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for RLM and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

For the value of L1Out and L1In,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network,

2>	 L1Out = 0;

2>  L1In = 0;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network,

2>	 If DRX is not configured,

3> L1Out is the number of occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS;

3> L1In is the number of occasions of the CSI-RS resource for RLM that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_In_CSI-RS, when DRX is not configured

2>	 If DRX is configured,

3>	 L1Out is the number of DRX cycles in which all the occasions of the CSI-RS resource for RLM within the DRX-onDurationTimer are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_Out_CSI-RS;

3>	 L1In is the number of DRX cycles in which all the occasions of the CSI-RS resource for RLM within the DRX-onDurationTimer are  overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_In_CSI-RS;

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

For a UE supporting LowBandCA-via-Switching-r19, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrentMeasGap-r17 and musim-GapPreference-r17, and when concurrent measurement gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are are configured,

-	an RLM-RS resource occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clauses 9.1.8 and 9.1.10,

-	P value for an RLM-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any RLM-RS resource occasion:

-	Ntotal is the total number of RLM-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W. For UEs supporting LB CA via switching, Ntotal also includes RLM-RS occasions that overlap with the SDL ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of RLM-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or number of RLM-RS that are not overlapped with SDL ON duration corresponding to the LB CA switching pattern within the window W, and

-	Navailable is the number of RLM-RS resource occasions that are not overlapped with any non-dropped GAP occasion, nor non-dropped MUSIM gap occasion, nor any SMTC occasion within the window W, and

-	an RLM-RS resource occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

-	TL1 is periodicity of the target RLM-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	for UEs that support LB CA via switching, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	RLM requirement in this clause is not applied when Noutside_MG = 0.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s), concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, and these GAPs] are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the RLM-RS resource is not overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the RLM-RS resource is not overlapped with GAP and RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSMin(xRP, TSMTCperiod), when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=Psharing factor1-TCSI-RSxRP, when the RLM-RS resource is partially overlapped with GAP and the RLM-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the RLM-RS resource outside GAP is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a GAP is configured only and the GAP is not NCSG,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a GAP occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG is configured,

-	an RLM-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

If the UE is configured with Pre-MG only, an RLM-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured , an RLM-RS resource or an SMTC occasion is not considered to be overlapped by a GAP occasion if the GAP occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1.

NOTE:	The overlap between CSI-RS for RLM and SMTC means that CSI-RS based RLM is within the SMTC window duration.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of RLM-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

When the configured aperiodic MUSIM gap is overlapping with RLM-RS resource occasion, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if RLM-RS resource occasions are fully overlapped with MUSIM gap(s) or the union of MUSIM gap(s) and GAPs, no requirement applies for CSI-RS based RLM.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of Mout and Min used in table 8.1.3.2-1, table 8.1.3.2-2, table 8.1.3.2-3 and table 8.1.3.2-4 are defined as:

-	Mout = 20 and Min = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs in at least one DL subband.

Table 8.1.3.2-1: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil((Mout+ L1Out)×P)×max(TCSI-RS, Tproc)) | Max(100, Ceil((Min+ L1In)×P) × max(TCSI-RS, Tproc)) |
| DRX ≤ 320 ms | Max(200, Ceil(1.5×(Mout+ L1Out)×P)× Max(TDRX, TCSI-RS, Tproc)) | Max(100, Ceil(1.5×(Min+ L1In)×P)× Max(TDRX, TCSI-RS, Tproc)) |
| DRX > 320 ms | Ceil((Mout+ L1Out)×P) × TDRX | Ceil((Min+ L1In)×P) × TDRX |
| NOTE 1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for RLM is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |  |

Table 8.1.3.2-2: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Max(200, Ceil((Mout+ L1Out)×P×N)× max(TCSI-RS, Tproc)) | Max(100, Ceil((Min+ L1In)×P×N) × max(TCSI-RS, Tproc)) |
| DRX ≤ 320 ms | Max(200, Ceil(1.5×(Mout+ L1Out)×P×N)× Max(TDRX, TCSI-RS, Tproc)) | Max(100, Ceil(1.5×(Min+ L1In)×P×N)× Max(TDRX, TCSI-RS, Tproc)) |
| DRX > 320 ms | Ceil((Mout+ L1Out)×P×N) × TDRX | Ceil((Min+ L1In)×P×N) × TDRX |
| NOTE 1: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10 ms, 20 ms or 40 ms. TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for RLM is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |  |

Table 8.1.3.2-3: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR1 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P) × measCyclePSCell | Ceil(Min×P) × measCyclePSCell |
| DRX ≤ 320 ms | Ceil(1.5 ×Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(1.5 ×Min×P) × Max(TDRX, measCyclePSCell) |
| DRX > 320 ms | Ceil(Mout×P) × Max(TDRX, measCyclePSCell) | Ceil(Min×P) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

Table 8.1.3.2-4: Evaluation period TEvaluate_out_CSI-RS and TEvaluate_in_CSI-RS for FR2 (deactivated PSCell)

| Configuration | TEvaluate_out_CSI-RS (ms) | TEvaluate_in_CSI-RS (ms) |
| --- | --- | --- |
| no DRX | Ceil(Mout×P×N) × measCyclePSCell | Ceil(Min×P×N) × measCyclePSCell |
| DRX ≤ 320 ms | Ceil(1.5 × Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(1.5 × Min×P×N) × Max(TDRX, measCyclePSCell) |
| DRX > 320 ms | Ceil(Mout×P×N) × Max(TDRX, measCyclePSCell) | Ceil(Min×P×N) × Max(TDRX, measCyclePSCell) |
| NOTE: TDRX is the DRX cycle length of SCG. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |  |

# <End of change  1>

# <Start of change 2>

#### 8.5.3.2	Minimum requirement

UE shall be able to evaluate whether the downlink radio link quality on the CSI-RS resource in set q0 estimated over the last TEvaluate_BFD_CSI-RS period becomes worse than the threshold Qout_LR_CSI-RS within TEvaluate_BFD_CSI-RS period.

The value of TEvaluate_BFD_CSI-RS is defined in table 8.5.3.2-1 or table 8.5.3.2-3 (deactivated PSCell) for FR1.

The value of TEvaluate_BFD_CSI-RS is defined in table 8.5.3.2-2 or table 8.5.3.2-4 (deactivated PSCell) for FR2 with N=1. The requirements of TEvaluate_BFD_CSI-RS apply provided that the CSI-RS for BFD is not in a resource set configured with repetition ON. The requirements shall not apply when the CSI-RS resource in the active TCI state of CORESET is the same CSI-RS resource for BFD and the TCI state information of the CSI-RS resource is not given, wherein the TCI state information means QCL Type-D to SSB for L1-RSRP or CSI-RS with repetition ON.

-	For a UE supporting LB-CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or [musim-GapPreference-r17] or both concurrentMeasGap-r17 and musim-GapPreference-r17, and when concurrent gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are configured,

-	an CSI-RS resource occasion for beam failure detection is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8 and 9.1.10,

-	P value for a BFD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable> 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any BFD-RS resource occasion:

-	Ntotal is the total number of BFD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W,

For UEs supporting LB CA via switching and configured with low band CA via switching:

-	for the BFD monitoring on PCell, Ntotal also includes SSB resource occasions that overlap with the SDL ON duration within the window,

-	for the BFD monitoring on SCell, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of BFD-RS resource occasions within the window W, that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or

-	that are not overlapped with SCell ON duration corresponding to the LB CA switching pattern, for the BFD on PCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching,

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the BFD on SCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, , and

-	Navailable is the number of BFD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion nor any SMTC occasion within the window W, and

-	an CSI-RS resource occasion for beam failure detection is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

TL1 is periodicity of the target BFD-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	for UE not supporting LB CA via switching, switching pattern periodicity is not applicable.

-	BFD requirement in this clause is not applied when Noutside_MG = 0.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,For FR1,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS.

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when the BFD-RS resource is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = Psharing factor, when the BFD-RS resource is not overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSMin(xRP, TSMTCperiod), when the BFD-RS resource is partially overlapped with GAP (TCSI-RS < xRP) and the BFD-RS resource is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when the BFD-RS resource is partially overlapped with GAP and the BFD-RS resource is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the BFD-RS resource outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a GAP is configured only and the GAP is not NCSG,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps a GAP occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG GAP only is configured,

-	a BFD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG only, a BFD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured, a BFD-RS resource or an SMTC occasion is not considered to be overlapped by a GAP occasion if the GAP occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

NOTE:	The overlap between CSI-RS for BFD and SMTC means that CSI-RS for BFD is within the SMTC window duration.

For the value of L1,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network

2>	If DRX is not configured,

3>	L1 is the number of occasions of the configured BFD-RS resource that are overlapping with the dynamic UL transmission on SBFD symbols during TEvaluate_BFD_CSI-RS;

2>	Else if DRX is configured,

3> L1 is the number of DRX cycles in which all the occasions of the BFD-RS resource within the DRX-onDurationTimer are overlapping with the dynamic UL transmission on SBFD symbols during TEvaluate_BFD_CSI-RS.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all occasions of the BFD-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of the BFD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

When the configured aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for beam failure detection, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if CSI-RS resource occasions for beam failure detection are fully overlapped with MUSIM gap(s), or the union of MUSIM gap(s) and GAPs, no requirement applies for CSI-RS based beam failure detection.

For either an FR1 or FR2 serving cell, longer BFD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MBFD used in table 8.5.3.2-1 and table 8.5.3.2-2 are defined as

-	MBFD = 10, if the CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

Note: If UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that CSI-RS resource(s) in set  used for BFD is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs in at least one DL subband.

The values of PBFD used in table 8.5.3.2-1 and table 8.5.3.2-2 are defined as

For each CSI-RS resource in the set  configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PBFD = 1.

For each CSI-RS resource in the set  configured for PSCell in NR-DC

-	PBFD = 2 if UE is configured for beam failure detection on SCell, 1 otherwise.

For each CSI-RS resource in the set  configured for a SCell

-	PBFD = Z in EN-DC or NE-DC or SA.

-	PBFD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing beam failure detection only for SCell.

Table 8.5.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS for FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((MBFD+L1) ´ P ´ PBFD) ´ Max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(1.5 × (MBFD+L1) ´ P ´ PBFD) ´ Max(TDRX, TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MBFD+L1) ´ P ´ PBFD) ´ TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for BFD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 8.5.3.2-2: Evaluation period TEvaluate_BFD_CSI-RS for FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Max(50, Ceil((MBFD+L1) ´ P ´ N ´ PBFD) ´ Max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(1.5 × (MBFD+L1) ´ P ´ N ´ PBFD) ´ Max(TDRX, TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MBFD+L1) ´ P ´ N ´ PBFD) ´ TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for BFD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 8.5.3.2-3: Evaluation period TEvaluate_BFD_CSI-RS for deactivated PSCell in FR1

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Ceil(MBFD ´ P ´ PBFD) ´ measCyclePscell |
| DRX cycle ≤ 320 ms | Ceil(1.5 × MBFD ´ P ´ PBFD) ´ Max(TDRX, measCyclePscell) |
| DRX cycle > 320 ms | Ceil(MBFD ´ P ´ PBFD) ´ Max(TDRX, measCyclePscell) |
| NOTE: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

Table 8.5.3.2-4: Evaluation period TEvaluate_BFD_CSI-RS for deactivated PSCell in FR2

| Configuration | TEvaluate_BFD_CSI-RS (ms) |
| --- | --- |
| no DRX | Ceil(MBFD ´ P ´ N ´ PBFD) ´ measCyclePscell |
| DRX cycle ≤ 320 ms | Ceil(1.5 × MBFD ´ P ´ N ´ PBFD) ´ Max(TDRX, measCyclePscell) |
| DRX cycle > 320 ms | Ceil(MBFD ´ P ´ N ´ PBFD) ´ Max(TDRX, measCyclePscell) |
| NOTE: DRX cycle is the configured DRX cycle of the PSCell. measCyclePSCell is the measurement cycle length of the deactivated PSCell. |  |

# <End of change 2>

# <Start of change 3>

#### 8.5.6.2	Minimum requirement

Upon request the UE shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set q1 estimated over the last TEvaluate_CBD_CSI-RS period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS period provided CSI-RS Ês/Iot is according to annex B.2.4.2 for a corresponding band.

The UE shall monitor the configured CSI-RS resources using the evaluation period in table 8.5.6.2-1 and 8.5.6.2-2 corresponding to the non-DRX mode, if the configured DRX cycle ≤ 320 ms.

The value of TEvaluate_CBD_CSI-RS is defined in table 8.5.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in table 8.5.6.2-2 for FR2 with scaling factor N, where

-	N = 2, 4 or 6 for serving cell in FR2-1 if the UE supports fastBeamSweepingMultiRx-r18 according to the conditions in clause 3.6.19

-	N=8 for other cases in FR2-1, and

-	N=12 for FR2-2.

For the value of L1,

1>	If the UE does not support supportSBFD or SBFD is not configured by the network,

2>	L1=0;

1>	Else if the UE supports supportSBFD and SBFD is configured by the network,

2>	If DRX is not configured,

3> L1 is the number of occasions of the CSI-RS resource for candidate beam detection that are overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_CBD_CSI-RS;

2>	If DRX is configured,

3> L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource for candidate beam detection within the DRX-onDurationTimer are  overlapping with dynamic UL transmission on SBFD symbols during TEvaluate_CBD_CSI-RS.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that all the occasions of the CSI-RS resource are in the same type (i.e., SBFD or non-SBFD) of symbols.

For a UE supporting LB-CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrentMeasGap-r17 and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent GAPs and periodic MUSIM gaps are configured,

-	an CSI-RS resource occasion for candidate beam detection is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clause 9.1.8 and 9.1.10,

-	P value for a CBD-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable> 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or periodic MUSIM gap(s) or NCSGs and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CBD-RS resource occasion:

-	Ntotal is the total number of CBD-RS resource occasions within the window W, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window W,

For UEs supporting LB CA via switching and configured with low band CA via switching:

-	for the CBD monitoring on PCell, Ntotal also includes SSB resource occasions that overlap with the SDL ON duration within the window,

-	for the CBD monitoring on SCell, Ntotal also includes SSB resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CBD-RS resource occasions within the window W,

-	 that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion, or

-	that are not overlapped with SCell ON duration corresponding to the LB CA switching pattern, for the CBD on PCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the CBD on SCell for a UE supporting LowBandSwitching-r19 and configured with low band CA via switching, and

-	Navailable is the number of CBD-RS resource occasions that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion nor any SMTC occasion within the window W, and

-	an CSI-RS resource occasion for candidate beam detection is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion, and

-	TL1 is periodicity of the target CBD-RS, and

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	for UE not supporting LB CA via switching, switching pattern periodicity is not applicable.

-	CBD requirement in this clause is not applied when Noutside_MG = 0.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 nor supporting musim-GapPreference-r17 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s), concurrent measurement gap(s) with NCSG(s), and periodic MUSIM gaps,

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P = 1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P = 1, when candidate beam detection RS is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P =Psharing factor, when candidate beam detection RS is not overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=11-TCSI-RSxRP - TCSI-RSTSMTCperiod,, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5 × TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5 × TSMTCperiod

-	P=11-TCSI-RSMin(xRP, TSMTCperiod), when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP

-	P=31-TCSI-RSxRP,, when candidate beam detection RS is partially overlapped with GAP and candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

where,

-	Psharing factor = 1, if the CBD-RS resource outside GAP is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	If the higher layer in TS 38.331 [2] signalling of smtc2 is present, TSMTCperiod follows smtc2; Otherwise TSMTCperiod follows smtc1. TSMTCperiod is the shortest SMTC period among all CCs in the same FR2 band, provided the SMTC offset of all CCs in FR2 have the same offset.

-	When a GAP is configured only and the GAP is not NCSG,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if it overlaps the GAP occasion, and

-	xRP = MGRP

-	Otherwise, when NCSG GAP only is configured,

-	a CBD-RS resource or an SMTC occasion is considered to be overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

-	If the UE is configured with Pre-MG only, an CBD-RS resource or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	When concurrent gaps or concurrent measurement gap(s) with Pre-MG(s) or concurrent measurement gap(s) with NCSG(s) are configured, a CBD-RS resource or an SMTC occasion is not considered to be overlapped by a GAP occasion if the GAP occasion is dropped according to clause 9.1.8, clause 9.1.12, clause 9.1.13, respectively.

NOTE:	The overlap between CSI-RS for CBD and SMTC means that CSI-RS for CBD is within the SMTC window duration.

Longer evaluation period would be expected if the combination of the CBD-RS resource, SMTC occasion and GAP configurations does not meet previous conditions.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 8.5.6.3.

When the configured aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for candidate beam detection, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and if CSI-RS resource occasions for candidate beam detection are fully overlapped with MUSIM gap(s), or the union of MUSIM gap(s) and GAPs, no requirement applies for CSI-RS based candidate beam detection.

For either an FR1 or FR2 serving cell, longer evaluation period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 serving cell, longer CBD evaluation period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.

The values of MCBD used in table 8.5.6.2-1 and table 8.5.6.2-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set q1 is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs.

If the UE supports supportSBFD and SBFD is configured by the network, the requirements in this clause apply provided that CSI-RS resource configured in the set q1 is transmitted with Density = 3 and over the bandwidth ≥ 24 PRBs in at least one DL subband.

The values of PCBD used in table 8.5.6.2-1 and table 8.5.6.2-2 are defined as

-	For each CSI-RS resource in the set q1 configured for PCell or PSCell in EN-DC or NE-DC or SA; or PCell in NR-DC

-	PCBD = 1.

-	For each CSI-RS resource in the set q1 configured for PSCell in NR-DC

-	PCBD = 2 if UE is configured for candidate beam detection on SCell, 1 otherwise.

-	For each CSI-RS resource in the set q1 configured for a SCell

-	PCBD = Z in EN-DC or NE-DC or SA.

-	PCBD = 2* Z in NR-DC.

-	Where Z is the number of band(s) on which UE is performing candidate beam detection only for SCell

-	PCBD is the number of band(s) on which UE is performing candidate beam detection only for SCell.

Table 8.5.6.2-1: Evaluation period TEvaluate_CBD_CSI-RS for FR1

| Configuration | TEvaluateC_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil((MCBD+L1) ´ P ´ PCBD) ´ max(TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MCBD+L1) ´ P ´ PCBD) ´ TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CBD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |  |

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2

| Configuration | TEvaluate_CBD_CSI-RS (ms) |
| --- | --- |
| non-DRX, DRX cycle ≤ 320 ms | Max(25, Ceil((MCBD+L1) ´ P ´ N ´ PCBD) ´ max(TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | Ceil((MCBD+L1) ´ P ´ N ´ PCBD) ´ TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS resource in the set q1. TDRX is the DRX cycle length. NOTE 2: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CBD is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |  |

# <End of change 3>

# <Start of change 4>

#### 9.5.4.2	CSI-RS based L1-RSRP Reporting

When groupBasedBeamReporting-r17 is not configured, the UE shall be capable of performing L1-RSRP measurements based on the configured CSI-RS resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

When groupBasedBeamReporting-r17 is configured, the UE shall be capable of performing L1-RSRP measurements based on the two configured CSI-RS resource sets for L1-RSRP, and the UE physical layer shall be capable of reporting group-based L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_CSI-RS.

The value of TL1-RSRP_Measurement_Period_CSI-RS is defined in table 9.5.4.2-1 for FR1 and in table 9.5.4.2-2 for FR2, where

-	For periodic and semi-persistent CSI-RS resources, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

-	For aperiodic CSI-RS resources M=1

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided with QCL-TypeD for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided qcl-info is configured for all resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	-	For aperiodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clauses 10.1.19.2 and 10.1.20.2 if number of resources in the resource set is smaller than maxNumberRxBeam. The requirements apply provided qcl-info is configured with QCL-TypeD for all resources in the resource set.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-RSRP for SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource within the DRX-onDurationTimer are  overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

3>	if UE is configured to report L1-RSRP for non-SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS resource that are overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS, when DRX is not configured

4>		When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource within the DRX-onDurationTimer are overlapping with SBFD symbols during TL1-RSRP_Measurement_Period_CSI-RS

For a UE supporting LB CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent measurement gap(s) with NCSG measurement gap(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement gap and musim-GapPreference-r17 and when concurrent gaps or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to clauses 9.1.8 and 9.1.10,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE measurement gaps or NCSGs, MUSIM gap(s)and per-FR measurement gaps or NCSGs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window W, including those overlapped with measurement gap occasions, MUSIM gap occasions or SMTC occasions within the window W.

For UEs supporting LB CA via switching and configured with low NR inter-band carrier aggregation:

-	for the PCell L1-RSRP measurement, Ntotal also includes CSI-RS resource occasions that overlap with the SDL SCell ON duration within the window,

-	for the SCell L1-RSRP measurement, Ntotal also includes CSI-RS resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CSI-RS resource occasions within the window W

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion or

-	that are not overlapped with SDL SCell ON duration corresponding to the LB CA switching pattern, for the PCell L1-RSRP measurement for a UE supporting LB CA via switching with low NR inter-band carrier aggregation configured or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the SCell L1-RSRP measurement for a UE supporting LB CA via switching with low NR inter-band carrier aggregation configured, and

-	L1-RSRP requirement in this clause is not applied when Noutside_MG = 0.

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any non-dropped GAP occasions, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.-	for UEs supporting LB CA via switching, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	a CSI-RS or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

-	TL1 is periodicity of the target CSI-RS.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent measurement gap(s) with NCSG measurement gap(s), and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured.

For FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAPs configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For FR2,

-	P=1, when CSI-RS is not overlapped with a GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=Psharing factor, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P=1, when aperiodic CSI-RS resource is not overlapped with GAP

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,xRP), when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP (TSMTCperiod < xRP)

Where:

-	Psharing factor = 1, if the CSI-RS configured for L1-RSRP measurement outside gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

TSMTCperiod = the configured SMTC period.

TCSI-RS = the periodicity of CSI-RS configured for L1-RSRP measurement

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if it overlapps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, a CSI-RS or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	a CSI-RS or an SMTC occasion is considered to be as overlapped with the GAP if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for L1-RSRP, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and CSI-RS resource occasions for L1-RSRP are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the CSI-RS based L1-RSRP measurement.

Table 9.5.4.2-1: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR1

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(K *(M+L1)*P)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: K = 1 when TCSI-RS ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. NOTE 4: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 or measurementEnhancementCA-r17. NOTE 5: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.5.4.2-2A: Void

# <End of change 4>

# <Start of change 5>

#### 9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and no dedicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only is defined in table 9.8.4.1-1 for FR1 and in table 9.8.4.1-2 for FR2, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clauses 10.1.28.1 and 10.1.28.3 if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS resource as CMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource as CMR within the DRX-onDurationTimer are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS resource as CMR that are overlapping with SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

4>		When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource as CMR within the DRX-onDurationTimer are overlapping with SBFD symbols during TL1-SINR_Measurement_Period_CSI-RS_CMR_Only

For a UE supporting LB CA via switching, or for a UE supporting concurrentMeasGapsPreMG-r18 and when concurrent measurement gap(s) with Pre-MG(s) are configured, or a UE supporting concurrentMeasGapsNCSG-r18 and when concurrent GAP(s) with NCSG(s) are configured, or a UE supporting concurrentMeasGap-r17 or musim-GapPreference-r17 or both concurrent measurement gap and musim-GapPreference-r17 and when concurrent GAPs or periodic MUSIM gaps or both concurrent gaps and periodic MUSIM gaps are configured,

-	a CSI-RS or an SMTC occasion is not considered to be overlapped by a gap occasion if the gap occasion is dropped according to 9.1.8 and 9.1.10,

-	P value for a CSI-RS resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

-	For a window W of duration max(TL1,  xRP_max, switching pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs or periodic MUSIM gap(s) and per-FR GAPs, and, in case of Pre-MG, all activated per-UE measurement gaps and per-FR measurement gaps, within the same FR as serving cell, and starting at the beginning of any CSI-RS resource occasion:

-	Ntotal is the total number of CSI-RS resource occasions within the window, including those overlapped with GAP occasions, MUSIM gap occasions or SMTC occasions within the window,

For UEs supporting LB CA via switching and configured with low NR inter-band carrier aggregation:

-	for the PCell L1-SINR measurement, Ntotal also includes CSI-RS resource occasions that overlap with the SDL SCell ON duration within the window,

-	for the SCell L1-SINR measurement, Ntotal also includes CSI-RS resource occasions that overlap with the PCell ON duration within the window, as defined by the configured switching pattern, and

-	Noutside_MG is the number of CSI-RS resource occasions within the window W

-	that are not overlapped with any non-dropped GAP occasion nor non-dropped MUSIM gap occasion or

-	that are not overlapped with SDL SCell ON duration corresponding to the LB CA switching pattern, for the PCell L1-SINR measurement for a UE supporting LB CA via switching with low NR inter-band carrier aggregation configured or

-	that are not overlapped with PCell ON duration corresponding to the LB CA switching pattern, for the SCell L1-SINR measurement for a UE supporting LB CA via switching with low NR inter-band carrier aggregation configured, and

-	L1-SINR requirement in this clause is not applied when Noutside_MG = 0.

-	Navailable is the number of CSI-RS resource occasions that are not overlapped with any non-dropped GAP occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	for UEs supporting LB CA via switching, switching pattern periodicity is the periodicity of the RRC configured semi-static switching pattern; otherwise, it is not applicable.

-	a CSI-RS or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target CSI-RS.

-	xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG.

Otherwise, for a UE neither supporting concurrentMeasGap-r17 nor concurrentMeasGapsPreMG-r18 nor concurrentMeasGapsNCSG-r18 or when neither of the above configurations applies, i.e. concurrent measurement gaps, concurrent measurement gap(s) with Pre-MG(s) and concurrent GAP(s) with NCSG(s) and UE does not support musim-GapPreference-r17 or when no MUSIM gaps are configured,

For the value of P in FR1,

-	P=11-TCSI-RSxRP, when in the monitored cell there are GAP configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no GAPs overlapping with any occasion of the CSI-RS.

For the value of P in FR2,

-	P=1, when CSI-RS is not overlapped with GAP and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < xRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P is Psharing factor,, when CSI-RS is not overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P= 11-TCSI-RSxRP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with [measurement gap] and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP and

-	TSMTCperiod ≠ xRP or

-	TSMTCperiod = xRP and TCSI-RS < 0.5*TSMTCperiod

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with GAP  and TSMTCperiod = xRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,xRP), when CSI-RS is partially overlapped with GAP (TCSI-RS < xRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with GAP.

-	P=Psharing factor1-TCSI-RSxRP, when CSI-RS is partially overlapped with GAP and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with GAP  (TSMTCperiod < xRP)

Where:

Psharing factor = 1, if the CSI-RS configured for L1-SINR measurement outside gap is

not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

not overlapped by the RSSI symbols indicated by ss-RSSI-Measurement and 1 data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured.

-	Psharing factor = 3, otherwise.

-	TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured.

-	TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

-	When a measurement gap is configured and the measurement gap is not NCSG,

-	a CSI-RS is considered to be overlapped with the GAP if it overlaps a measurement gap occasion, and

-	xRP = MGRP

-	If the UE is configured with Pre-MG, a CSI-RS reourse or an SMTC occasion is only considered to be overlapped by the Pre-MG if the Pre-MG is activated.

-	Otherwise, when NCSG measurement gap only is configured,

-	a CSI-RS is considered to be overlapped with the GAP  if

-	it overlaps the VIL1 or VIL2 of NCSG, or

-	it overlaps the ML of NCSG in FR2, and there exists a target carrier to be measured within NCSG that is intra-frequency carrier or inter-frequency carrier in the same band as the serving cell, or inter-frequency carrier in different band as the serving cell and UE does not support IBM between the target carrier and the serving cell,

-	and

-	xRP = VIRP

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

When UE is configured with aperiodic MUSIM gap and the aperiodic MUSIM gap is overlapping with CSI-RS resource occasion for L1-SINR, longer evaluation period would be expected.

When UE is configured with MUSIM gap(s), and CSI-RS resource occasions for L1-SINR are fully overlapped with MUSIM gap(s) or fully overlapped with the union of MUSIM gap(s) and GAPs, no requirement applies for the CSI-RS based L1-SINR measurement.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and GAP configurations does not meet previous conditions.

Table 9.8.4.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.1-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

# <End of change 5>

# <Start of change 6>

#### 9.8.4.2	L1-SINR reporting with SSB based CMR and dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

For UE supporting On-demand SSB operation and when OD- SSB transmission is indicated, the EMP for L1-SINR are

-	the periodicity of OD-SSBs in the configured DL BWP if no first SSB transmission is configured, or

-	the periodicity of the union of OD-SSBs and first SSBs if first SSBs are transmitted and the OD-SSB activation is indicated.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in table 9.8.4.2-1 for FR1 and in table 9.8.4.2-2 for FR2 when highSpeedMeasFlagFR2-r17 is not configured, and defined in table 9.8.4.2-3 for FR2 power class 6 UE when highSpeedMeasFlagFR2-r17 is configured, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = 2, 4 or 6 in FR2-1 for UE supporting fastBeamSweepingMultiRx-r18, according to the conditions described in clause 3.6.19,

-	N = 8 otherwise.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

4>		When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS/CSI-IM resource as IMR within the DRX-onDurationTimer are is overlapping with dynamic UL transmission or with non-SBFD symbols during TL1-SINR_Measurement_Period_SSB_CMR_IMR

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS/CSI-IM resource as IMR within the DRX-onDurationTimer are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_SSB_CMR_IMR

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	the value of PCMR shall be derived in the same way as the value of P used for SSB based L1-RSRP measurement in clause 9.5.4.1, in which the occasions and period of the SSB for CMR shall be used instead.

-	the value of PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2, in which the occasions and period of the NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform SSB measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For L1-SINR measurement with SSB as CMR and CSI-RS or CSI-IM as IMR, the requirement shall apply if the CSI-RS is configured as IMR with repetition field as “repetition = OFF” or CSI-IM is configured as IMR.

For L1-SINR measurement with SSB as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if SSB occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap

Table 9.8.4.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)* max(TSSB, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TSSB, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.2-2: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TSSB, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB, Tproc)) |
| DRX cycle > 320 ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement if UE not supporting On-demand SSB operation or when OD-SSB transmission is not indicated, otherwise, TSSB = EMP of the SSB index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.2-3: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR configured with highSpeedMeasFlagFR2-r17 for FR2-1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N1Note 3)* max(TSSB, Tproc)) |
| DRX cycle ≤ 80 ms | max(TReport, ceil((M+L1)*P*N1Note 3*M2)*max(TDRX,TSSB, Tproc)) |
| 80 ms< DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320 ms | ceil(1.5*(M+L1)*P*N)*TDRX |
| NOTE 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. NOTE 3: N1 = 2 when highSpeedMeasFlagFR2-r17 = set1; N1 = 6 when highSpeedMeasFlagFR2-r17 = set2. NOTE 4: M2 = 1.5 if SMTC periodicity > 40 ms; otherwise M2 = 1 NOTE 5: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

#### 9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to CSI-RS resource configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as CSI-RS resource configured as CMR.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR is defined in table 9.8.4.3-1 for FR1 and in table 9.8.4.3-2 for FR2, where

For the value of M,

-	M=1 shall be applied if

-	aperiodic NZP-CSI-RS as CMR or dedicated IMR, or

-	aperiodic CSI-IM as dedicated IMR, or

-	periodic and semi-persistent NZP-CSI-RS as CMR or dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured, or

-	periodic and semi-persistent CSI-IM as dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured;

-	M=3 otherwise.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else (if UE supports supportSBFD and SBFD is configured by the network)

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else (if higher layer parameter timeRestrictionForChannelMeasurement is not configured)

3>	if UE is configured to report L1-SINR for SBFD symbols

4>	When DRX is not configured, L1 is the maximum between number of occasions of the CSI-RS resource as CMR that are overlapping with dynamic UL transmission or with non-SBFD symbols and number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with dynamic UL transmission or with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource as CMR or all the occasions of the CSI-RS/CSI-IM resource as IMR within the DRX-onDurationTimer are overlapping with dynamic UL transmission or with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

3>	if UE is configured to report L1-SINR for non-SBFD symbols

4>	When DRX is not configured, L1 is the maximum between number of occasions of the CSI-RS resource as CMR that are overlapping with non-SBFD symbols and number of occasions of the CSI-RS/CSI-IM resource as IMR that are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the CSI-RS resource as CMR or all the occasions of the CSI-RS/CSI-IM resource as IMR within the DRX-onDurationTimer are overlapping with non-SBFD symbols, during TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	The value of PCMR and PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2, in which the occasions and period of the CSI-RS for CMR and NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead respectively.

For UE supporting measurement gap occasion cancellation, the UE is not required to perform CSI-RS measurements during the cancelled gap occasions.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet previous conditions.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS as IMR, the requirement shall apply only if CSI-RS resources as CMR and IMR are configured with the same repetition field and the number of CSI-RS resources in the resource sets for CMR and IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-IM as IMR, the requirement shall apply only if the number of CSI-RS resources in the resource set for CMR and the number of CSI-IM resources in the resource set for IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if CSI-RS occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap.

Table 9.8.4.3-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P)*max(TCSI-RS, Tproc)) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. NOTE 4: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR or IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

Table 9.8.4.3-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil((M+L1)*P*N)* max(TCSI-RS, Tproc) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*(M+L1)*P*N)*max(TDRX,TCSI-RS, Tproc)) |
| DRX cycle > 320 ms | ceil((M+L1)*P*N)*TDRX |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. NOTE 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. NOTE 4: If UE indicates needForScaledCSIProcTimeDualDL and the CSI-RS resource for CMR or IMR is across 2 DL subbands, Tproc = 8ms; otherwise Tproc = 0. |  |

# <End of change 6>

# <Start of change 7>

## 9.18	L1 Cross Link Interference measurements

### 9.18.1	Introduction

The UE capable of performing L1 CLI measurements shall be able to measure L1-SRS-RSRP and L1-CLI-RSSI which are defined in TS 38.215 [4] within the active DL BWP. The measurements requirements in this clause apply for TDD mode only.

The L1 CLI measurement requirements specified in this clause are applicable only for RRC_CONNECTED intra-frequency:

-	when L1-SRS-RSRP measurement resource is configured within active DL BWP

-	when L1-CLI-RSSI measurement resource is configured within active DL BWP

The measurements shall be performed for a serving cell, including PCell, PSCell, or SCell, on the resources configured for L1-SRS-RSRP and L1-CLI-RSSI measurements within the active BWP.

For performing CLI measurements in FR2, UE can optionally be configured with CLI measurement resources containing an explicit indication of a TCI state providing a TypeD QCL source [26, TS 38.214]. If the TCI state is not configured, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

The UE does not expect to be configured with L1-CLI-RSSI measurement resources and L1-SRS-RSRP measurement resources on the same symbol.

The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

### 9.18.2	L1-SRS-RSRP measurements

#### 9.18.2.1	Introduction

When configured by the network, the UE shall be able to perform L1-SRS-RSRP measurements on the SRS-RSRP-MeasurementResourceSet configured in a CSI-ResourceConfig.

The requirements apply when the subcarrier spacing for SRS-RSRP measurement resource configuration is the same as the subcarrier spacing of the active DL BWP of serving cell. The UE is not required to measure SRS using different SCS compared to the downlink active BWP SCS of the same carrier.

When the UE measures L1-SRS-RSRP, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

#### 9.18.2.2	Requirements applicability

The requirements in clause 9.18.2 apply, provided:

-	SRS resources configured for L1-SRS-RSRP measurements are measurable.

An SRS resource configured for L1-SRS-RSRP shall be considered measurable when for each relevant SRS the following conditions are met:

-	L1-SRS-RSRP related side conditions given in clauses 10.1.22.1 for FR1 and FR2 for a corresponding band,

-	SRS_RP and SRS Ês/Iot according to Annex B.2.7 for a corresponding band.

#### 9.18.2.3	Measurement Reporting Requirements

The UE shall send L1-SRS-RSRP reports only for report configurations configured for aperiodic report when cli-SRS-RSRP is configured.

The UE shall report the L1-SRS-RSRP value as a 7-bit value in the range [-140, -44] dBm with 1 dB step size according to clause 10, if nrofReportedCLImeasureResource is configured to one. If nrofReportedCLImeasureResource is configured to be larger than one, the UE shall use differential L1-SRS-RSRP based reporting as defined in clause 10. The differential L1-SRS-RSRP is quantized to a 4-bit value with 2 dB step size. The mapping between the reported L1-SRS-RSRP value and the measured quantity is described in clause 10.

##### 9.18.2.3.1	Aperiodic Reporting

Reported L1-SRS-RSRP measurements contained in aperiodic triggered L1-SRS-RSRP reports shall meet the requirements in clauses 10 for FR1 and 10 for FR2, respectively.

The UE shall only send aperiodic L1-SRS-RSRP measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-SRS-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

#### 9.18.2.4	Measurement capability

The UE shall be capable of performing L1-SRS-RSRP measurements on the SRS resources configured for measurement, provided that the total number of L1-SRS-RSRP measurement resources for the UE does not exceed maxNrofSRS-RSRP-MeasurementResources.

#### 9.18.2.5	L1-SRS-RSRP measurement period

The UE shall be capable of performing L1-SRS-RSRP measurement based on the configured SRS resource. For periodic and semi-persistent SRS resource, the UE shall be capable of reporting L1-SRS-RSRP measured over measurement period of TL1_SRS_RSRP_measurement_period for FR1 and FR2.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else if UE supports supportSBFD and SBFD is configured by the network

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else

3>	if UE is configured to report L1-SRS-RSRP for SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the L1-SRS-RSRP resource that are overlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period

4>		When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the L1-SRS-RSRP resource within the DRX-onDurationTimer areoverlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period

3>	if UE is configured to report L1-SRS-RSRP for non-SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the L1-SRS-RSRP resource that are overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of theL1-SRS-RSRP resource within the DRX-onDurationTimer are overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period

Table 9.18.2.5-1 Measurement period TL1_SRS_RSRP_measurement_period

| Configuration | TL1_SRS_measurement_period (ms) |
| --- | --- |
| No DRX | Max(TReport, (M +L1) X TSRS) |
| DRX cycle ≤ 320 ms | Max(TReport, Ceil(1.5 X (M +L1)) X max(TSRS, TDRX)) |
| DRX cycle > 320 ms | (M +L1) X TDRX |
| NOTE 1: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. |  |

If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1_SRS_RSRP_measurement_period.

#### 9.18.2.6	Scheduling availability of UE during L1-CLI measurements

Scheduling availability when the UE is performing L1-CLI measurements which are L1-SRS-RSRP and L1-CLI-RSSI are described in the following clause.

##### 9.18.2.6.1	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR1

The following scheduling restriction applies due to L1-SRS-RSRP measurements.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 15 kHz and 30 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbol(s) on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 15 kHz and 30 kHz subcarrier spacing.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurement, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs L1-SRS-RSRP measurement, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where CLI measurements are performed apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

##### 9.18.2.6.2	Scheduling availability of UE performing L1-SRS-RSRP measurement on FR2

The following scheduling restriction applies due to L1-SRS-RSRP measurements.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	For the UE which supports L1-SRS-RSRP-FDM_DL but the SRS resources are not TypeD QCL-ed with PDCCH/PDSCH, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 1 data symbol before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 60 kHz subcarrier spacing.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 120 kHz subcarrier spacing.

-	For the UE which does not support L1-SRS-RSRP-FDM_DL, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the OFDM symbol(s) on which the UE performs L1-SRS-RSRP measurements, and on 2 data symbols before the OFDM symbol(s) used for L1-SRS-RSRP measurements for 120 kHz subcarrier spacing.

-	For the UE which supports L1-SRS-RSRP-FDM_DL but the SRS resources are not TypeD QCL-ed with PDCCH/PDSCH, the UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on the OFDM symbols on which the UE performs L1-SRS-RSRP measurements, and on 2 data symbols before the OFDM symbol(s) used for SRS-RSRP measurements for 120 kHz subcarrier spacing.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where L1-SRS-RSRP measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

### 9.18.3	L1-CLI-RSSI measurements

#### 9.18.3.1	Introduction

When configured by the network, the UE shall be able to perform L1-CLI-RSSI measurement of configured CLI-RSSI-MeasurementResourceSet, and:

- UE measures L1-CLI-RSSI within DL subband, if one L1-CLI-RSSI measurement resource is configured within a DL subband or one L1-CLI-RSSI measurement resource is configured across two DL subbands,

- UE measures L1-CLI-RSSI within UL subband.

When the UE measures L1-CLI-RSSI, the downlink reference timing in the serving cell shall be applied.

#### 9.18.3.2	Requirements applicability

The requirements in clause 9.18.3 apply, provided:

-	The measurement resources configured for L1-CLI-RSSI measurements are measurable.

A measurement resource configured for L1-CLI-RSSI shall be considered measurable when for each relevant L1-CLI-RSSI resource the following conditions are met:

-	L1-CLI-RSSI related side conditions given in clauses 10 for FR1 and FR2 for a corresponding band.

#### 9.18.3.3	Measurement Reporting Requirements

The UE shall send L1-CLI-RSSI reports only for report configurations for periodical and aperiodic report when cli-RSSI is configured.

The UE shall report the L1-CLI-RSSI value as a 7-bit value in the range [-100, -25] dBm with 1 dB step size according to clause 10 if nrofReportedCLIMeasureResources is configured to one. If nrofReportedCLIMeasureResources is configured to be larger than one, the UE shall use differential L1-CLI-RSSI based reporting as defined in clause 10. The differential L1-CLI-RSSI is quantized to a 4-bit value with 2 dB step size. The mapping between the reported L1-CLI-RSSI value and the measured quantity is described in clause 10.

##### 9.18.3.3.1	Periodic Reporting

Reported L1-CLI-RSSI measurements contained in periodically triggered measurement reports shall meet the requirements in clause 10.

The UE shall transmit the periodic L1-CLI-RSSI reporting on PUCCH over the air interface according to the periodicity defined in clause 5.2.1.4 in TS 38.214 [26].

##### 9.18.3.3.2	Aperiodic Reporting

Reported L1-CLI-RSSI measurements contained in aperiodically triggered L1-CLI-RSSI measurement reports shall meet the requirements in clauses 10.

The UE shall only send aperiodic L1-CLI-RSSI measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-CLI-RSSI reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

#### 9.18.3.4	Measurement capability

The UE shall be capable of performing L1-CLI-RSSI measurement based on the configured resource, provided that the maximum number of L1-CLI-RSSI measurement resources for the UE does not exceed maxNrofCLI-RSSI-MeasurementResources.

#### 9.18.3.5	L1-CLI-RSSI measurement period

The UE capable of supportL1-CLI-RSSI-Measurement shall perform L1-CLI-RSSI measurement based on the configured L1-CLI-RSSI measurement resource. For periodic and semi-persistent L1-CLI-RSSI resource, the UE shall be capable of reporting L1-CLI-RSSI measured over measurement period of TL1-CLI_RSSI_measurement_period for FR1 and FR2.

For the value of L1,

1>	If UE does not support supportSBFD or SBFD is not configured by the network

2>	L1=0

1>	else if UE supports supportSBFD and SBFD is configured by the network

2>	if higher layer parameter timeRestrictionForChannelMeasurement is configured

3>	L1=0

2>	else

3>	if UE is configured to report L1- CLI-RSSI for SBFD symbols

4>		When DRX is not configured, L1 is the number of occasions of the L1- CLI-RSSI resource that are overlapping with non-SBFD symbols during TL1-CLI-RSSI_measurement_period

4>		When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the L1- CLI-RSSI resource within the DRX-onDurationTimer are overlapping with non-SBFD symbols during TL1-CLI-RSSI_measurement_period

3>	if UE is configured to report L1- CLI-RSSI for non-SBFD symbols

4>	When DRX is not configured, L1 is the number of occasions of the L1- CLI-RSSI resource that are overlapping with SBFD symbols, during TL1-CLI-RSSI_measurement_period

4>	When DRX is configured, L1 is the number of DRX cycles in which all the occasions of the L1- CLI-RSSI resource within the DRX-onDurationTimer are overlapping with SBFD symbols, during TL1-CLI-RSSI_measurement_period

Table 9.18.3.5-1 Measurement period TL1-CLI_RSSI_measurement_period

| Configuration | TL1-CLI_RSSI measurement_period (ms) |
| --- | --- |
| No DRX | Max(TReport, (M+L1) X TCLI-RSSI) |
| DRX cycle ≤ 320ms | Max(TReport, Ceil(1.5 X (M+L1)) X max(TCLI-RSSI, TDRX)) |
| DRX cycle > 320ms | (M+L1) X TDRX |
| NOTE 1: TCLI-RSSI is L1-CLI-RSSI measurement periodicity configured RSSI-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise. |  |

If the configured L1-CLI-RSSI measurement resources are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TL1-CLI_RSSI_measurement_period.

#### 9.18.3.6	Scheduling availability of UE during L1-CLI-RSSI measurements

Scheduling availability restrictions when the UE is performing L1-CLI-RSSI are described in the following clause.

##### 9.18.3.6.1	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR1

The following scheduling restriction applies due to the L1-CLI-RSSI measurement:

- The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-CLI-RSSI measurements.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) to be measured and configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

##### 9.18.3.6.2	Scheduling availability of UE performing L1-CLI-RSSI measurement on FR2

The following scheduling restriction applies due to the L1-CLI-RSSI measurement:

- The UE is not expected to transmit PUCCH/PUSCH/SRS on OFDM symbols on which the UE performs L1-CLI-RSSI measurements.

- The UE is not expected to receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on OFDM symbols on which the UE performs L1-CLI-RSSI measurements when RSSI resources are not TypeD QCL-ed with PDCCH/PDSCH/CSI-RS.

When TDD intra-band carrier aggregation is configured, the scheduling restrictions on serving cell where CLI measurements are performed apply on all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

# <End of change 7>