3GPP TSG-RAN WG4 Meeting #116bis			                        R4-2513745

Prague, CZ, October 13rd – 17th, 2025

Agenda item:	7.7.3

Source: 	CMCC

Title: 	Discussion on coexistence for UAV

Document for:	Approval

# 1. Introduction

A new WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV) is approved in RAN #109 meeting [1]. One objective is to analyse co-existence for UAV and consider the details relevant to UAV including antenna array model, and specify requirements if needed. In this contribution, we focus on co-existence for UAV.

# 2. Discussion

## 2.1 Co-existence Scenario

Similar to the scenario assumption of ATG, Figure 1 presents the coexistence scenario based on the WID assumption.

Figure 1. Co-existence scenario for UAV

Compared with TN network, UAV network has similar cell coverage. In WID, TDD with the same TDD pattern is explicitly stated. In the table below, we list four main co-existence scenarios.

Proposal 1: co-existence interference scenarios are suggested as in table 1.

To further narrow down the co-existence scenario scope, some analyses are conducted on each of the four scenarios. By comparing with the existing interference in the UAV PC3 scenario, it was analyzed whether additional co-existence studies are necessary. Scenario 1 was the interference from ground UE to the UAV BS. Since a similar scenario would exist in the UAV PC3 scenario and the assumption in R20 was that BSs for UAV services were deployed on the ground by adjusting the tilt angle upwards, the interference received by the R20 UAV BS was smaller, so no additional co-existence study is needed in Rel-20.

Scenario 2 involves interference from the UAV BS to the ground UE. This interference still exists in the UAV PC3 scenario as well, so no additional coexistence research is required in this WID.

Observation 1: The assumption in the UAV UE antenna type and the increase in power will not affect the interference in scenarios 1 and 2.

Proposal 2: For scenario 1 and 2, no additional coexistence study is needed in Rel-20.

Scenario 3 involves interference from the ground BS to the UAV UE. Assuming that the UAV uses an omni-directional antenna for reception, the interference will be the same as that in Scenario PC3 of the UAV. However, if the reception uses a directional antenna, additional analysis will be required.

Proposal 3: If only omnidirectional antenna is used for reception, no additional analysis is required.

Scenario 4 is the main research scenario of this WI, which focuses on the interference that occurs when the Tx power of UAVs is increased and it affects the ground BS.

Proposal 4: In scenario 4, the increase in power of the UAV led to interference to the ground BS the main focus of this coexistence study.

Table 1: co-existence interference scenario between UAV and TN network

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with UAV | TN UL | UAV UL | no additional coexistence study is needed |
| 2 | TN with UAV | UAV DL | TN DL | no additional coexistence study is needed |
| 3 | TN with UAV | TN DL | UAV DL | If only omnidirectional antenna is used for Rx, no additional analysis is required. |
| 4 | TN with UAV | UAV UL | TN UL | More analysis is needed. |

## 2.2 Co-existence Analysis

Based on the analysis in Section 2.1, coexistence analysis is required in some scenarios. Currently, there are two types of coexistence analysis: coexistence simulation or link calculation. If the coexistence interference situation can be explained through link calculation, complex coexistence simulation can be avoided. Therefore, it is proposed to conduct link calculation based on the existing TN UE PC2 indicators to verify whether coexistence simulation is necessary.

Proposal 5: It is suggested to conduct coexistence analysis through link calculation

Table 2. UAV UE parameters

| Frequency | 2GHz | 2.6GHz | 4.9GHz |
| --- | --- | --- | --- |
| Bandwidth | 25MHz | 100MHz | 100MHz |
| ACLR | 31dB | 31dB | 31dB |
| ACS | 26 | 20 | 33 |

Regarding the interference from the UAV to the ground BS in Scenario 4, the first work to consider is at what distance between the UAV UE and the UAV BS does the UAV need to use a transmission power of 33 dBm. For the reuse of the parameters of the ground macro stations by the UAV BS,

Table 3. UAV BS parameters of REFSENS

| Parameters | Values |
| --- | --- |
| UAV BS IM | 2dB |
| UAV BS SNR | -1dB |
| UAV BS NF | 7dB |
| UAV BS antenna gain | 23dB |

Based on the candidate frequency bands, three coexistence cases of 2GHz, 2.6GHz, and 4.9GHz were set up. The maximum communication distance under the assumption that the UAV UE transmission power is 32dBm in these three cases was calculated. Set the simulation scenarios in R18 ATG as a reference.，the path loss (PL) is composed of components as follows:

PL=PLb=FSPLd, fc+SF+CLα, fc,

The free space path loss (FSPL) in dB for a separation distance d (between ATG UE and ATG BS) in meter and frequency fc in GHz is given by

(6.6-2)

is clutter loss, and  is shadow fading loss represented by a random number generated by the normal distribution, i.e.,  ~.

Table 4: Shadow fading and clutter loss for suburban and rural scenarios

| Elevation | 2GHz 3.5GHz |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS |  |
|  | (dB) | (dB) | (dB) |
| 10° | 1.79 | 8.93 | 19.52 |
| 20° | 1.14 | 9.08 | 18.17 |
| 30° | 1.14 | 8.78 | 18.42 |
| 40° | 0.92 | 10.25 | 18.28 |
| 50° | 1.42 | 10.56 | 18.63 |
| 60° | 1.56 | 10.74 | 17.68 |
| 70° | 0.85 | 10.17 | 16.50 |
| 80° | 0.72 | 11.52 | 16.30 |
| 90° | 0.72 | 11.52 | 16.30 |

Table 5 presents the detailed parameters and calculation results of the three cases.

Table 5. Maximum communication distance for UAV with 32dBm Tx power

| Frequency | 2GHz | 2.6GHz | 4.9GHz |
| --- | --- | --- | --- |
| Bandwidth | 25 MHz | 100 MHz | 100 MHz |
| BS receiver REFSENS | -92 dBm | -86 dBm | -86 dBm |
| UAV Tx / EIRP | 32dBm | 32 dBm | 32 dBm |
| UAV BS antenna gain | 23dB | 23dB | 23dB |
| Clutter loss | 19.52dB | 19.52dB | 19.52dB |
| Shadow fading loss | 8.93dB | 8.93dB | 8.93dB |
| PL | 120dB | 112.8dB | 112.7dB |
| Distance | 12 | 4 | 2.1 |

The calculation in Table 5 shows that for the 2.6GHz and 4.9GHz frequency bands, a high power of 33dBm will be enabled only when the distance is 4km and 2.1km. Referring to Figure 1, at this time, the UAV terminal will cause interference to Ground BS1 and Ground BS2. For the interference to Ground BS1, Figure 2 provides a scenario analysis and assumes that the distance between the UAV and Ground BS1 is 4km/2.1km as calculated in Table 6. We have made the following calculations.

Figure 2. Co-existence scenario for UAV UE with Ground BS1

Table 6. Interference from UAV UE to Ground BS1

| Frequency | 2.6GHz | 4.9GHz |
| --- | --- | --- |
| UAV Tx / EIRP (dBm) | 33 | 33 |
| Bandwidth (MHz) | 100 | 100 |
| UAV Tx power spectral density (dBm/MHz) | 13 | 13 |
| Ground BS interference threshold (dBm/MHz) | -110 | -110 |
| Ground BS antenna gain (dB) | 23 | 23 |
| Side lobe suppression (dB) | 13 | 13 |
| Distance (km) | 4 | 2.1 |
| PL (dB) | 112.8 | 112.7 |
| Clutter loss (dB) | 19.52 | 19.52 |
| Shadow fading loss (dB) | 8.93 | 8.93 |
| UAV ACLR (dB) | 31 | 31 |
| BS ACS (dB) | 46 | 46 |
| ACIR (dB) | 30.9 | 30.9 |
| Additional isolation required | -39.1 | -39 |

It is easy to see from the results calculated in Table 6 that when the UAV's transmission power is 33 dBm, there is still a considerable margin for the UAV's interference on Ground BS1. Therefore, it can be determined that there is no interference from UAV UE to BS1.

Observation 2: When the UAV's transmission power is 33 dBm, there is still a considerable margin for the UAV's interference on Ground BS1.

Another scenario is the interference of UAV on Ground BS2. Figure 3 depicts the interference situation. In this case, we assume that the UAV terminal is located 200 meters directly above Ground BS2 and perform the calculations as described in Table 7.

Figure 3. Co-existence scenario for UAV UE with Ground BS2

Table 7. Interference from UAV UE to Ground BS2

| Frequency | 2.6GHz | 4.9GHz |
| --- | --- | --- |
| UAV Tx / EIRP (dBm) | 33 | 33 |
| Bandwidth (MHz) | 100 | 100 |
| UAV Tx power spectral density (dBm/MHz) | 13 | 13 |
| Ground BS interference threshold (dBm/MHz) | -110 | -110 |
| Ground BS antenna gain (dB) | 23 | 23 |
| Side lobe suppression (dB) | 30 | 30 |
| Distance (km) | 0.2 | 0.2 |
| PL (dB) | 86.8 | 92.3 |
| Clutter loss (dB) | 0 | 0 |
| Shadow fading loss (dB) | 0 | 0 |
| UAV ACLR (dB) | 31 | 31 |
| BS ACS (dB) | 46 | 46 |
| ACIR (dB) | 30.9 | 30.9 |
| Additional isolation required (dB) | -1.6 | -7.1 |

By calculation, it can be seen that when the UAV is 200 meters directly above the Ground BS2, the interference it causes to the ground BS can be disregarded.

Observation 3: When the UAV is 200 meters directly above the ground BS with 33 dBm transmission power, the interference the ground BS suffers can be ignored.

The above two tables selected two typical positions to analyze the interference to the ground when the UAV with 33 dBm Tx power. The results all show that the interference to the ground can be disregarded. In addition to this, compared with TN UL->TN UL, UAV UE has max 6-7dB higher antenna gain/power gain calculated from deployment scenario while victim BS antenna gain towards UAV UE is at least 12-15dB less than that towards TN UE. besides, UAV UE number is much less than TN network. for TN network, one network has 30-50 TN UE but UAV can only support less than 5 UE to avoid crash. So in total, legacy PC3/PC2 TN requirements can be reused and no additional co-existence analysis is needed for scenario 4.

Proposal 6: no additional co-existence simulation is needed for scenario 4, and legacy PC3/PC2 TN UE requirements can be reused.

## 2.3 Rx antenna type

This section mainly focuses on discussion whether omni-directional antennas could meet the downlink coverage requirements. Considering the most adverse scenario, we still select the NLOS with 10° scenario in Table 4, and analyze the downlink coverage distances of the 2.6GHz, 4.9GHz two frequency bands under these assumptions. The calculation results are shown in Table 8.

Table 8. Maximum communication distance for UAV DL

| Frequency | 2.6GHz | 4.9GHz |
| --- | --- | --- |
| Bandwidth | 100 MHz | 100 MHz |
| UAV UE IM | 2dB | 2 dB |
| UAV UE SNR | -1 dB | -1 dB |
| UAV UE NF | 9 dB | 9 dB |
| UAV UE receiver REFSENS | -84 dBm | -84 dBm |
| BS Tx | 46 dBm | 46 dBm |
| UAV BS antenna gain | 23dB | 23dB |
| Clutter loss | 19.52dB | 19.52dB |
| Shadow fading loss | 8.93dB | 8.93dB |
| PL | 124.8dB | 125.3dB |
| Distance | 16km | 9km |

When using omni-directional antennas without considering any antenna gain, the downlink coverage distances for the two frequency bands are 16km and 9km respectively. Therefore, it can be concluded that the omni-directional antennas can meet the downlink coverage requirements.

Proposal 7: Omni-directional antennas can meet the downlink coverage requirements, further consider the DL directional antenna assumption is needed.

# 3. Conclusions

In this contribution, UAV simulation assumptions are discussed with following proposals:

Proposal 1: co-existence interference scenarios are suggested as in table 1.

Observation 1: The assumption in the UAV UE antenna type and the increase in power will not affect the interference in scenarios 1 and 2.

Proposal 2: For scenario 1 and 2, no additional coexistence study is needed in Rel-20.

Proposal 3: If only omnidirectional antenna is used for reception, no additional analysis is required.

Proposal 4: In scenario 4, the increase in power of the UAV led to interference to the ground BS the main focus of this coexistence study.

Proposal 5: It is suggested to conduct coexistence analysis through link calculation

Observation 2: When the UAV's transmission power is 33 dBm, there is still a considerable margin for the UAV's interference on Ground BS1.

Observation 3: When the UAV is 200 meters directly above the ground BS with 33 dBm transmission power, the interference the ground BS suffers can be ignored.

Proposal 6: no additional co-existence simulation is needed for scenario 4, and legacy PC3/PC2 TN UE requirements can be reused.

Proposal 7: Omni-directional antennas can meet the downlink coverage requirements, further consider the DL directional antenna assumption is needed.

# 4. Reference

[1] RP-252954, New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV), HUAWEI