3GPP TSG-RAN WG4 Meeting #116bis				R4-2513778

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	9

Source: 	KDDI Corporation

Title: 	UE feature list for Rel-19 non-collocated Type 4

Document for:	Approval

- Introduction

This contribution provides the RAN4 UE feature list for Rel-19 non-collocated Type 4.

- NonCol_intraB_ENDC_NR_CA_Ph2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NonCol_intraB_ENDC_NR_CA_Ph2 | 47-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1. | 33-1 (RAN4 R18 feature) | Yes | N/A | Intra-band non-collocated NR CA operation with up to 4layer per CC is not supported. | Per BC | N/A | FR1 only | N/A | Supported for band n77/n78 only | Optional with capability signaling |
|  | 47-2 | Support of inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation | Indicates the UE supports TDD-TDD inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation with MTTD/MRTD requirements according to Table 7.5.2.1-1/Table 7.6.2.1-1 in TS 38.133 and UE RF requirements for inter-band non-contiguous EN-DC with overlapping or partially overlapping bands including 7.10B in TS 38.101-3. And the UE also supports inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation with MRTD according to Table 7.6.3 in TS 38.133 and UE RF requirements for inter-band non-contiguous EN-DC with overlapping or partially overlapping bands except for 7.10B in TS 38.101-3. | 33-2 (RAN4 R18 feature) and 2-19 (R16 RAN4 feature) | Yes | N/A | Inter-band non-contiguous EN-DC with overlapping or partially overlapping bands operation with up to 4layer per CC is not supported. | Per BC | N/A | FR1 only | N/A | Supported for band B42 and n77/n78 only | Optional with capability signaling |