3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2513786

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Further discussion RRM core requirement for beam management

Agenda item:		6.11.4

Document for:	Approval

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two confirmed use cases (i.e., beam management and positioning accuracy enhancements) [2]. Particularly for the use case of beam management, it is required to provide specification support for DL TX beam prediction for both UE-sided and NW-sided model, and correspondingly necessary RAN4 core requirements for this use case and LCM procedure including performance monitoring [2], as follows:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In RAN#109, this work item has been extended for one quarter by considering the following outstanding issues [RP-252840]:

| Task(s) within work which are not complete: | Core requirements for CSI - CSI prediction framework - Requirements for LCM Core requirements for beam prediction - measurement/observation window and reporting delay - TCI state switching requirements - LCM requirements Core requirements for positioning - Reporting delay requirements |
| --- | --- |

In this contribution, we would like to further discuss on core requirement for AI-based beam management use cases, including the impact on both BM-Case1 and BM-Case2.

# 2. RAN4 Core Requirement for supporting UE-Sided Model

## 2.1 AI-BM model inference delay requirement

Based on the existing discussion, seems the group have the common understanding to specify the requirement for model inference for UE-sided model. Specifically, we expect corresponding RRM requirement shall be introduced: to guarantee (1) the accuracy of inference results accordingly to the agreed KPI; (2) the delay of reporting inference results (containing the inference latency).

### 2.2.1 Absolute RSRP accuracy for predicted RSRP

In previous RAN4 discussion, the definition of the absolute RSRP accuracy is provided:

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

In RAN4#114, RSRP accuracy is agreed to one of the KPIs if RSRP is contained in the report. Furthermore, in RAN4#114bis, it is discussed that the absolute RSRP requirement is applicable to which beam:

Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams

In current RAN1 design, only the beam with the largest RSRP is reported by using the absolute RSRP reporting, which is similar to current L1-RSRP report in CSI reporting.

Agreement For UE-sided model inference, support the following report format (i.e., CSI field mapping order) for BM-Case1, for beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #K. RSRP #1. differential RSRP #2. …. differential RSRP #K. - CRI or SSBRI #k is mapped to RSRP #k, where k = 1,2,…,K - RSRP #1 is absolute predicted RSRP - Differential RSRP #2~#K are differential predicted RSRP with reference to the largest predicted RSRP corresponding to CRI or SSBRI #1

The following agreement is achieved in RAN4#116:

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

It should be noted that in the existing L1-RSRP measurement accuracy conformance testing requirement, only the “strongest SSB” within the periodic L1-RSRP report are checked for the absolute accuracy requirement, while the 2nd strongest SSB is exanimated for the diff-RSRP value. We copied the relevant testing procedure in TS38.533 for reference:

4. The UE shall start sending valid L1-RSRP reports, a valid report shall meet the absolute L1-RSRP requirement for SSB#0 and the relative L1-RSRP requirement for SSB#1. The SS shall check following requirements: - R1: The L1-RSRP value of SSB#0 reported by the UE is compared to the expected L1-RSRP value for SSB#0. If the value is outside the limits in Table 7.7.4.1.5-3, Table 5.7.4.1.5-3a or Table 7.7.4.1.5-4 (depending on the test configuration) or the UE fails to report the measurement value for SSB#0, the number of failed iterations for R1 is increased by one. Otherwise, the number of passed iterations for R1 is increased by one. - R2: The DIFF-RSRP value of SSB#1 reported by the UE is compared to the expected DIFF-RSRP value. If the resulting value is outside the limits in Table 5.7.1.1.5-5 or the UE fails to report the measurement value for SSB#1, the number of failed iterations for R2 is increased by one. Otherwise, the number of passed iterations for R2 is increased by one.

Accordingly, the following proposal is given for RSRP accuracy issue.

Proposal 1: The absolute RSRP accuracy requirement shall only be applied to Top-K of predicted beams:

- K = 1, at least for Rel-19 progress.

### 2.2.2 Relative RSRP accuracy for predicated RSRP

Similar to absolute RSRP reporting, as agreed in RAN4#116, if RSRP prediction is reported, the relative RSRP requirements applies to the top-K, where the value of K is still FFS.  For simplicity, it is proposed to specify requirement similar as legacy relative test, i.e., only have relative reporting to be tested between Top-1 and Top-2 beams.

Proposal 2: The relative RSRP accuracy requirement shall only be applied to Top-K of predicted beams:

- K = 2, i.e., relative RSPR between Top-1 and Top-2 beam, at least for Rel-19 progress.

Based on the discussion in RAN4#114bis, the following agreement is achieved for relative RSRP accuracy:

Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

The definition of relative RSRP accuracy shall be defined as:

Proposal 3: For relative RSRP accuracy requirement shall be updated as below:

- Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value given by absolute RSRP reporting.

  - Reported L1-RSRP can be predicted,

  - Reported L1-RSRP can also be measured RSRP if Set B is subset of Set A

- Relative RSRP accuracy requirements apply for Case 1, and also for Case 2

### 2.2.3 Metric/KPIs for beam ID prediction accuracy

The following agreement is achieved in RAN4#114 for beam ID prediction accuracy:

- Agreements in ad-hoc session (R4-2503012) Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

In RAN4#114bis, the definition of beam ID prediction accuracy is agreed as

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

For the remaining issue, including “FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation”, since there is no time left to evaluate 2nd ranked predicted beam’s accuracy, we propose to only consider top-1 in Rel-19.

Proposal 4: For the beam ID prediction accuracy in Rel-19, only top-1 (i.e., Top-N, N=1) predicted beam is considered in the successful rate evaluation/requirement.

The value of K can be further decided in performance part, together with x value, which shall be based on numerical results. Therefore, the beam ID prediction accuracy definition can be updated as

Proposal 5: The beam ID prediction accuracy shall be updated as:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x and K values in performance part

## 2.2 AI-BM TCI state switching delay requirement

Based on RAN4#115 and #116, the conditions for TCI state knowledge are agreed as:

<Agreement in RAN4#115> Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

<Agreement in RAN4#116> Issue 2-3: TCI State Handling Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

One of the remaining issue is to confirm the condition that the target TCI state is known shall applied for both AI-BM case 1 and case 2.

Proposal 6: Confirm the below condition for determining the target TCI state is known shall applied for both AI-BM case 1 and case 2:

-  The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1  RSRP report of the target TCI state (clause 8.10.2 of 38.133)

## 2.3 LCM related core requirement

Based on RAN4 agreement achieved in RAN4#116, here is no need to create deactivation requirements for AI functionality, while UE has to be ready to start measurements for inference after sending RRC reconfiguration complete:

Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

RAN4 has discussed the activation delay for AI/ML functionality activation based on the following RAN2 agreement:

RAN2#129 agreements: - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Based on the two kinds of interpretations of above RAN2 agreed procedure, RAN4 has asked RAN2’s understanding on this [R4-2508085].

- Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4

- Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

RAN2 has discussed the following two interpretations and reply RAN4 with the following agreements [R2-2506520/ R4-2513016]:

Agreements

- RRC processing delay shouldn’t be impacted by the model loading delay

- If the UE is ready for inference (i.e. the model is ready to generate inference result based on configuration) by end of RRC processing delay, it reports configuration applicable.  If not, it reports configuration inapplicable and doesn’t set the release flag.   The network is not expected to release inference configuration.

- Once the configuration is applicable (i.e. the model is ready to generate inference result based on configuration), UE reports applicability to network via UAI (applicable to all AI/ML based CSI reporting).

- On the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation from RAN2 point of view and no further RAN2 work.

- RRCReconfigurationComplete containing applicability reports has a processing latency requirement of 16 ms with respect to the reception of RRCReconfiguration, from RAN2 point of view.

Based on above RAN2 agreement, RAN4 shall revisit the agreement that “UE has to be ready to start measurements for inference after sending RRC reconfiguration complete”.

Proposal 7: RAN4 shall revisit the agreement that “UE has to be ready to start measurements for inference after sending RRC reconfiguration complete”, by considering UE can report configuration inapplicable and doesn’t set the release flag.

For the time duration for an AI functionality to become available for inference, RAN2 conclude that it is up to UE implementation from RAN2 point of view and no further RAN2 work. Then it is RAN4’s scope to specify or not specify the requirements for activation delay, while it is still not clear to us the starting/ending point of the activation procedure.

Proposal 8: RAN4 may not need to define AI functionality activation delay requirement in Rel-19, unless

- RAN4 reach the agreement on the starting and ending points for activation delay procedure

- RAN4 has clear implication of relevant requirement on UE implementation to load practical AI/ML model

## 2.4 Performance monitoring core requirement

It was agreed not to define performance monitoring delay requirements for LCM purpose. However, if the monitoring delay is identified as critical in the future, this issue can be revisited.

For accuracy requirement, the following agreement was reached in RAN4#115 meeting:

Agreement from RAN4#115: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed.

Agreement (RAN 1 #120b)

For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type:

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |  |
| SP report | Not support | Support | Support |  |
| P report | Support | Support | Support |  |

Agreement (RAN1 #119)

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Agreement (RAN 1 #120bis)

- For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB

RAN1 achieved following agreement for performance monitoring mechanism and metric:

RAN1#121 Agreement

For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N)

- Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring

  - condition:

    - for the transmission occasion of monitoring resources, it has a linked inference report

    - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference

  - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance.

- Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig

- the size of CSI field associated with the RS-PAI is log2(N+1)

RAN1 has agreed on the definition of report quantity of beam prediction accuracy, which can be defined as the total count of accurate reference signal prediction instance(s), Np, that meets the conditions and Np is calculated over N valid performance monitoring instances. And the condition in the definition of the report quantity is different from the beam prediction accuracy metrics being discussed in RAN4 as part of the performance requirements. Hence, if RAN4 introduce performance monitoring accuracy requirement, RAN4 needs to clarify the purpose of this requirement: if it is related to inference performance, it should be guaranteed by inference performance requirement; if it is related to performance monitoring requirement, the UE expected genie performance monitoring behavior needs to be clarified.

Proposal 9: RAN4 can define performance monitoring requirement in Rel-19 only if

- RAN4 can agree on the testing condition in which expected performance monitoring result is known, i.e., a condition in which UE is expected to have perfect good inference condition and then the expected performance monitoring result is Np = N

# 3. Conclusion

In this contribution, we would like to further discuss on core requirement for AI-based beam management use cases, including the impact on both BM-Case1 and BM-Case2.

AI-BM model inference delay requirement

Proposal 1: The absolute RSRP accuracy requirement shall only be applied to Top-K of predicted beams:

- K = 1, at least for Rel-19 progress.

Proposal 2: The relative RSRP accuracy requirement shall only be applied to Top-K of predicted beams:

- K = 2, i.e., relative RSPR between Top-1 and Top-2 beam, at least for Rel-19 progress.

Proposal 3: For relative RSRP accuracy requirement shall be updated as below:

- Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) -  (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), where the beam index n owns the largest reported value given by absolute RSRP reporting.

  - Reported L1-RSRP can be predicted,

  - Reported L1-RSRP can also be measured RSRP if Set B is subset of Set A

- Relative RSRP accuracy requirements apply for Case 1, and also for Case 2

Proposal 4: For the beam ID prediction accuracy in Rel-19, only top-1 (i.e., Top-N, N=1) predicted beam is considered in the successful rate evaluation/requirement.

Proposal 5: The beam ID prediction accuracy shall be updated as:

The successful rate for the correct prediction,

  - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB,

  - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation

    - FFS on N values

  - FFS on x and K values in performance part

AI-BM TCI state switching delay requirement

Proposal 6: Confirm the below condition for determining the target TCI state is known shall applied for both AI-BM case 1 and case 2:

-  The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1  RSRP report of the target TCI state (clause 8.10.2 of 38.133)

LCM related core requirement

Proposal 7: RAN4 shall revisit the agreement that “UE has to be ready to start measurements for inference after sending RRC reconfiguration complete”, by considering UE can report configuration inapplicable and doesn’t set the release flag.

Proposal 8: RAN4 may not need to define AI functionality activation delay requirement in Rel-19, unless

- RAN4 reach the agreement on the starting and ending points for activation delay procedure

- RAN4 has clear implication of relevant requirement on UE implementation to load practical AI/ML model

Performance monitoring core requirement

Proposal 9: RAN4 can define performance monitoring requirement in Rel-19 only if

- RAN4 can agree on the testing condition in which expected performance monitoring result is known, i.e., a condition in which UE is expected to have perfect good inference condition and then the expected performance monitoring result is Np = N

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402389, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN#110, Feb. 2024.

[5] 3GPP TR 38.810, v16.7.0, “NR; Study on test methods”.

[6] 3GPP TR 38.827, v16.8.0, “Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)”.

[7] R4-2405652, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#110bis, April 2024.

[8] R4-2406617, “WF on AI/ML for NR Air Interface”, Qualcomm, RAN#110bis, April 2024.

[9] R4-2409761, “Discussion on testability and interoperability issues for beam management”, Samsung, RAN#111, May 2024.

[10] R4-2410570, “WF on AI/ML”, Qualcomm, RAN#111, May 2024.

[11] R4-2411635, “Further discussion on testability and interoperability issues for beam management”, Samsung, RAN#112, Aug. 2024.

[12] R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm, RAN#112, Aug. 2024.

[13] R4-2416492, “Further discussion on beam management core requirements”, RAN#112bis, Oct. 2024.

[14] R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

[15] R4-2418093, “Further discussion on beam management core requirements”, RAN#113, Nov. 2024.

[16] R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm, RAN#113, Nov. 2024.

[17] R4-2501964, “Further discussion on beam management core requirements”, RAN#114, Feb. 2025.

[18] R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm, RAN#114, Feb. 2025.

[19] R4-2504216, “Further discussion on beam management core requirements”, RAN#114bis, Apr. 2025.

[20] R4-2505105, “WF on the requirements for AI/ML air interface”, Qualcomm, RAN#114bis, Apr. 2025.

[21] R4-2508081, “Updated simulation assumption for BM”, vivo, NTU, Nokia, etc., RAN#115, May 2025.

[22] R4-2508080, “WF on NR_AIML_air_part1”, Qualcomm, RAN#115, May 2025.

[23] R4-2511890, “WF on AI/ML for air interface for [116][131]”, Qualcomm, RAN#116, Oct. 2025.

# 5. Appendix – Existing Agreements on AI-BM

In RAN4#110bis, for the metrics/KPIs for Beam Management requirements/tests, the following agreement is achieved:

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

In RAN4#111, for reported measurements and ground truth, the following agreements are achieved:

Issue 2-3: Reported measurements and ground truth Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

And the following agreement is achieved in RAN4#112:

Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-7: UE reporting for network side models Agreement: postpone discussion until other WGs define any reporting mechanisms

And the following agreement is achieved in RAN4#112bis:

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point.

The following agreement is achieved in RAN4#113:

Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. ### Agreements in ad-hoc session (R4-2420335): Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

The following agreement is achieved in RAN4#114:

- Agreements in ad-hoc session (R4-2503012) Issue 2-1: Metrics/KPIs for beam prediction If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options Issue 2-4: QCL Source RS for TCI states Agreement: wait for RAN1 discussion to conclude without sending any LS ### Agreements in main session Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

The following agreement is achieved in RAN4#114bis:

## RRM core requirement and testing framework for beam management ### Agreements in main session: Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" Issue 2-2: Absolute RSRP accuracy Agreement: - Absolute RSRP accuracy requirement applies to Top-1 of predicted beams at least - FFS whether to apply to other beams Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2 Issue 2-4: Measurement period for inference Agreement: - For measurement delay component of the prediction delay: - Use “legacy” L1-RSRP measurement as the baseline for Case 1 - FFS whether same values for M,N,P should be reused - FFS for case 2 Issue 2-6: Test system setup Companies are invited to bring further analysis on the test system setup. At least the following aspects to be considered in the analysis: - Whether the test can/should reflect performance in the field and whether field data should be usable in the test - What channel model should be used - UE prediction model input (e.g. whether any spatial information like UE Rx beam, UE beam pattern, etc is used) - Pros and cons of proposed single AoA and multi AoA setups For the multi AoA test setups, companies are invited to bring further analysis on the following aspects: - Define candidate test setups which are already feasible (e.g. based on eIFF, MPAC etc.) - Propose simplified channels suitable for the defined candidate test setup(s) - Validate the proposed simplified channels through AIML BM system level simulations, especially by comparing the performance with the non-simplified (theoretical) channels. - Assess the implementation of the simplified channels on the candidate systems, after the probe layout and the channel model have been clarified.

The following agreement is achieved in RAN4#115:

## RRM core requirement and testing framework for beam management ### Agreements in main session: Issue 2-2: Measurement period for inference Agreement: Case 1: The starting point is to use M, N, P (reference the values from an existing clause in the same version of the spec where the measurement period is defined) Case 2: postpone discussion until RAN1 concludes Issue 2-3: Rx beam knowledge and TCI handling Agreement: - if the predicted Tx beam in Set A is QCL Type-D to a known Tx beam, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, depending on UE capability, the corresponding Rx beam may be known or unknown. - Known Rx beam means no additional Rx beam sweeping is needed - Unknown Rx beam means additional Rx beam sweeping is needed. - Rx beam is known mean TCI state is known if conditions below are also met The following conditions related to whether the TCI state is known or unknown is defined as the starting point. Companies are invited to bring analysis. - The target TCI state is known when the following is met: - The TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement and the UE has sent at least one L-1 RSRP report of the target TCI state (clause 8.10.2 of 38.133) - Applies for case 1 - FFS for case 2 - Whether RS resource is only from set B should be clarified - FFS on whether prediction also impacts if a TCI state is known or unknown - Companies are invited to bring analysis on how prediction would impact whether a TCI state is known or unknown - FFS on the time delay if prediction is used as a condition for a known TCI state

The following agreement is achieved in RAN4#116:

- RRM core requirement and testing framework for beam management - Agreements in main session: Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+time for the first available reporting occasion Issue 2-3: TCI State Handling Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability. Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided. - Agreements in ad-hoc session (R4-2511891) Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes Issue 2-6: Test system setup Agreement: Consider multi AoA based RRM testing setup as baseline(enhanced IFF system with 4 probes used for FR2 RRM conformance testing) Single AoA tests can also be performed in the multi AoA based RRM test setup Further analyze what spatial channel models can be emulated with this test setup - Companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup Single AoA tests are not precluded Issue 2-7: Test system channel model Agreement: Companies to bring analysis into what simplifications to CDL channels are needed to be able to emulate the channels in the enhanced IFF chamber. Take CDL-C Umi as one example Other channel models can be further discussed