3GPP TSG-RAN WG4 Meeting #116bis		R4-2513796

Prague, Czech Republic, 13th – 17th October, 2025

Agenda item:		6.12.2.1

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 MIMO UEIBM

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of performance part of this WI are described in [1] as:

- Specify necessary performance requirements for the enhancements listed above.

In last RAN4#116 meeting, RAN4 finalized the core part discussion on RRM core requirements. In last RAN4#116 meeting, the open issues of RRM performance part of Rel-19 MIMO Phase 5 are captured in [2]. In this contribution, we provide our view of the RRM performance part of UEIBM.

- Discussion

In last RAN4#116 meeting, RAN4 finalized the RRM core requirements.

Test cases

In last RAN4#116meeting, the test scope issues are captured in [2]:

#### Issue 2-1-1: Test scope <Way forward > QC; Apple; Samsung; CMCC; Huawei; ZTE; Nokia; Ericsson; MediaTek. High level: Define TCs to verify UEIBM; Y; Y; Y; Y; Y; Y; Y; Y; Y. Whether to cover Event-2/Event-1/Event-7?; Event-2: Y Event-1/Event-7: FFS; Event-2: Y Event-1/Event-7: No; Event-2: Y Event-1: limited TC Event-7: No; All; Event-2 FFS on others; All; Event-2; Event-2&7; Event-2. Whether to cover Mode A and Mode B?; Either A or B; Both; Both; Both; Mode A&B as Test configurations. Can be skip depends on UE capability; Mode A&B as Test configurations. Can be skip depends on UE capability. Event-trigger for Intra-cell BM for SSB; Y; N; Y; Y; Y; Y; Y; Y; Y. Event-trigger for Intra-cell BM for CSI-RS; Y; Y; Y; Y; Y; Y; Y; Y; Y. Event-trigger for Inter-cell BM for SSB; Y; Y; Y; Y; FFS; Y; Y; ?; Y. Event-trigger for Inter-cell BM for CSI-RS; N; N; N; Y; Y; N; N. Whether to cover no time window and eventInstanceCount-r19 > 1?; N; N; Y; Y; Y; Y; Y. Frequency Range – FR2?; Y. No FR1; Y. FR2; Y. FR2. Beam switching delay for TCI states switching; Y Both known and unknown TCI; N; FFS on core req.; FFS on core req.; No MAC CE-based TCI state switching FFS on DCI-based TCI state switching. L1 measurement reporting delay; Y. CMI; N.

According to the core requirements, RAN4 to add new core requirements for both SSB and CSI-RS based event triggered L1-RSRP measurement and event triggered reporting delay.

RAN4 to discuss the test coverage of UEIBM, at least the following should be tested:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

For the test coverage of different events, prioritize to test event-2. We are open to choose limited TCs for event-1. For event-7, we prefer not to add test case for it.

For the testability of event-7, we don’t think legacy chamber for FR2 test can be used for event-7.

Firstly, in RAN1 and RAN4 RRM test, the event-2 is the prioritized one.

  - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

  - The number of N beams (new) can be {1, 2, 3, 4} which is configured

In OTA test, one TCI states means one beam in FR2.

In typical legacy Chamber, for example, there are only 4 fixed probes with fixed separation angel.

Left: Keysight                                                          Right: R&S

Even in Rel-18 MIMO OTA, for FR2 MIMO OTA testing, 3D Multi-Probe Anechoic Chamber (3D-MPAC) was introduced by 6 probes. The 6 probes are designed with different power weights and delay to simulate the required channel model at the test zone. The probes are more like a quasi 1AoA which is not suitable for more AoA setting in RRM tests.

Figure B.2.1-1: 3D MPAC system layout for NR FR2 MIMO OTA testing

Although we haven’t agreed details of how to test event-2, the most possibility or our proposal is that RAN4 will design two beams: one is for current beam, the other is for new beam. Then UE estimates the L1-RSRP for current beam and/or new beam and trigger the event report to TE. No matter the tests focus on L1-RSRP only or TCI state switching, the testability has no issue for FR2 chamber with 4 probes with 2 active probes if the number of new beams is limited.

In Rel-15/Rel-16/Rel-17 RRM test, RAN4/RAN5 usually used single probe or up to 2 active probes for up to 2 beams in FR2.

if 2AoAs (Rel-15, e.g. Setup.3),

30 degrees, = “1&2”

60 degrees, = “2&3” or “3&4”

90 degrees, = “1&3”

120 degrees, = “2&4”

150 degrees, = “1&4”

In Rel-18 multi-Rx WI and MIMO WI, RAN4 discussed whether to use more AoAs for more beams. (TR38.751 for RF)

If 3AoAs from 4 fixed probes,

1&2&3 = 30(1 to 2) +90 (1 to 3)

1&2&4 = 30(1 to 2) +150(1 to 4)

1&3&4 = 90(1 to 3) +150(1 to 4)

2&1&3 = 60(2 to 3) +90 (1 to 3)

2&3&4 = 60(2 to 3) +120(2 to 4)

1&3&4 = 60(3 to 4) +150(1 to 4)

1&3&4 = 90(1 to 3) +150(1 to 4)

1&2&4 = 120(2 to 3) + 150 (1 to 4)

30+90, 30+150, 60+90, 60+120, 60+150, 90+150, 120+150 are valid AoAs.

The definition of event-7 in RAN1 is:

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

In Rel-17 MIMO unified TCI framework, the activated TCI states can be up to 8. Then Q is configured by gNB. For example, find the 3rd best from the 8 TCI states which means UE shall evaluate the L1-RSRP for all active TCI states then pick up which is the 3rd best.

If the event-7 is using just two active TCI states, it is the same as event-2. Q-th can be 1st or 2nd. Otherwise, it’s hard to support more beams from 4 fixed probes.

Therefore, we have concern for testability for event-7.

For the test coverage of mode-A and mode-B:

RAN1 introduced both Mode-A and Mode-B as:

Mode A (dynamically scheduling UCI by gNB)

Mode B (UCI in pre-configured resource(s) for second UL channel)

Mode A is for UE request an uplink resource for beam report while mode B uses the pre-configured resource for beam report. In the UE feature list discussion, mode A is the basic UE capability. Mode B depends on UE capability. However, mode B is quite typical used due to it can save the scheduling info. We support to cover both mode A and mode B.

In core requirements discussion, RAN4 achieved the core requirements in last RAN4#116 meeting for the case of eventInstanceCount-r19 > 1. We support to define the test case to verify it.

Observation 1: The testability of Event-7 has issue by using existing chamber for more AoAs.

Proposal 1: RAN4 to add new test cases to cover:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1.

No TCs are added for event-7.

TCs are defined to cover both Mode A and Mode B.

We support to define such TC for the case of eventInstanceCount-r19 > 1.

Specify the test cases for FR2 test.

Therefore, we propose the test cases for UEIBM as:

Proposal 2: Test case list for UEIBM:

TC1: event-2 sTRP serving cell SSB based L1-RSRP + FR2, mode A

TC2: event-2 sTRP serving cell CSI-RS based L1-RSRP + FR2, mode B

TC3: event-2 sTRP additional PCI cell SSB based L1-RSRP + FR2, mode A

TC4: event-1 sTRP serving cell CSI-RS based L1-RSRP + FR2, mode B

TC5: event-2 sTRP serving cell SSB based L1-RSRP + FR2, mode B, eventInstanceCount-r19 = 1

- Conclusion

In this contribution, we analyze the test cases of Rel-19 MIMO UEIBM and provide our proposals:

Observation 1: The testability of Event-7 has issue by using existing chamber for more AoAs.

Proposal 1: RAN4 to add new test cases to cover:

- sTRP serving cell SSB based L1-RSRP

- sTRP serving cell CSI-RS based L1-RSRP

- sTRP additional PCI cell SSB based L1-RSRP

Prioritized for the tests are for event-2. We are open to choose limited TCs for event-1.

No TCs are added for event-7.

TCs are defined to cover both Mode A and Mode B.

We support to define such TC for the case of eventInstanceCount-r19 > 1.

Specify the test cases for FR2 test.

Proposal 2: Test case list for UEIBM:

TC1: event-2 sTRP serving cell SSB based L1-RSRP + FR2, mode A

TC2: event-2 sTRP serving cell CSI-RS based L1-RSRP + FR2, mode B

TC3: event-2 sTRP additional PCI cell SSB based L1-RSRP + FR2, mode A

TC4: event-1 sTRP serving cell CSI-RS based L1-RSRP + FR2, mode B

TC5: event-2 sTRP serving cell SSB based L1-RSRP + FR2, mode B, eventInstanceCount-r19 = 1

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2512130, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung