3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513817

Prague, Czech, 13th – 17th October 2025

Agenda Item:	6.8.3.1

Source: 	Nokia

Title: 	Transmitter co-existence spurious emission requirements

Document for:	Approval

# 1.	Introduction

The revised WI on NR base station (BS) RF requirement evolution for FR1/FR2 and testing was approved at TSG RAN#109 [1]. One of the objectives of this WI is related to the request from ETSI/TFES in [2]:

Transmitter co-existence spurious emission requirements - Evaluate co-existence transmitter spurious emission limits according to the request from ETSI/TFES - Improve transmitter co-existence spurious emission requirements if needed. - Note: start the work after March, 2025 and focus on bands in the upper region of FR1 with priority above 5.9 GHz..

This topic was discussed at TSG RAN4#116 [3] and the WF was agreed [4]. System level simulation assumptions for calibration have been agreed to be based on TR 38.922 [5]. This contribution provides proposals based on simulation results using the agreed assumptions to progress this topic in RAN4.

# 2.	Discussion

It was agreed in the WF [4]:

The following is agreed towards simulation assumptions and their alignment. - CDF %-tile For calibration: Use CDF curve of coupling loss to align results. Aim to follow the stepwise approach for results alignment, using the following Excel spreadsheet as starting point. For results derivation: Select just one value: TBD - ISD For calibration: select just one value (select 450m, or 500m): 500m For results derivation: [Same value as for calibration.] - Grid shift For calibration: select just one value (50%, or 100%): 100% For results derivation: TBD - Antenna correlation value for the OoB antenna gain For calibration: first 0, then 1. For results derivation: (0, 0.5, or 1): TBD - Evaluation scenarios Grid: 19sites, 3 sectors, wrap around For calibration: - First step: WA for BS-BS (Urban macro in TR 38.922) - Second step: MR for BS-UE (Dense urban in TR 38.922) For results derivation: [Same value as for calibration.] - Pathloss model For calibration (decide which model to use): FFS as further inputs expected; aim to discuss offline after the meeting to align before RAN4#116bis. For results derivation: [Same value as for calibration.] - Other assumptions For calibration: Based on TR 38.922, with interfering system at 2GHz and victim system at 6.425GHz. - Timeline of this objective Proceed with the assumption that the Core objective for Tx spur co-ex is extended by 1 quarter till December 2025. - Simulation calibration in October meeting. - Aim for conclusions and final CRs (if any) for November meeting. - How and where to capture outcomes of this discussion Companies agree to capture conclusions in a TR. Which TR to use as baseline is TBD

2.1	Downlink

The current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL (UE receiver) protection [6] was derived with the worst case of BS to UE MCL of 53 dB for the microcell environment [7]:

- 53 dB for the microcell environment With the worse case of MCL = 53 dB the spurious level can be derived as follows: Spurious Level – MCL < Thermal Noise Noise Figure of the UE receiver: NF = 9dB Spurious Level < Thermal Noise + MCL = kTo + NF + MCL = –174dBm/Hz + 9dB + 53dB Spurious Level < –52dBm/1MHz

To provide a like-for-like comparison, the BS to UE CL from the system level simulation using the agreed simulation assumptions in dense urban environment at 6425 MHz frequency range are provided in figures 1 and 2 below. As the AAS antenna gain pattern in the out-of-band frequency range has not been agreed in RAN4, the simulation results are provided here with 0 (totally uncorrelated) and 1 (totally correlated) correlation between 2 antenna elements at the aggressor (2 GHz) frequency band in view from the victim (6425 MHz) frequency band.

(a) Overall								(b) Zoom-in

Figure 1: BS to UE CL at 6425 MHz carrier frequency (0 correlation)

(a) Overall								(b) Zoom-in

Figure 2: BS to UE CL at 6425 MHz carrier frequency (1 correlation)

It can be observed from figures 1 and 2 that the MCL in the simulated dense urban environment is about 58 dB with 0 correlation and about 48 dB with 1 correlation. Therefore, a real-life scenario where the correlation is between 0 and 1, the MCL in the dense urban environment is between 58 dB and 48 dB. As such the 53 dB MCL (the middle point between 58 dB and 48 dB) used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency.

2.2	Uplink

The current -49 dBm/MHz transmitter co-existence spurious emission requirements for UL (BS receiver) protection [8] was derived with the worst case of BS to BS MCL of 67 dB for the macrocell environment [9]:

###### 7.4.1.2.1.3 BS-to-BS propagation model The test scenario described in [20] implies that the base stations of the two operators are in line-of-sight with clearance of the first Fresnel zone. Therefore the propagation model applied is the free space loss model (see [17]). The base station antenna gain used to calculate the power received in this case is 10 dB, instead of 13 dB, to consider the tilt of the antennas. Thus, since the distance between BSs of different operators is 577/2 m, the path loss is 87 dB, and, including the antenna gains, 67 dB.

To provide a like-for-like comparison, the BS to BS CL from the system level simulation using the agreed simulation assumptions in urban macro environment at 6425 MHz frequency range are provided in figures 3 and 4 below. The simulation results are provided here with 0 (totally uncorrelated) and 1 (totally correlated) correlation between 2 antenna elements at the aggressor (2 GHz) frequency band in view from the victim (6425 MHz) frequency band.

(a) Overall								(b) Zoom-in

Figure 3: BS to BS CL at 6425 MHz carrier frequency (0 correlation)

(a) Overall								(b) Zoom-in

Figure 4: BS to BS CL at 6425 MHz carrier frequency (1 correlation)

It can be observed from figures 3 and 4 that the MCL in the simulated urban macro environment is about 60 dB with 0 correlation and about 47 dB with 1 correlation. Therefore, a real-life scenario where the correlation is between 0 and 1, the MCL in the dense urban environment is between 60 dB and 47 dB. As such the 67 dB MCL used to derive the current -49 dBm/MHz transmitter co-existence spurious emission requirements for UL protection seems to be too optimistic at 6425 MHz carrier frequency.

# 3.	Conclusion

This contribution has provided system level simulation results in dense urban scenario to progress the objective of the WI for transmitter co-existence spurious emission requirements in RAN4.

Observation 1: The 53 dB BS to UE MCL used to derive the current -52 dBm/MHz transmitter co-existence spurious emission requirements for DL protection seems to remain reasonable at 6425 MHz carrier frequency.

Observation 2: The 67 dB BS to BS MCL used to derive the current -49 dBm/MHz transmitter co-existence spurious emission requirements for UL protection seems to be too optimistic at 6425 MHz carrier frequency.

Proposal 1: To keep the 53 dB BS to UE MCL assumption for deviation of the transmitter co-existence spurious emission requirements for DL protection in the upper region of FR1.

Proposal 2: To further consider the 67 dB BS to BS MCL assumption for deviation of the transmitter co-existence spurious emission requirements for UL protection in the upper region of FR1.

# References

[1]	RP-252578, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, Huawei, Nokia, Ericsson.

[2]	RP-242432, “Transmitter spurious emissions: co-existence requirement limits for high frequency bands”, ETSI TC MSG/TFES

[3]	R4-2511450, “Topic summary for [116][303] NR_BS_RF_Part2_CLTA_SE”, Moderator (Huawei).

[4]	R4-2512578, “Way Forward for [116][303] NR_BS_RF_Part2_CLTA_SE (SE part)”, Huawei.

[5]	3GPP TR 38.922 v19.2.0: “NR; Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”.

[6]	3GPP TS 38.104 v19.2.0: “NR; Base Station (BS) radio transmission and reception”.

[7]	R4-99a01, “BS TDD Spurious Emission Requirements for Co-Existence UTRA-FDD/ UTRA-TDD”, Siemens.

[8]	R4-030641, “General corrections on co-existence and co-location requirements for UTRA-FDD BS”, Nortel Networks, Ericsson, Siemens, Motorola.

[9]	3GPP TR 25.942 v18.0.0: “Radio Frequency (RF) system scenarios”.