3GPP TSG-RAN WG4 Meeting #116	R4-2513827

Prague, Czechia, October 13th – October 17th, 2025

Source:	Nokia

Title:	Performance requirements using SU SCM

Agenda item:	7.5.3

Document for:	Discussion

Introduction

It has been agreed in RAN#109 [1] to specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 using the rCDL-C1 spatial channel model from TR 38.753.

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In this contribution we treat four main topics:

- Specification of (optional) PDSCH requirements for 8T8R 2CW.

- Specification of requirements for PMI performance

- Update of rCDL-C1 model with RAN1 Rel-19 values.

- Addressing concerns of frequency generalization and PMI bias.

In this first meeting we do not have a companion simulation contribution; however, the prior study simulations remain valid, including the study simulation summary [3].

Specification of PDSCH requirements

The preceding Rel-19 study (FS_NR_demod_SCM) has carried out all the technical work to define normative requirements for the 8T8R 2CW case. Most details are present in TR 38.753, with the only decision left to make being whether to have a single operating point for both CWs or to have separate operating points for both CWs.

Requirement configuration

To re-use the aligned simulation results from TR 38.753, we propose to reuse the configuration [2] from there:

Re-use the TR 38.753 2CW configuration to define SU 8T8R 2CW/8-layer requirements. I.e., reuse configuration from [R4-2508622], including rCDL-C1 with 10 Hz Doppler, and use random wideband type I precoding.

For PDSCH, no modification of rCDL-C1 is necessary. PMI preferences are not a concern for PDSCH with random precoding and a single set of requirements is traditionally used for all bands. Update to match recent RAN1 angular changes for Uma are straightforward given the table derivation framework in TR 38.753.

For PDSCH, no modification of the rCDL channel model framework is necessary. Updating the parameter table for the “C1” variant, following recent RAN1 angular changes for Uma, is straightforward from the table derivation framework in TR 38.753.

Requirement definition

As found in the study, the SNR operating point for a fixed relative throughput for each CW varies between each CW. Looking at a per CW TPUT curve from the study, this is visible as follows:

Figure 1: TPUT curves for SU 8T8R 8layers (2CWs) at low speed.

Hence, when using the customary “SNR@70% relTPUT” metric with a fixed MCS for both CWs, we end up with two SNR operating points. This may be captured as follows

Table 1: Proposal for 2CW demodulation requirements

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | CW1 SNR (dB) | CW2 SNR (dB) |
| 1-x | R.PDSCH.XXX | 40 / 30 | 64QAM, 0.50 | FR1.30-XX | rCDL-C1-10 | 70 | XXX | XXX |

To summarize in proposal form:

Adopt the customary “SNR@70% relTPUT” metric with fixed MCS for both CWs, and extending the metric application to two codewords, by allowing two separate SNR operating points, i.e., one for each CW.

Optional requirements

Following the WID [1], the PDSCH requirements are to be optional. To achieve this, we propose to use optional UE capability and applicability rules. For example:

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SCM compatible MIMO receiver | FR1 TDD | PDSCH | Clause X.X.X Test X-X |  |

This is similar to the current requirement status of the “Simplified SU-MIMO 8Rx receiver” UE capability and applicability rule. Or in proposal form:

RAN4 to make SU PDSCH requirement optional, by following the simplified SU-MIMO 8Rx example, i.e., via optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules.

Specification of PMI reporting requirements

Alignment reached

The RAN#109 WID [1] is also quite clear in defining PMI reporting requirements when alignment is reached. The Rel-19 study is quite clear in that eType II alignment is already reached. This can be directly seen from the report TR 38.753 in section 7.1, as well as the full alignment collection results, and the alignment collection subset (both in [3]).

Observation for rCDL-C1: - In eType-II codebook case, three clusters of results can be observed: - Cluster 1: include source #3, #7 are in 70% normalized throughput and source #7 in 90% normalized throughput. - Cluster 2: include source #4, #5, #6, #9 in 70% normalized throughput and source #3, #4, #5, #6, #9 in 90% normalized throughput. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput. - Cluster 3: include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput.

Figure 2: TR 38.753 observations for rCDL-C1 eType-II follow PMI alignment.

Hence, we propose:

RAN4 to agree alignment of PMI performance results with 4T4R and 8T4R given eTypeII precoding. Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

RAN4 to define PMI reporting performance requirements with 4T4R and 8T4R given eTypeII follow precoding re-using the configuration from the Rel-19 study, informatively summarised in [R4-2508622].

Concerning whether “gamma” KPIs are used (as in legacy), or whether SNR@70%or90%TPUT is used (as done in the study), we have a slight preference for the absolute performance numbers used in the study.	
The background, why LTE used “the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reports compared to the case when the transmitter is using random precoding” at the 70% relative TPUT point obtained at SNR_follow, and why 5GNR changed the TPUT to 90%, is unclear at this point. Though this likely pertained to an effort to show >1 relative TPUT improvement with new code book based precoding schemes. As the previous Rel-19 study indicated, the legacy channel models were not adapted to let interference cancelation intended multi-beam and combination codebooks show their strengths. While these fundamental limitations of the legacy channels are addressed via SCMs, the enhanced 5GNR codebooks still have limited performance gains in the baseline SU MIMO use case with assumed limited numbers of antenna elements (i.e., “AAV3”/direct port mapping without array abstraction from TR 38.753).

As such we think it is not useful to chase gains figures via non-relevant KPIs. RAN4 should return to absolute performance KPIs in SCM for PMI reporting.

RAN4 to define PMI reporting performance requirements with the metric of SNR@70% follow PMI throughput.

Direct reuse of rCDL

The WID [1] is requesting to consider potential modifications of the rCDL model and requests to update the rCDL-C model parameter table, such as to take recent Rel-19 RAN1 modifications of the UMa use case into account. The latter is easy to achieve using the table derivation framework in TR 38.753, the former is unneeded, as we will discuss in the following.

Thus, we propose already at the beginning of this section:

For PMI reporting, no modification of the rCDL channel model is necessary. Update of parameter for “C1” variant due to recent RAN1 angular spread changes for the UMa scenario is straightforward by following the table derivation framework in TR 38.753.

PMI preference in SCMs

It is the nature of a spatial channel model (SCM), that precoders and receivers that match the spatial preferences of the channel, result in better throughput/BLER/spectral efficiency performance than those who do not. Capturing the performance impact of the receiver/precoder algorithms adapting themselves to the channel, is one of the major advantages of SCMs over legacy channel models, as discussed in FS_NR_demod_SCM and captured in TR 38.753.

It is the nature of a spatial channel model (SCM), that precoders and receivers that match the spatial preferences of the channel perform better than those who do not.

Certain AIML use cases may have a need for random, but partially (and controllably) predictable, time-wise evolution of the spatial characteristics of the used SCM, in order to appropriately test features that involve, e.g., PMI prediction performance. 	
Such time-wise spatial properties evolution may be included in rCDL via application of the legacy beamsteering framework, or via mu_desired table evolution, or via direct DoA/DoD angle evolution. However, such extensions shall be handled by the features that use the channel model for AIML prediction use cases, and not by this demod enhancement work item. Each WI is responsible for the advancement of the requirement framework as is needed by the WI.

If random, but partially predictable, time-wise evolution of the spatial characteristics of a SCM is needed, this can be achieved via, .e.g., the direct application of the legacy beamsteering framework, or via mu_desired CDL table evolution, or via direct DoA/DoD angle evolution.

The legacy beamsteering framework (e.g., LTE TS36.101, Appendix B.2.3A.4) is arguably the least effort approach to include time-wise spatial properties evolution, as it simply rotates the effective precoder selected by the PMI feedback. This low effort approach is sufficient, if PMI selection distributions are to be evolving over time, i.e., if RAN4 intends to consider adversarial DUTs in conformance testing. 	
This rotation/“bending” of the selected PMI to precoder mapping is done in RE-domain directly on top of the precoder formulation and thus independent of the chosen propagation environment. See Figure 3 for a graphical representation of this.

Figure 3: Function of legacy beamsteering approach to change PMI selection over time.

Or put as an observation:

The legacy beamsteering framework rotates the effective precoder selected by the PMI. This rotation of  the selected PMI to precoder mapping is done in RE-domain directly on top of the precoder formulation and thus independent of the chosen propagation environment. This low effort approach is sufficient, if PMI selection distributions are to be evolving over time.

However, the PMI reporting requirements from this SU SCM objective do not need to consider time-wise evolution of the spatial characteristics of a SCM, as this only matters if we consider dishonest devices. For follow PMI measures to be preferring certain PMIs in SCM is a feature of deployments, as well as SCMs, and do not impact minimum performance requirements targeting the PMI selection algorithm implementation performance of a commercial device.

PMI reporting requirements from this SU SCM objective do not need to consider time-wise evolution of the spatial characteristics of a SCM.

Frequency dependency of angular spreads

Traditionally a single set of requirements is used for all bands in TS 38.101-4. We do not see a reason to deviate from this in Rel-20 SCM requirements. The implementation of the receiver or CSI selection algorithm does not change on a per band basis, so the same requirements are applicable in a test environment, as long as the test environment approximates a single band that the algorithms are applied to.

No need to deviate from the 5GNR approach of using single set of requirements for all bands per frequency range. The implementation of the receiver or CSI selection algorithm does not change on a per band basis.

We also note that the correlation matrices used in legacy TDL and derived under lambda/2 assumptions, impose certain fixed angular spreads of the directive broadside channel formed with such a correlation. Even using low correlation, imposes a “360 degrees assumption” on all bands it is used in.	
If this frequency independence was not an issue for legacy systems, we don’t see a reason why this would change for Rel-20.

Conclusion

The following observations and proposals were made:

=== PMI

== PDSCH / Requirement configuration

- Re-use the TR 38.753 2CW configuration to define SU 8T8R 2CW/8-layer requirements. I.e., reuse configuration from [R4-2508622], including rCDL-C1 with 10 Hz Doppler, and use random wideband type I precoding.

For PDSCH, no modification of the rCDL channel model framework is necessary. Updating the parameter table for the “C1” variant, following recent RAN1 angular changes for Uma, is straightforward from the table derivation framework in TR 38.753.

== PDSCH / Requirement definition

Adopt the customary “SNR@70% relTPUT” metric with fixed MCS for both CWs, and extending the metric application to two codewords, by allowing two separate SNR operating points, i.e., one for each CW.

== PDSCH / Optional requirements

RAN4 to make SU PDSCH requirement optional, by following the simplified SU-MIMO 8Rx example, i.e., via optional UE capability “SCM compatible MIMO receiver” and corresponding applicability rules.

== PMI / Alignment reached

RAN4 to agree alignment of PMI performance results with 4T4R and 8T4R given eTypeII precoding. Outlier removal to be applied to form requirements, if outliers are not aligned in second simulation results delivery round.

RAN4 to define PMI reporting performance requirements with 4T4R and 8T4R given eTypeII follow precoding re-using the configuration from the Rel-19 study, informatively summarised in [R4-2508622].

RAN4 to define PMI reporting performance requirements with the metric of SNR@70% follow PMI throughput.

=== PMI

== PMI / Direct reuse of rCDL

For PMI reporting, no modification of the rCDL channel model is necessary. Update of parameter for “C1” variant due to recent RAN1 angular spread changes for the UMa scenario is straightforward by following the table derivation framework in TR 38.753.

== PMI / Direct reuse of rCDL / PMI preference in SCMs

- It is the nature of a spatial channel model (SCM), that precoders and receivers that match the spatial preferences of the channel perform better than those who do not.

If random, but partially predictable, time-wise evolution of the spatial characteristics of a SCM is needed, this can be achieved via, .e.g., the direct application of the legacy beamsteering framework, or via mu_desired CDL table evolution, or via direct DoA/DoD angle evolution.

The legacy beamsteering framework rotates the effective precoder selected by the PMI. This rotation of  the selected PMI to precoder mapping is done in RE-domain directly on top of the precoder formulation and thus independent of the chosen propagation environment. This low effort approach is sufficient, if PMI selection distributions are to be evolving over time.

PMI reporting requirements from this SU SCM objective do not need to consider time-wise evolution of the spatial characteristics of a SCM.

== PMI / Direct reuse of rCDL / Frequency dependency of angular spreads

No need to deviate from the 5GNR approach of using single set of requirements for all bands per frequency range. The implementation of the receiver or CSI selection algorithm does not change on a per band basis.

- References

- RP-252957, WID: NR demodulation performance: Phase 6, RAN#109, September 2025.

- R4-2508622, Simulation assumptions for SCM, Huawei, HiSilicon, MediaTek, RAN4#115.

- R4-2509413, Results collection table for NR SCM, BT plc, RAN4#116.