3GPP TSG-RAN4 Meeting #116bis	R4-2513833

Prague, Czech Republic, 13 October – 17 October 2025

Source:			Ericsson

Title:			6 MHz channel bandwidth – BS RF impacts

Agenda item:		7.1.5.1

Document for:		Approval

# Introduction

In last RAN#109, a new WI ([1]) has been approved to enhance UE RF requirements for the deployment in NR FR1. One of the objectives of this WI is to specify 6 MHz channel bandwidth for NR bands n5, n8, n12, n85 and n29.

In this contribution, we look at the impacts to TS 38.104 when introducing 6 MHz channel bandwidth.

# Discussion

## Background

The new WI ([1]) approved in last RAN#109 meeting, embraces several enhancements which are UE related, except the following one which would impact both UE and BS RF, adding 6 MHz channel BW support in RAN4 specifications.

## Open issues

### Support of 6 MHz channel bandwidth – scope

The scope of the new WI ([1]) mentions explicitly that BS conducted and OTA requirements shall be specified. Still, it doesn’t mention if:

- NB-IoT should be supported in 6 MHz channel BW.

- IAB and NCR shall support or not 6 MHz channel BW.

As the WI often refers to the work done for the introduction 7 MHz channel bandwidth, to be consistent with 7 MHz, we would make the following proposals and observation:

Proposal 1: NB-IoT operating in NR in-band is also supported within 6 MHz channel bandwidth and clarify in the NCR specifications that 6 MHz channel bandwidth is not supported.

Observation 1: The IAB specifications already clarify that channel bandwidth less than 10 MHz are not supported.

## Specification impact overview

Focusing on the BS RF parameters, the following Table 2 gives an overview of the impacts in the NR specifications (TS 38.104) when introducing 6 MHz channel bandwidth.

Table 1: BS impacts overview (conducted requirements)

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 6. Conducted transmitter requirements |  |  |
| 6.1 General | No |  |
| 6.2 Base station output power | No |  |
| 6.3 Output power dynamics | No |  |
| 6.4 Transmit ON/OFF | No |  |
| 6.5 Transmitted signal quality | No |  |
| 6.6 Unwanted emissions | No |  |
| 6.6.1 General | No |  |
| 6.6.2 Occupied bandwidth | No |  |
| 6.6.3 ACLR | Yes | 6 MHz shall be added to Table 6.6.3.2-1, Table 6.6.3.2-2a and Table 6.6.3.2-3 |
| 6.6.4 Operating Band Unwanted Emissions | No |  |
| 6.6.5 Transmitter spurious emission | No |  |
| 6.7 Transmitter intermodulation | No |  |
| 7. Conducted receiver characteristics |  |  |
| 7.1 General | No |  |
| 7.2 Receiver sensitivity level | Yes | 6 MHz shall be added to Table 7.2.2-1, Table 7.2.2-2 and Table 7.2.2-3 |
| 7.3 Dynamic range | Yes | New row(s) with 6 MHz requirements shall be added in Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3 And Table 7.3.2-1a, Table 7.3.2-2aand Table 7.3.2-3a if NB-IoT should be supported. |
| 7.4 In-band selectivity and blocking | Yes | 6 MHz shall be added to Table 7.4.1.2-1, Table 7.4.1.2-2, Table 7.4.2.2-1, Table 7.4.2.2-2 and Table 7.4.2.2-2 NB interferer offset for 6 MHz shall be added to Table 7.4.2.2-3 |
| 7.5 Out of band blocking | No |  |
| 7.6 Receiver spurious emissions | No |  |
| 7.7 Receiver intermodulation | Yes | 6 MHz interferer and NB interferer for 6 MHz shall be added to Table 7.7.2-2 and Table 7.7.2-4 |
| 7.8 In channel selectivity | Yes | 6 MHz (15kHz SCS only) shall be added to Table 7.8.2-1, Table 7.8.2-2 and Table 7.8.2-3 And Table 7.8.2-1a, Table 7.8.2-2a and Table 7.8.2-3a if NB-IoT should be supported. |

Refer to our other contribution for an overview of the systems impacts and the UE specification impacts ([2]).

Proposal 2: Agree with the system impacts listed in Table 1.

## Further detailed analysis

### FRC

RAN4 agreed to use 5 MHz FRCs when specifying 7 MHz channel bandwidth requirements.

Reusing the same approach and same rationale, considering the limited impact on testing effort, we also propose to reuse 5 MHz FRCs when specifying 6 MHz channel bandwidth requirements.

Proposal 3: Do not specify 6 MHz FRC and reuse the existing 5 MHz FRCs to test Rx requirements for 6 MHz channel bandwidth.

### Assumption for Tx and Rx requirements

In the following subsections, we are assuming a spectrum utilization of 31 RBs (as proposed in [2]) to determine the Tx and Rx limits for 6 MHz channel bandwidth.

### Tx requirements

#### Total power dynamic range

Following the requirement calculation for the other NR channel bandwidths and SCS, which itself follows the requirement calculation for E-UTRA, the total power dynamic range for 6 MHz channel bandwidth and 15kHz SCS shall be 14.7 dB.

| BS channel | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
| bandwidth (MHz) | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 6 | 14.7 | N/A | N/A |

Proposal 4: Specify the total power dynamic range for 6 MHz channel bandwidth and 15kHz SCS with a value of 14.7 dB.

### Rx requirements

In the following, it’s assumed that the 5 MHz channel BW FRC will be reused to specify 6 MHz channel BW Rx requirements. The following proposals are based on TR 38.817-02.

#### REFSENS

Proposal 5: Specify REFSENS for 6 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 6 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

#### Dynamic range

The interferer signal mean power for 6MHz channel BW / 31 RBs shall be -81.6 dBm for WA BS, -76.6 dBm for MR BS and -73.6 dBm for LA BS.

Proposal 6: Specify Dynamic Range for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 6 | 15 | G-FR1-A2-1 | -70.7 | -65.7 | -62.7 | -81.6 | -76.6 | -73.6 |

#### ACS

Proposal 7: Specify the ACS interferer frequency offset with ±2.5125 MHz value.

#### Narrow band blocking

Proposal 8: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±360(+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

#### Intermodulation

Proposal 9: Specify the interfering signals for the intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 6 | ±7.52 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

#### Narrowband Intermodulation

Proposal 10: Specify the narrowband intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±630 | CW |
|  | ±1560 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

#### In-channel selectivity

Instead of specifying a new FRC to specify ICS for 6 MHz channel bandwidth, we would propose to specify it based on the requirement for 5 MHz channel bandwidth, similar to what is done for 10, 15, 20, 25, 30 and 35 MHz channel bandwidth, reusing G-FR1-A1-7 FRC and a 10RBs interferer.

Table 7.8.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5, 6 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 7 | 15 | G-FR1-A1-7 | -100.6-ΔminSENS | -79.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |

Proposal 11: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW.

### Annex - EVM window

Proposal 12: Specify the EVM window for 6 MHz channel BW according to the following table:

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 6 | 512 | 36 | 14 | 40 |

# Conclusion

In this contribution, we gave an overview of the BS RF impacts when introducing 6 MHz channel bandwidth. We made the following observations and proposals:

Proposal 1: NB-IoT operating in NR in-band is also supported within 6 MHz channel bandwidth and clarify in the NCR specifications that 6 MHz channel bandwidth is not supported.

Observation 1: The IAB specifications already clarify that channel bandwidth less than 10 MHz are not supported.

Proposal 2: Agree with the system impacts listed in Table 1.

Proposal 3: Do not specify 6 MHz FRC and reuse the existing 5 MHz FRCs to test Rx requirements for 6 MHz channel bandwidth.

Proposal 4: Specify the total power dynamic range for 6 MHz channel bandwidth and 15kHz SCS with a value of 14.7 dB.

Proposal 5: Specify REFSENS for 6 MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA |
| 6 | 15 | G-FR1-A1-1 (Note 1) | -101.7 | -96.7 | -93.7 |

Proposal 6: Specify Dynamic Range for 7MHz channel BW according to the following table:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  |  | Interfering signal mean power (dBm) / BWConfig |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | WA | MR | LA | WA | MR | LA |
| 6 | 15 | G-FR1-A2-1 | -70.7 | -65.7 | -62.7 | -81.6 | -76.6 | -73.6 |

Proposal 7: Specify the ACS interferer frequency offset with ±2.5125 MHz value.

Proposal 8: Specify the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap with value: ±360(+m*180) kHz, with m=0, 1, 2, 3, 4, 9, 14, 19, 24.

Proposal 9: Specify the interfering signals for the intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge (MHz) | Type of interfering signal (Note 3) |
| --- | --- | --- |
| 6 | ±7.52 | CW |
|  | ±17.5 | 5 MHz DFT-s-OFDM NR signal (Note 1) |

Proposal 10: Specify the narrowband intermodulation requirement according to the following table:

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (kHz) (Note 3) | Type of interfering signal |
| --- | --- | --- |
| 6 | ±630 | CW |
|  | ±1560 | 5 MHz DFT-s-OFDM NR signal, 1 RB (Note 1) |

Proposal 11: Specify the In-channel selectivity requirements for 7 MHz channel BW based on the requirements for 5 MHz channel BW.

Proposal 12: Specify the EVM window for 6 MHz channel BW according to the following table:

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 6 | 512 | 36 | 14 | 40 |

# References

- RP-252901, UE RF enhancements for NR FR1, Phase 5,

- R4-2513834, 6 MHz channel bandwidth – system parameters and UE RF impacts, Ericsson