3GPP TSG-RAN4 Meeting #116bis	R4-2513835

Prague, Czech Republic, 13 October – 17 October 2025

Source:			Ericsson

Title:			US 4.9 GHz band – system parameters

Agenda item:		5.11.2

Document for:		Approval

# Introduction

In RAN#108, a WI ([1]) has been approved to introduce a new 4.9GHz band for US.

In last RAN4#116 meeting, as captured in Chair notes ([3]), RAN4 agreed to conclude the study phase, meaning also that the specification work could start from this meeting. This contribution is discussing the UE RF requirements, analyzing the relevant regulation.

# Discussion

## Background

The Commission allocated the 4.9 GHz band for public safety use in 2002. Since then, the rules have evolved in response to changing conditions, including band use and technological innovation. In 2020, to stabilize the band, the Public Safety and Homeland Security Bureau (PSHSB) and the Wireless Telecommunications Bureau (WTB) announced a freeze on applications in this band. In January 2023, the Seventh Report and Order and Ninth Further Notice established a comprehensive and coordinated nationwide approach to managing the 4.9 GHz band, adding the role of a nationwide Band Manager. In the Eight Report and Order ([2]), FCC enforces the role of the Band Manager, further describing his role and responsibilities.

It should also be noted that the existing NR band n79 (4.4-5.0 GHz) encompasses this new 4.9 GHz band.

## FCC part 90 and 3GPP impacts

### Overview

The new 4.9 GHz US band is a public safety band and is specified in FCC §90. The following sub-parts are notably key inputs for the specification of this new band in 3GPP:

- §90.1213: band plan.

- §90.1215: Power limits.

- §90.210: Emission masks.

### Band plan (§90.1213)

The following band plan has been defined but as stated in §90.1213(b), channels can be aggregated up to 50 MHz for wider operation, which justifies the requested channel BWs listed in the WI ([1]).

Observation: The targeted 3GPP channel bandwidths for the US 4.9GHz band are compatible with FCC band plan.

## System parameters

Based on the WI, the follow system parameters should be specified when introducing the new US 4.9GHz band.

As the latest NR band is n110 and the latest LTE band is 113, we suggest using band number n114 for the new US 4.9 GHz band.

Proposal: Specify the following system parameters for the new US 4.9 GHz band.

- Operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n114 | 4940 – 4990 | 4940 – 4990 | TDD |  |

- BS channel bandwidth per operating band

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| n114 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

- Channel raster

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

- Sync raster

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 30 kHz | Case C | 8850 – <1> – 8877 |

# Conclusion

In this contribution, we studied the FCC regulation for the new US 4.9 GHz band and made the following proposal for the system parameters:

Proposal: Specify the following system parameters for the new US 4.9 GHz band.

- Operating band

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high (MHz) | Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high (MHz) | Duplex mode | Notes |
| --- | --- | --- | --- | --- |
| n114 | 4940 – 4990 | 4940 – 4990 | TDD |  |

- BS channel bandwidth per operating band

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
| n114 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

- Channel raster

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

- Sync raster

| NR operating band | SS Block SCS | SS Block pattern (NOTE 1) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 30 kHz | Case C | 8850 – <1> – 8877 |

# References

- RP-251887, New WID on Introduction of NR TDD 4.9GHz Band for US Operation, Fujitsu

- Eight Report and Order, WP Docket No. 07-100, 22 October 2024.

- RAN4 Chair notes: https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_116/Inbox/Chair_Notes/Main_Session_Notes/1.Mon/7.Draft_R4-2509228_%5B116%5D%5B115%5D_US4.9GHz_Topic_Summary_moderator.docx