3GPP TSG-RAN WG4 Meeting #116bis	R4-2513869

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	6.1.2

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 6Rx

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The WID for performance part has been updated as below [2] according latest RAN4#114 meeting agreements [3].

The objectives of performance part for UE RF enhancements for NR FR1 and FR2 Phase 4 include: 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - 6 MIMO layers performance requirements will be only defined for FWA. - 4 MIMO layers performance requirements will be defined and will apply to FWA and handheld UE devices. - Specify the SDR requirements to support 6Rx General for all the areas - Specify release independence requirements for performance part in TS 38.307/36.307, if needed and feasible.

In RAN4#114bis, RAN4#115 and RAN4#116 meetings, work plan, general scopes and some test assumptions were agreed [4][5][6]. In this contribution, we provide our further analysis on the open issues of 6Rx PDSCH demodulation performance and CSI requirements, and 6Rx SDR requirements.

- Discussion

  - General

In last meeting, three open issues for general part as below [6].

Issue 1-1: Test applicability rule for 6Rx tests - Proposals - Option 1: Introduce the test applicability rules by following core part agreement. - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices. - Option 1a: Introduce UE declaration for the proper test applicability design if no explicit capability signalling finally confirmed by RAN2 - Option 2: Postpone the discussion until RAN2 concludes the UE capability signalling discussion Issue 1-2: Test applicability rule for different number Rx antenna ports - Proposals - Option 1: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage as following. Supported RX antenna ports; Test type; Test list. UE supports 6Rx, 4Rx and 2Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.X (Note 2); If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. or; PDCCH; All tests in Clause 5.3.3 (Note 2,3). UE supports both 6Rx and 4Rx; PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 6Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.X (Note 2). PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2. UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports both 8Rx, 6Rx, and 4Rx; PDSCH; All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6. PDCCH; All tests in Clause 5.3.3 (Note 2,3). PBCH; All tests in Clause 5.4.3 (Note 1). UE supports both 8Rx, 6Rx and 2Rx; PDSCH; All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2); If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4. PDCCH; All tests in Clause 5.3.2 (Note 2,4). PBCH; All tests in Clause 5.4.2. UE supports only 8Rx Or UE supports both 8Rx and 6Rx; PDSCH; All tests in Clause 5.2.4 (Note 2). PDCCH; N/A. PBCH; N/A. UE supports only 6Rx; PDSCH; All tests in Clause 5.2.X (Note 2). PDCCH; N/A. PBCH; N/A. Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx or 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx or 6 Rx with data source from system simulator, and the other 4 Rx or 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx or 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx or 6 Rx with data source from system simulator, and the other 6 Rx or 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1.. - Option 2: Postpone the discussion of test applicability rule until the relevant test cases are better defined Issue 1-3: Specification structure - Proposals - Option 1: Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification - Section 5.2.3 4Rx requirements - Section 5.2.4 6Rx requirements - Section 5.2.5 8Rx requirements

Test applicability rule for 6Rx tests

About the UE capability, RAN#109 formally approved RAN2 CR [7][8] to introduce maxNumberMIMO-LayersPDSCH-v1900 for FWA only as below. It is obviously the applicability for 6Rx performance requirements with 6 MIMO layers could use the new capability to describe the test applicability. But for 6Rx performance requirements with 4 MIMO layers, considering FWA and handheld UE are based on the same capability parameter maxNumberMIMO-LayersPDSCH, the applicability for Handheld and FWA should be based on declaration if separate demodulation requirements are introduced.

maxNumberMIMO-LayersPDSCH, maxNumberMIMO-LayersPDSCH-v1900 Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2 (except for FR2-NTN). If supportOf2RxXR is indicated, for single CC standalone NR, it is mandatory with capability signalling to support 2 MIMO layers in the bands specified in Table 7.3.2-2b in TS 38.101-1 [2]. If absent, the UE does not support MIMO on this carrier. The maxNumberMIMO-LayersPDSCH-v1900 is only included for FWA UE as defined in TS 38.101-1 [2]. The UE supporting maxNumberMIMO-LayersPDSCH-v1900 shall also indicate support of maxNumberMIMO-LayersPDSCH for the corresponding component carrier. When maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 are reported together for a component carrier, the network which is able to decode maxNumberMIMO-LayersPDSCH-v1900 ignores maxNumberMIMO-LayersPDSCH. For the bands where pdsch-1024QAM-2MIMO-FR1-r17 is indicated, MIMO layers for 1024 QAM is the smaller value between 2 and maxNumberMIMO-LayersPDSCH.

Proposal 1: For test applicability rule of 6Rx test,

- The applicability for 6Rx performance requirements with 6 MIMO layers could use the new capability to describe the test applicability.

- The applicability for 6Rx performance requirements with 4 MIMO layers should be based on declaration if separate demodulation requirements are introduced for Handheld and FWA devices.

Test applicability rule for different number Rx antenna ports

For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage.

Proposal 2: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage as following.

| Supported RX antenna ports | Test type | Test list |  |
| --- | --- | --- | --- |
| UE supports 6Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.X (Note 2) | If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports both 6Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports both 8Rx, 6Rx, and 4Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
|  | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 8Rx, 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx Or UE supports both 8Rx and 6Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| UE supports only 6Rx | PDSCH | All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx or 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx or 6 Rx with data source from system simulator, and the other 4 Rx or 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx or 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx or 6 Rx with data source from system simulator, and the other 6 Rx or 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. |  |  |  |

Specification structure

It’s no doubt a new sub-clause for 6Rx should be introduced for 6Rx performance requirements. About the position of this new sub-clause, it would be perfect if it could be between 4Rx and 8Rx in the new Rel-19 specification.

Proposal 3: Support introducing a new sub-clause for 6Rx and support putting it between 4Rx and 8Rx in the new Rel-19 specification if possible.

  - PDSCH demodulation performance requirements

The agreement for introducing 6Rx PDSCH demodulation performance requirements for FR1 TDD single carrier has been achieved. Therefore, related test assumptions and parameters should be discussed and aligned.

    - Performance requirements for 4 MIMO Layers

Test cases and MIMO correlation for 4 MIMO layers for Handheld UE testing

Currently the number of test cases and MIMO correlation for 4 MIMO layers test are still open, the proposals are as below.

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Proposals - Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: α = 0 and β = 0.07179 - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

Considering the correlation matrixes of FWA and Handheld UE are usually different, i.e., certain correlation for handheld UE while low correlation for FWA UE, we think the best solution is defining separate test cases for different UE types. The correlation factors α = 0 and β = 0.07179 for handheld UE, while α = 0 and β = 0 for FWA UE are suggested. If only one test case will be defined for 4 MIMO layers for both FWA and Handheld UE, we prefer to use ULA Low.

Proposal 4: Prefer to define two separate test cases for 4 MIMO layers for FWA and Handheld UE.

Proposal 5: If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low (α = 0 and β = 0) for FWA and ULA Medium (α = 0 and β = 0.07179) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

    - Performance requirements for 6 MIMO Layers for FWA

Test applicability rule for FWA

In last RAN4#116 meeting, one proposal for FWA test applicability rule raised as “FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests”, the purpose of this proposal is to reduce the testing burden. However, for the same number of Rx antennas, the cases for different MIMO layers have different test requirements and different test assumptions, it’s hard to get the conclusion that the lower layer case could pass once the higher layer case passed. At the same time, the test burden problem could be reduced by the applicability rules for different Rx antennas.

Proposal 6: For the test applicability rule for FWA, do not prefer to introduce a new rule as “FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests”.

DMRS configuration for 6 MIMO layers case

Per our understanding and the description of DMRS configurations described in 38.211 and 38.331, DMRS type 1 with one additional OFDM symbol for DMRS ports and maxLength=1 is the typical DMRS configuration (Rel-15 1+1 DMRS symbols) for 4 MIMO layers as Figure 2-1 (a), it is straightforward to configure maxLength=2 in order to support 6 MIMO layers as Figure 2-1 (b), this is the typical configuration for real network.  For Rel-18 1+1(6L), it only could be supported when the feature of Rel-18 enhanced DMRS is supported.

38.212, DMRS ports configurations

- Rel-15 typical 4 ports configuration

- Rel-15 typical 6 ports configuration

- Rel-18 6 ports configuration

Figure 2-1. DMRS ports configurations for Rel-15 DMRS and Rel-18 DMRS

Proposal 7: For DMRS configuration for 6 MIMO layers case, use the typical configuration Rel-15 Type 1 DMRS 2+2.

Proposal 8: For DMRS ports for 6 MIMO layers case, propose to use DRMS ports {1000-1003, 1004, 1006} for test assumption.

  - SDR requirements

SDR requirements for 1024QAM with 4 MIMO layers

For the SDR requirements for 1024QAM with 4 MIMO layers, one proposal is to consider MCS23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers.

Issue 3-1-2: SDR requirements for 1024QAM with 4 layers - Proposals - Option 1: Not Define SDR requirements for 1024QAM with 4 layers - Option 2: Consider MCS23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers

However, for 1024QAM, current spec only defining SDR requirements up to 2 MIMO layers even for 8Rx since some contributors conclude 1024QAM is not feasible for 4 MIMO layers with 8Rx, hence it may cause problems with specification consistency if we introduce SDR requirements for 1024QAM with 4 MIMO layers for 6Rx but not introduced for 8Rx. Therefore, we prefer not to define of 1024QAM with 4 MIMO layers for 6Rx.

Proposal 9: For the SDR requirements of 6Rx with 1024QAM, prefer not to define SDR requirements for 1024QAM with 4 MIMO layers.

If RAN4 finally decide to introduce SDR requirements for 1024QAM with 4 MIMO layers, we recommend to consider MCS23 in Table 4 according our simulation results [9]. Furthermore, related applicability rules are needed, and it’s a good choice to define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules.

Proposal 10: If RAN4 finally decides to define SDR requirements for 1024QAM with 4 MIMO layers, prefer to consider MCS23 according our simulation results.

Proposal 11: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced.

SDR requirements

According the simulation results in our companion contribution [9], MCS28 could be achieved for 64QAM with 6 MIMO layers and scaling factor 1; MCS27 could be achieved for 256QAM with 6 MIMO layers and scaling factor 1; MCS24 could be achieved for 1024QAM with 2 MIMO layers and scaling factor 1. The MCS values for SDR requirements are proposed as below.

Proposal 12: For QPSK/16QAM/64QAM/256QAM, define SDR requirements for 6 MIMO layers as below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Proposal 13: For 1024QAM, define SDR requirements for 2 MIMO layers as below.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

  - CSI reporting performance requirements

CQI reporting requirements

For the CQI estimation processing, there is no UE handling difference for 6Rx and 4Rx/8Rx, hence we don’t think new CQI reporting requirements are necessary for 6Rx. In current spec, CQI requirements with rank 4 under static channel are defined for 8Rx, from this point of view, it is also acceptable to define CQI requirements with rank 4 under static channel for 6Rx. No need to define wideband CQI reporting requirements with inter-cell interference under fading conditions at all.

Proposal 14: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx.

RI reporting requirements

Similar reasons as above for CQI and PMI reporting requirement, we don’t prefer to define new test requirements for RI reporting for 6Rx.

Proposal 15: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

- Conclusion

In this contribution, we provide our further analysis and views on the PDSCH demodulation performance, SDR requirements and CSI reporting performance requirements on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

For General

Proposal 1: For test applicability rule of 6Rx test,

- The applicability for 6Rx performance requirements with 6 MIMO layers could use the new capability to describe the test applicability.

- The applicability for 6Rx performance requirements with 4 MIMO layers should be based on declaration if separate demodulation requirements are introduced for Handheld and FWA devices.

Proposal 2: For UE supports 6Rx and at least one of other number of antennas, i.e., 8Rx, 4Rx, 2Rx, corresponding applicability rules should be introduced to skip some test cases with less UE antennas once the test cases with more UE antennas could pass, in order to balance the number of test cases and coverage as following.

| Supported RX antenna ports | Test type | Test list |  |
| --- | --- | --- | --- |
| UE supports 6Rx, 4Rx and 2Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.X (Note 2) | If UE passes tests in Clause 5.2.X, UE can skip Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
| or | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
| UE supports both 6Rx and 4Rx | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports 8Rx, 6Rx, 4Rx and 2Rx or UE supports both 8Rx, 6Rx, and 4Rx | PDSCH | All tests in Clause 5.2.3 (Note 2, 3) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.3 Table 5.2.3.1.1-4, Table 5.2.3.2.1-4 and Test 4-1 in Clause 5.2.3 Table 5.2.3.1.1-6, Table 5.2.3.2.1-6 |
|  | PDCCH | All tests in Clause 5.3.3 (Note 2,3) |  |
|  | PBCH | All tests in Clause 5.4.3 (Note 1) |  |
| UE supports both 8Rx, 6Rx and 2Rx | PDSCH | All tests in Clause 5.2.2 (Note 2, 4) All tests in Clause 5.2.4 (Note 2) | If UE passes tests in Clause 5.2.4, UE can skip Test 2-1 and Test 2-2 in Clause 5.2.2 Table 5.2.2.1.1-4, Table 5.2.2.2.1-4 |
|  | PDCCH | All tests in Clause 5.3.2 (Note 2,4) |  |
|  | PBCH | All tests in Clause 5.4.2 |  |
| UE supports only 8Rx Or UE supports both 8Rx and 6Rx | PDSCH | All tests in Clause 5.2.4 (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| UE supports only 6Rx | PDSCH | All tests in Clause 5.2.X (Note 2) |  |
|  | PDCCH | N/A |  |
|  | PBCH | N/A |  |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration Note 2: ‘maxMIMO-Layers-r16’ is not configured during the performance requirements testing for UE supporting Release 16 per-BWP MIMO layer adaptation. Note 3: 8Rx or 6Rx capable UEs are tested on any of the 4Rx supported RF bands by connecting 4 out of 8 Rx or 6 Rx with data source from system simulator, and the other 4 Rx or 2 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 4Rx should be applied. Note 4: 8Rx or 6Rx capable UEs are tested on any of the 2Rx supported RF bands by connecting 2 out of 8 Rx or 6 Rx with data source from system simulator, and the other 6 Rx or 4 Rx are connected with zero input, depending on UE’s declaration and AP configuration. Requirements specified with 2Rx should be applied. Note 5: Tests defined for 8Rx capable UEs according to whether the UE is a ‘simplified 8Rx receiver’ or ‘baseline 8Rx receiver’ are specified in Table 5.1.1.3-1. |  |  |  |

Proposal 3: Support introducing a new sub-clause for 6Rx and support putting it between 4Rx and 8Rx in the new Rel-19 specification if possible.

For PDSCH demodulation performance requirements

Proposal 4: Prefer to define two separate test cases for 4 MIMO layers for FWA and Handheld UE.

Proposal 5: If separate test cases for 4 MIMO layers for FWA and Handheld UE will be defined, we prefer to use ULA Low (α = 0 and β = 0) for FWA and ULA Medium (α = 0 and β = 0.07179) for Handheld UE. If only one test case will be defined for 4 MIMO layers for FWA and Handheld UE, we prefer to use ULA Low.

Proposal 6: For the test applicability rule for FWA, do not prefer to introduce a new rule as “FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests”.

Proposal 7: For DMRS configuration for 6 MIMO layers case, use the typical configuration Rel-15 Type 1 DMRS 2+2.

Proposal 8: For DMRS ports for 6 MIMO layers case, propose to use DRMS ports {1000-1003, 1004, 1006} for test assumption.

For SDR requirements

Proposal 9: For the SDR requirements of 6Rx with 1024QAM, prefer not to define SDR requirements for 1024QAM with 4 MIMO layers.

Proposal 10: If RAN4 finally decides to define SDR requirements for 1024QAM with 4 MIMO layers, prefer to consider MCS23 according our simulation results.

Proposal 11: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced.

Proposal 12: For QPSK/16QAM/64QAM/256QAM, define SDR requirements for 6 MIMO layers as below.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12] |  |  |  |

Proposal 13: For 1024QAM, define SDR requirements for 2 MIMO layers as below.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

For CSI reporting requirements

Proposal 14: For CQI reporting requirements, prefer not to define new CQI reporting requirements for 6Rx.

Proposal 15: For RI reporting requirements, prefer not to define new RI reporting requirements for 6Rx.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-250780, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#107 meeting, Huawei

[3] R4-2503014, WF on 6Rx UE requirements, RAN4#114 meeting, AT&T

[4] R4-2504750, Way Forward for [114bis][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#114bis meeting, Huawei, HiSilicon

[5] R4-2508727, Way Forward for Topic [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#115 meeting, Huawei, HiSilicon

[6] R4-2512617, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, RAN4#116 meeting, Huawei, HiSilicon

[7] R2-2506466, Introduction of UE capability on 6 DL MIMO layers, RAN2#131 meeting, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple

[8] R2-2506467, Introduction of UE capability on 6 DL MIMO layers, RAN2#131 meeting, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple

[9] R4-2513870, Simulation results on UE demodulation performance requirements for 6Rx, RAN4#116bis meeting, Samsung