3GPP TSG-RAN WG4 Meeting #116bis	R4-2513872

Prague, CZ, Oct. 13 – 17, 2025

Agenda item:	6.12.3

Source:	Samsung

Title:	Initial Simulation Results on Rel-19 NR MIMO Phase 5 UE performance

Document for:	Information

- Introduction

In the RAN plenary RAN#102 meeting the WID [1] for Rel-19 NR MIMO Phase 5 has been approved and further revised in RAN#105 meeting [2]. The details about objective 2 for up to 128 CSI-RS ports as below.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

This is our third meeting on demodulation and CSI reporting performance requirements for Rel-19 NR MIMO Phase 5 WI. General impacts of each objective have been discussed in RAN4#115 and RAN4#116 meeting, agreements are captured in [3][4]. In RAN4#115 meeting, PMI reporting requirements for typeI-SinglePanel-r19 has been agreed [3]. In RAN4#116 meeting, no consensus on PMI reporting requirements for eTypeII-r19, but agreements on simulation assumptions achieved [4].

In this contribution, we provide our initial simulation results and analysis for both typeI-SinglePanel-r19 and eTypeII-r19.

- Simulation for PMI reporting performance

  - Simulation assumptions

According agreements in RAN4#115 and RAN4#116 meetings [3][4], the simulation assumptions for PMI reporting performance test could be summarized as Table 2-1.

Table 2-1. Simulation Assumptions for typeI-SinglePanel-r19 and eTypeII-r19 perforamnce test

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | FDD: 10 TDD: 40 |
| Subcarrier spacing |  | kHz | FDD: 15 TDD: 30 |
| Duplex Mode |  |  | FDD and TDD |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | XP-Medium 64 x 2 and 64x4 (N1,N2) = (8,4) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| NZP CSI-RS for CSI acquisition | Number of aggregated NZP CSI-RS resources (K) |  | 2 |
|  | portMappingMethod |  | Method 1 |
|  | Number of slots for CSI-RS transmission |  | 1 |
|  | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 64 |
|  | CDM Type |  | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS resource 1 (l0, l1) |  | Row 17,(3, 10) |
|  | First OFDM symbol in the PRB used for CSI-RS resource 2 (l0, l1) |  | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | FDD: 8 TDD: 16 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report interval and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | FDD: 7 TDD: 17 |
| CSI request |  |  | FDD: 1 in slots i, where mod(i, 5) = 1, TDD: 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 and eTypeII-r19 |
|  | Codebook Mode |  | Scheme A |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2) |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction |  | 00000010 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | FDD: 11 TDD: 11.5 |
| Maximum number of HARQ transmission |  |  | 4 |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: For FDD, If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). For TDD, If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-12), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |

  - Simulation results for FDD

Based on simulation assumptions listed in Table 2-1, we have following simulation results. The simulation results for both MCS20 (Table 1) and MCS22 (Table 1) are provided according the agreement in last RAN4#116 meeting [4].

Issue 1-2-4: MCS level for 64 CSI-RS ports Agreement: - Start simulation work with both MCS20 and MCS22. Down select further based on simulation results.

PMI reporting performance for FDD 2Rx

For FDD with 64Tx and 2Rx antennas cases, the relative throughputs for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS20 (Table 1) and MCS22 (Table 1) are captured as Figure 2-1, and the ideal SNR working points at 90% maximum throughput are summarized in Table 2-2. Accordingly, the gamma value with legacy definition as below could be summarized in Table 2-3.

Legacy test metric gamma definition for PMI reporting requirements

γ=tuetrnd tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trndis the throughput measured at SNRue with random precoding.

Figure 2-1. Relative throughputs comparison with different MCS values for FDD 64T2R2Layer

Table 2-2. Ideal SNR working points at 90% maximum throughput for FDD 64T2R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 1-1 | eTypeII-r19 | 1.2 | 4.8 |
| 1-2 | TypeI-SP-r19 | 6.2 | 10.7 |

Table 2-3. Gamma value for FDD 64T2R2Layer

| Test num. | Codebook | Gamma value |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 1-1 | eTypeII-r19 | 62.1 | 41.9 |
| 1-2 | TypeI-SP-r19 | 12.1 | 7.5 |

PMI reporting performance for FDD 4Rx

For FDD with 64Tx and 4Rx antennas cases, the relative throughputs for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS20 (Table 1) and MCS22 (Table 1) are captured as Figure 2-2, and the ideal SNR working points at 90% maximum throughput are summarized in Table 2-4. Accordingly, the gamma value with legacy definition could be summarized in Table 2-5.

Figure 2-2. Relative throughputs comparison with different MCS values for FDD 64T4R2Layer

Table 2-4. Ideal SNR working points at 90% maximum throughput for FDD 64T4R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 2-1 | eTypeII-r19 | -2.7 | 0.5 |
| 2-2 | TypeI-SP-r19 | 1.3 | 2.8 |

Table 2-5. Gamma value for FDD 64T4R2Layer

| Test num. | Codebook | Gamma value |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 2-1 | eTypeII-r19 | 82.3 | 46.7 |
| 2-2 | TypeI-SP-r19 | 17.6 | 20.1 |

From above simulation results for MCS20 (Table 1) vs MCS22 (Table 1), we could observe that the 90% SNR working points of the cases with MCS20 (Table 1) are lower than MCS22 (Table 1), and the 90% SNR working points with MCS20 (Table 1) 4Rx are even lower than zero for eTypeII-r19 codebook, which is not suitable for defining gamma value.

Observation 1: For FDD 2Rx and 4Rx, the 90% SNR working points of the cases with MCS20 (Table 1) are lower than MCS22 (Table 1), and the 90% SNR working points with MCS20 (Table 1) 4Rx is even lower than zero for eTypeII-r19 codebook, which is not suitable for defining gamma value.

Observation 2: With MCS22 (Table 1), the gamma value for 2Rx FDD: 40 for eTypeII-r19, 7.5 for TypeI-SinglePannel-r19; the gamma value for 4Rx FDD: 45 for eTypeII-r19, 20 for TypeI-SinglePannel-r19.

  - Simulation results for TDD

PMI reporting performance for TDD 2Rx

For TDD with 64Tx and 2Rx antennas cases, the relative throughputs for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS20 (Table 1) and MCS22 (Table 1) are captured as Figure 2-3, and the ideal SNR working points at 90% maximum throughput are summarized in Table 2-6. Accordingly, the gamma value with legacy definition as below could be summarized in Table 2-7.

Figure 2-3. Relative throughputs comparison with different MCS values for TDD 64T2R2Layer

Table 2-6. Ideal SNR working points at 90% maximum throughput for TDD 64T2R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 3-1 | eTypeII-r19 | 0.0 | 1.9 |
| 3-2 | TypeI-SP-r19 | 5.9 | 9.7 |

Table 2-7. Gamma value for TDD 64T2R2Layer

| Test num. | Codebook | Gamma value |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 3-1 | eTypeII-r19 | 95.4 | 129.4 |
| 3-2 | TypeI-SP-r19 | 13.8 | 9.3 |

PMI reporting performance for TDD 4Rx

For TDD with 64Tx and 4Rx antennas cases, the relative throughputs for following PMI eTypeII-r19, following PMI TypeI-SinglePanel-r19 and Random PMI with MCS20 (Table 1) and MCS22 (Table 1) are captured as Figure 2-4, and the ideal SNR working points at 90% maximum throughput are summarized in Table 2-8. Accordingly, the gamma value with legacy definition as below could be summarized in Table 2-9.

Figure 2-4. Relative throughputs comparison with different MCS values for TDD 64T4R2Layer

Table 2-8. Ideal SNR working points at 90% maximum throughput for TDD 64T4R2Layer

| Test num. | Codebook | SNR (dB) @ 90% maximum throughput |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 4-1 | eTypeII-r19 | -3.0 | -1.0 |
| 4-2 | TypeI-SP-r19 | -0.2 | 1.7 |

Table 2-9. Gamma value for TDD 64T4R2Layer

| Test num. | Codebook | Gamma value |  |
| --- | --- | --- | --- |
|  |  | MCS20 | MCS22 |
| 4-1 | eTypeII-r19 | 102.4 | 61.3 |
| 4-2 | TypeI-SP-r19 | 30.6 | 24.8 |

From above simulation results for MCS20 (Table 1) vs MCS22 (Table 1), we could observe that the 90% SNR working points of the cases with MCS20 (Table 1) are lower than MCS22 (Table 1), and the 90% SNR working points with MCS20 (Table 1) are a little bit too low for defining gamma value.

Observation 3: For TDD 2Rx and 4Rx, the 90% SNR working points of the cases with MCS20 (Table 1) are lower than MCS22 (Table 1), and the 90% SNR working points with MCS20 (Table 1) are a little bit too low for defining gamma value.

Observation 4: With MCS22 (Table 1), the gamma value for 2Rx TDD: 125 for eTypeII-r19, 9 for TypeI-SinglePannel-r19; the gamma value for 4Rx FDD: 60 for eTypeII-r19, 24 for TypeI-SinglePannel-r19.

- Conclusion

In this contribution, we provide our initial simulation results for PMI reporting performance for both eTypeII-r19 and TypeI-SinglePannel-r19 codebooks on “MIMO Phase 5” topic.

Based on the simulation results, observations could be summarized as following.

Observation 1: For FDD 2Rx and 4Rx, the 90% SNR working points of the cases with MCS20 (Table 1) are lower than MCS22 (Table 1), and the 90% SNR working points with MCS20 (Table 1) 4Rx is even lower than zero for eTypeII-r19 codebook, which is not suitable for defining gamma value.

Observation 2: With MCS22 (Table 1), the gamma value for 2Rx FDD: 40 for eTypeII-r19, 7.5 for TypeI-SinglePannel-r19; the gamma value for 4Rx FDD: 45 for eTypeII-r19, 20 for TypeI-SinglePannel-r19.

Observation 3: For TDD 2Rx and 4Rx, the 90% SNR working points of the cases with MCS20 (Table 1) are lower than MCS22 (Table 1), and the 90% SNR working points with MCS20 (Table 1) are a little bit too low for defining gamma value.

Observation 4: With MCS22 (Table 1), the gamma value for 2Rx TDD: 125 for eTypeII-r19, 9 for TypeI-SinglePannel-r19; the gamma value for 4Rx FDD: 60 for eTypeII-r19, 24 for TypeI-SinglePannel-r19.

- Reference

[1] RP-234007, New WID: NR MIMO Phase 5, RAN#102 meeting, Samsung

[2] RP-242394, WID revision: NR MIMO Phase 5, RAN#105 meeting, Samsung

[3] R4-2508627, Way Forward for [115][329] NR_MIMO_demod, RAN4#115 meeting, Samsung

[4] R4-2512527, Way Forward for [116][327] NR_MIMO_demod, RAN4#116 meeting, Samsung