3GPP TSG-RAN WG4 Meeting #116-bis														R4-2513876

Prague, Czech, 13th – 17th October, 2025

Agenda item:			7.3.3

Source:	Samsung

Title:	Discussion on test method for NTN Ka and Ku bands

Document for:	Discussion

1	Introduction

The core part for Rel-20 UE OTA is focused on VSAT test method in Ka and Ku bands [1, RP-252903]

- Define enhanced test methodologies to cover NTN Ka and Ku bands - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority - FR1 OTA test method extension is not precluded, if also applicable - Study whether and how to support circular polarization measurements - Linear polarization is the baseline - Specify the required quiet zone to cover VSAT device type based on industry needs - 60cm QZ as a starting point - Specify preliminary MU assessment for test methodologies

The NTN requirements for Ka and Ku bands cover many VSAT types and test cases, in order to proceed the work efficiently, we may need to prioritize partial cases. So, in this contribution we share our initial views on some aspects RAN4 needs to focus on.

2	Discussion

As indicated in TS 38.101-5, there are already 6 kinds of VSAT types including fixed VSAT and mobile VSAT with both mechanical steering antenna and electronic steering antenna:

Table 9.2.1.0-1 (of TS38.101-5): The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

In Rel-15/16 FR2 mmWave testability study, only power class 3 handheld UE is considered. Typical VSAT type(s) may also need to be selected as starting point for Ku and Ka bands test method study.

Moreover, Ku and Ka bands requirements involves with a whole set of Tx and Rx test cases. As a starting point, it may also necessary to focus on MOP and reference sensitivity. MOP includes minimum peak EIRP, TRPmax and EIRP max, reference sensitivity is EISREFSENS which his declared by manufacturer.

Proposal 1:	RAN4 needs to select some VSAT type(s) and test cases as starting point, e.g., mobile VSAT type(s), minimum peak EIRP, TRPmax, EIRPmax and EISREFSENS test cases.

According to the WID [1], FR2 test methods (DFF, IFF, NFTF) extension is the first priority. Based on the experience of FR2 test methods, the first issue is to evaluate the minimum range length based on far field criteria and then the SNR range based on minimum range length. The following statements are cited from TR38.810:

The setup in Figure 5.2.1.2-3 is used to derive the minimum range length for NR FR2 DFF systems where the sphere enclosing the DUT matches the QZ and the DUT antenna with radiating aperture diameter D located in the corner of the DUT. With this setup, the minimum range length, RDFF, can be determined as RDFF = RQZ – D/2 + RFF = RQZ – D/2 + 2D2/ which is tabulated in Table 5.2.1.2-1 for two different QZ sizes assuming D=5cm. Figure 5.2.1.2-3: Illustration of DFF System for minimum range length definition Table 5.2.1.2-1: Minimum Range Length of DFF System for D = 5cm f [GHz] QZ [cm]; 24.25; 30; 40; 50; 52.6. 15; 0.45; 0.55; 0.72; 0.88; 0.93. 30; 0.53; 0.63; 0.79; 0.96; 1.00.

Observation 1:	The minimum range length is related with frequency, Quiet Zone size and radiating aperture diameter D.

In Annex B of TR38.810, testable SNR range is also accessed. The SNR upper bound is as following for D=5cm.

Table B.2.2.5.4-1: Predicted SNR upper bound values for Indirect far field (IFF) multi band UE Angle of Arrival; Beam type; Allocated RBs; Maximum SNR. Rx Beam peak; Fine; 24; [22.5dB]. 66; [18.1dB]. Rough; 24; [15.4dB]. 66; [10.9dB]. Spherical coverage; Fine; 24; [10.1dB]. 66; [5.1dB]. Rough; 24; [1.5dB]. 66; Not usable.

The SNR range is one of the bottle neck for FR2 testability even assuming D=5cm. For VSAT, the radiating aperture diameter D may be larger than 5cm and the QZ size is much larger than 15/30cm, and hence the SNR range would be an issue to be analysed.

Observation 2:	SNR range is one of the bottle neck for FR2 testability, larger radiating aperture diameter D and larger QZ size for VSAT may deteriorate this further.

Proposal 2:	RAN4 needs to discuss the radiating aperture diameter assumption and QZ size for the evaluation of minimum range length and SNR range.

In FR2 testing methods and many other OTA work items, black-box approach is adopted, i.e., the geometry centre of the device to be aligned with the test QZ centre. But for VSAT device type, the testing target is probably to focus on the radiating aperture since the testing area is up to manufacturer declaration restricted to upper sphere. This may be beneficial for minimum range length and SNR range.

Proposal 3:	RAN4 needs to discuss if the centre of radiation aperture to be aligned with QZ centre (like white-box approach)

Depending on the evaluation results, the enhanced test method in Rel-16 FR2 testability enhancement may not be precluded.

Beam peak search and measurement grid are important aspects in FR2 test methods. However, the meaning and necessity of beam peak search between FR2 PC3 testing and Ku Ka band VSAT testing may be different. Though the requirements for VSAT are specified at TX/RX beam peak directions, the directions may not be unique but can be a set of directions in range. Especially for mechanical steering VSAT types, beam peak search may not be needed. The measurement grid for FR2 PC3 testing was derived for beam peak search and spherical EIRP/EIS and TRP respectively. But for VSAT types, the measurement grids do not have to be so dense considering the test area is up to declaration on minimum elevation angle.

Observation 3:	the rational of beam peak search and measurement grid can be different between FR2 PC3 testing and Ku Ka VSAT testing.

Proposal 4:	RAN4 needs to discuss if beam peak search for VSAT testing can be skipped and only focus on the single set of measurement grid.

Polarization is another important issue. In Rel-15 FR2 testability study, circular polarization was once discussed and there were high challenges identified in testability at that moment. TE vendors’ feedback is necessary if the situation changed over past years. Even circular polarization becomes available, the supported frequency span may be narrow, the eccentricity may degrade measurement accuracy or the circular polarization antenna at TE side may need to be manually changed when testing different frequency bands. In FR1 TRP TRS work item in Rel-19, circular polarization discussion was also discussed and it is concluded to adopt linear polarization at TE side for NTN handheld UE testing. One of the issues is that the test system upgrading to support circular polarization will bring in additional insertion loss. This will further deteriorate the SNR range.

Observation 4:	Many testability challenges on circular polarization have been identified in FR2 testing study item and FR1 NTN OTA testing work item

As indicated in the WID, linear polarization is the baseline. RAN4 should carefully examine the testability challenges for circular polarization before moving to next step.

Proposal 5:	keep linear polarization as the baseline. RAN4 should carefully examine the testability challenges for circular polarization before moving to next step.

3	Conclusion

Proposal 1:	RAN4 needs to select some VSAT type(s) and test cases as starting point, e.g., mobile VSAT type(s), minimum peak EIRP, TRPmax, EIRPmax and EISREFSENS test cases.

Observation 1:	The minimum range length is related with frequency, Quiet Zone size and radiating aperture diameter D.

Observation 2:	SNR range is one of the bottle neck for FR2 testability, larger radiating aperture diameter D and larger QZ size for VSAT may deteriorate this further.

Proposal 2:	RAN4 needs to discuss the radiating aperture diameter assumption and QZ size for the evaluation of minimum range length and SNR range.

Proposal 3:	RAN4 needs to discuss if the centre of radiation aperture to be aligned with QZ centre (like white-box approach)

Observation 3:	the rational of beam peak search and measurement grid can be different between FR2 PC3 testing and Ku Ka VSAT testing.

Proposal 4:	RAN4 needs to discuss if beam peak search for VSAT testing can be skipped and only focus on the single set of measurement grid.

Observation 4:	Many testability challenges on circular polarization have been identified in FR2 testing study item and FR1 NTN OTA testing work item

Proposal 5:	keep linear polarization as the baseline. RAN4 should carefully examine the testability challenges for circular polarization before moving to next step.

Reference

[1] RP-252903	WID: Enhancement of UE OTA test method and requirements for NR,	 RAN#109

[2] TR38.810		Study on test methods