# 3GPP TSG-RAN WG4 Meeting #116bis                            R4-2513926

# Prague, CZ, 13th-17th Oct., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on co-existence for HPUE

# Agenda Item:   7.1.3

# Document for:  Approval

- Introduction

In last RAN plenary meeting, a new WID on UE RF requirement enhancements for the deployment in NR FR1 was approved [1]. The objective for HPUE of this WID is:

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

In this contribution, we provide some of our considerations on the co-existence issue for CPE/FWA UE with 4x26dBm PA configuration.

- Discussion

2.1 General

Before the study of ACLR requirement, we firstly give a review of power class and PA configurations in legacy NR specification. 1Tx PC1 UE has been studied for public safety scenario, and for other power classes, more than one antennas are supported additionally for UL MIMO or Tx Diversity, the antenna configurations associated with the power class are categorized in Table 1.

Table 1 antenna configurations associated with the power class

|  | PC1(31dBm) | PC1.5(29dBm) | PC2(26dBm) | PC3(23dBm) |
| --- | --- | --- | --- | --- |
| General | 1Tx | 2Tx | 1Tx | 1Tx |
| UL MIMO | NA | 2Tx/4Tx | 2Tx/4Tx | 2Tx/4Tx |
| Tx Diversity | NA | 2Tx/4Tx | 2Tx | 2Tx |

The general ACLR requirements is also defined for different power class in TS38.101-1 as shown below:

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

For UL MIMO and Tx Diversity scenarios with more than one antenna, it is stated that general ACLR requirements applies. It can be concluded that NR UE ACLR requirements are specified per power class, and can be applied to different number of antennas.

Observation 1: NR UE ACLR requirements are specified commonly regardless of the number of antennas per power class.

As the 1Tx PA configuration for public safety scenario is not suitable for typical UE, so that PC1 with 4Tx for CPE/FWA UE is assumed for further study in Rel-20. To implement 4Tx PC1, 4x26dBm PA configuration is then considered. Under the 4x26dBm PA configuration, it is tricky for us to decide whether legacy PC1 or a new power class (32dbm) should be defined. In our opinion, the new introduced power class used for 4x26dBm PA configuration will impact the out of band emission requirements, so before the co-existence study for ACLR requirements for 4x26dBm PA configuration, the power class should be decided in advance. If legacy PC1 is reused, then legacy ACLR requirement could be reused, otherwise the ACLR requirement need to be re-evaluated.

Proposal 1: For 4x26dBm PA configuration, the power class should be decided in advance before ACLR requirements specifying, and:

- If PC1 is considered , the legacy ACLR requirement for 1Tx PC1 could be reused for 4Tx PC1;

- If new Power Class (32dBm) is defined, then the ACLR requirement need to be re-evaluated.

2.2 Coexistence simulation assumptions if needed

Assuming ACLR requirement needs to be re-evaluated for 4x26dBm PA configuration, some coexistence study should be conducted. Coexistence study for HPUE in sub 1G band such as Band 12, Band 5, and Band n71 has been specified in TR 37.880, and the simulation assumptions can be referred to as starting point, then we can just focus on identifying the differences which need to be revised.

Proposal 2: Coexistence simulation assumptions for HPUE in TR37.880 can be referred to as starting point.

To check whether the models or parameters defined for BS, HPUE and Normal UE can be reused or not, we provide a summary in Table 2 as below.

Table 2 summary of the model or parameters for legacy HPUE coexistence simulation assumption check

| Legacy HPUE simulation assumption | Whether to reuse or not | Reversion Mark |
| --- | --- | --- |
| coexistence scenarios | Partial | We focus on UL HPUEs interfere normal NR UE in adjacent channel |
| Macro cell Propagation model | Yes | L (R)= 69.55 +26.16log10(f)–13.82log10(Hb)+[44.9-6.55log10(Hb)]log(R) – 4.78(log10 (f))2+18.33 log10 (f) -40.94 where: R is the base station-UE separation in kilometres f is the carrier frequency in MHz Hb is the base station antenna height above ground in metres Center Frequency f is changed |
| Power control modelling | Yes | Pt=Pmax×min1,maxRmin,CLCLx−ileγ CLx-ile need to be updated |
| Cell layout | Yes | Figure 5.1.5-1 Multi system cell layout with different cell radius |
| Other simulation parameters | No | Including center frequency, bandwidth, antenna array configurations which need to be reconfirmed |

Proposal 3: To consider the legacy HPUE coexistence simulation assumptions check listed in table 2.

- Conclusion

In this contribution, some initial views on HPUE coexistence study are provided and some observations and proposals are made as following:

Observation 1: NR UE ACLR requirements are specified commonly regardless of the number of antenna per power class.

Proposal 1: For 4x26dBm PA configuration, the power class should be decided in advance before ACLR requirements specifying, and:

- If PC1 is considered , the legacy ACLR requirement for 1Tx PC1 could be reused for 4Tx PC1;

- If new Power Class (32dBm) is defined, then the ACLR requirement need to be re-evaluated.

Proposal 2: Coexistence simulation assumptions for HPUE in TR37.880 can be referred to as starting point.

Proposal 3: To consider the legacy HPUE coexistence simulation assumptions check listed in table 2.

Table 2 summary of the model or parameters for legacy HPUE coexistence simulation assumption check

| Legacy HPUE simulation assumption | Whether to reuse or not | Reversion Mark |
| --- | --- | --- |
| coexistence scenarios | Partial | We focus on UL HPUEs interfere normal NR UE in adjacent channel |
| Macro cell Propagation model | Yes | L (R)= 69.55 +26.16log10(f)–13.82log10(Hb)+[44.9-6.55log10(Hb)]log(R) – 4.78(log10 (f))2+18.33 log10 (f) -40.94 where: R is the base station-UE separation in kilometres f is the carrier frequency in MHz Hb is the base station antenna height above ground in metres Center Frequency is changed |
| Power control modelling | Yes | Pt=Pmax×min1,maxRmin,CLCLx−ileγ CLx-ile need to be updated |
| Cell layout | Yes | Figure 5.1.5-1 Multi system cell layout with different cell radius |
| Other simulation parameters | No | Including center frequency, bandwidth, antenna array configurations which need to be reconfirmed |

- Reference

- RP-252962 New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109, Beijing, CN, Sept. 2025.

- 3GPP TR 37.880: "High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71".

- 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".