3GPP TSG-RAN WG4 Meeting #116bis	R4-2513977

Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.14.4

Source:	Ericsson

Title:	Device demodulation requirements for Ambient IoT

Document for:	Discussion

# 1	Introduction

RAN4#116 agreed with the way forward on the Ambient IoT Device 1 demodulation requirements [1]. This contribution discusses the open issues on the device demodulation requirements.

# 2	Discussion

RAN4#116 discussed whether to define Device 1 demodulation requirements or not.

Issue 2-1: Whether to define PRDCH demodulation requirements for Ambient-IoT Way forward - Option 1: Define PRDCH demodulation requirement. - Option 2: Not define PRDCH demodulation requirements for Ambient-IoT considering that it can be verified by the RF REFSENS requirement

According to the latest Ambient IoT device radio transmission and reception [2], approved in RAN#109, RAN4 has defined A-IoT Device 1 Rx REFSENS requirement by 90% of detection success rate of PRDCH. As specified in TS 38.291, PRDCH is the only channel that Ambient IoT Device 1 receives.

RAN4#106 discussed whether to define the Device demodulation requirements, where RAN4 assumed to use 10% BLER of PRDCH, which corresponds to 90% of PRDCH detection success rate. We should also point out that Device 1 RF tests are performed OTA only.

Observation 1: A-IoT Device 1 RF Rx REFSENS requirement is set to achieve 10% BLER of PRDCH.

Observation 2: A-IoT Device 1 RF requirements are performed OTA.

As we discussed in RAN4#106, one difference of RAN4 demodulation requirement compared with RAN4 RF Rx requirement is that the demodulation requirements use fading channel model, although RF requirements use static channel condition. According to the WI summary [3], D2R transmission is backscattered on a carrier wave (CW) provided externally because it is assumed Device 1 is like RFID tag. Moreover, it is assumed that the Device 1 and BS are directly communicated (D1T1) indoors with FR1 FDD band n8 (900MHz).

Observation 3: Rel-19 A-IoT assumes Device 1 is used indoors with FR1 band n8.

Considering the Rel-19 A-IoT Device 1 use case, we can assume Device 1 is stationary. Even if we consider the mobility scenario, the challenge is the testability of fading channel scenario in OTA chamber in the carrier frequency of 900MHz, as it is concerned by TE vendors in RAN4#116.

We therefore think the RF Rx REFSENS requirements are sufficient to verify the Device 1 reception performance.

Observation 4: Device 1 RF Rx REFSENS requirements can verify the functionality of PRDCH reception.

Proposal 1: RAN4 does not define PRDCH demodulation requirements in Rel-19 Ambient-IoT WI.

# 3	Summary

Observations:

Observation 1: A-IoT Device 1 RF Rx REFSENS requirement is set to achieve 10% BLER of PRDCH.

Observation 2: A-IoT Device 1 RF requirements are performed OTA.

Observation 3: Rel-19 A-IoT assumes Device 1 is used indoors with FR1 band n8.

Observation 4: Device 1 RF Rx REFSENS requirements can verify the functionality of PRDCH reception.

Proposals:

Proposal 1: RAN4 does not define PRDCH demodulation requirements in Rel-19 Ambient-IoT WI.

# References

- R4-2512603, “Way Forward for [116][335] A-IoT_demod”, Huawei, CMCC.

- TS 38.191 V1.1.0, “Ambient IoT device radio transmission and reception”, 3GPP.

- RP-252928, “WI summary for Rel-19 Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA Inc.