3GPP TSG RAN WG4 #116-bis	R4-2513983

# Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.15.2.2

Source: 	Ericsson

Title:	Discussion on performance requirements for SSB adaptation

Document for:	Discussion

# Introduction

In last meeting, there is one initial agreement about SSB adaptation test case as follow.

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement

In this contribution, we’ll continue the discussion on the test case design and the related test case list.

# Test case design

Scenario

During RAN4 meetings, RAN4 agreed to support SSB adaptation in the following scenarios.

- L1 measurement

  - L1-RSRP, L1-SINR, CBD

- L3 measurement

In last meeting, the related test case list issue is captured as follow.

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD

From our understanding, RAN4 at least to define the test cases for L1-RSRP, L3-RSRP. In last RAN2 meeting, RAN2 agreed only to use DCI format 2_9 to indicate SSB adaptation. After SSB adaptation, SMTC is also updated.

Proposal 1:  For SSB adaptation, RAN4 to define new test cases to verify SSB(SMTC) adaptation for L3 measurement.

Proposal 2:  For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation for L1-RSRP, L1-SINR and CBD.

Test Principle

RAN4 to discuss the following principles to design the SSB adaptation test cases:

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

To guarantee test converge, both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase should be verified. However, RAN4 shall not define any test cases to verify the transition period similar as legacy test cases. To simplify the test case setting, we prefer the following proposal:

Proposal 3: RAN4 to define the SSB adaptation test cases with the following general princinples:

- Both FR1 and FR2

- Only test non-DRX mode

- without SSB time index detection

- Reuse legacy trigger events for measurement report

- RAN4 not to define the SSB adaptation test case for L1/L3 transition period.

Test Configuration

To verify SSB adaptation, RAN4 needs to introduce new NCD-SSB configuration with 20ms and 80ms. The legacy SMTC configuration with 20ms and 80ms can be reused.

Proposal 4:  RAN4 to verify both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase.

- SMTC periodicity from 80ms to 20ms

- SSB periodicity from 80ms to 20ms

# Test Case List

To tradeoff the burden of tests and the test coverage, we prefer to consider the following test cases.

Proposal 5: RAN4 to define SSB adaptation test cae as follow.

| No. | Test category | Test purpose |
| --- | --- | --- |
| 1 | SSB based L1-RSRP measurement when DRX is not used for SSB adaptation | L1-RSRP measurement |
| 2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used for SSB adaptation | L1-SINR measurement |
| 3 | Beam Failure Detection and Link Recovery Test for FR2 SCell configured with SSB-based BFD and LR in non-DRX mode | Candidate beam detection |
| 4 | SA event triggered reporting tests without gap for SSB adaptation | L3-RSRP measurement(agreed) |

- Summary

In this contribution, we have discussed the performance part of SSB adaptation. Based on the discussions, we made the following proposals and observations:

Proposal 1:  For SSB adaptation, RAN4 to define new test cases to verify SSB(SMTC) adaptation for L3 measurement.

Proposal 2:  For SSB adaptation, RAN4 to define new test cases to verify SSB adaptation for L1-RSRP, L1-SINR and CBD.

Proposal 3: RAN4 to define the SSB adaptation test cases with the following general princinples:

- Both FR1 and FR2

- Only test non-DRX mode

- without SSB time index detection

- Reuse legacy trigger events for measurement report

- RAN4 not to define the SSB adaptation test case for L1/L3 transition period

Proposal 4:  RAN4 to verify both SSB adaptation from a sparse SSB periodicity to a dense SSB periodicity and vice vase.

- SMTC periodicity from 80ms to 20ms

- SSB periodicity from 80ms to 20ms

Proposal 6: RAN4 to define SSB adaptation test cae as follow.

| No. | Test category | Test purpose |
| --- | --- | --- |
| 1 | SSB based L1-RSRP measurement when DRX is not used for SSB adaptation | L1-RSRP measurement |
| 2 | L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used for SSB adaptation | L1-SINR measurement |
| 3 | Beam Failure Detection and Link Recovery Test for FR2 SCell configured with SSB-based BFD and LR in non-DRX mode | Candidate beam detection |
| 4 | SA event triggered reporting tests without gap for SSB adaptation | L3-RSRP measurement(agreed) |

- References

- R4-2512327, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson