3GPP TSG-RAN WG4 Meeting #116 bis	R4-2514054

Prague, CZ, October 13th – October 17th, 2025

Source:	Nokia

Title:	Discussion on UEIBM testing

Agenda item:	6.12.2.1

Document for:	Discussion

Introduction

In this contribution we discuss the necessary test cases for the objectives of the UEIBM.

UE-initiated beam management

With reference to the performance requirements, companies have differing views on which test cases need to be defined.

Issue 2-1-2: Test scope

<Way forward >

- Further discuss based on the table below:

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

Test case to cover Event-2/Event-1/Event-7

In RAN4#115 it was agreed to reuse for Event1 and Event 7, the same design as the requirements defined for Event 2. In view of this, defining a test case to verify UEIBM requirements for Event-2 shall suffice. Defining separate test cases for Event-1 and Event-7 will only increase the test burden and not verify any new requirements.

RAN4 to at least define a test case for Event-2.

Test case for Mode A and Mode B:

Mode A is the basic UE mode of operation, i.e., all UEs supporting UEIBM reporting should support Mode A, whereas Mode B is subject to UE capabilities, i.e. only a subset of the UEs supporting UEIBM may support Mode B. It is straightforward to say that if the UE supports only one of these reporting modes, i.e. Mode A, the UE shall be tested with that mode. But if the UE supports both modes, UE should be tested with both modes. Furthermore, it has to be discussed how to define the test cases for the two modes. Initially, we would think there is no need to duplicate the test cases, but the modes can be listed as options under each test case, and the mode(s) to be used for the test case shall be selected based on UE support.

If UE only supports Mode A, UE to be tested with Mode A.

If UE supports both Mode A and Mode B, UE to be tested with both modes.

Include reporting Mode A and Mode B as test configurations under each UEIBM test. No need to duplicate test cases.

Event-trigger for intra-cell/inter-cell BM for CSI-RS and SSB

RAN4 has agreed to define the event-triggered L1 UEIBM reporting delay requirements which apply to FR2 intra-cell SSB and CSI-RS based reporting, and inter-cell SSB based reporting. In the case of inter-cell CSI-RS based reporting, it has been agreed to reuse the existing requirements structure. In view of this, RAN4 shall define test cases for Event-trigger for intra-cell and inter-cell BM for CSI-RS and SSB.

RAN4 shall define test cases for Event-trigger for intra-cell and inter-cell BM for CSI-RS and SSB based reporting.

Test case for no time window and eventInstanceCount-r19 > 1:

Since core requirements have been defined for eventInstanceCount-r19>1, RAN4 should define a test case for the window-based reporting approach. Test cases should be defined for both SSB and CSI-RS based reporting.

RAN4 to define test cases when eventInstanceCount-r19>1 for SSB and CSI-RS based reporting.

Beam switching delay for TCI states switching:

RAN4 has agreed that the existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management. Hence, we see no need to define a new test case for UEIBM.

Since no new TCI state switch delay requirements have been introduced for UEIBM, no need to define a new test case.

The condition met indicator defined by RAN1 is an optional UE capability and is applicable for the window-based approach only when the number of reported beams is greater than 1. Since, this will more likely be a corner case, and we see no need to define a test case for this scenario.

RAN4 to not define a test case for the condition met indicator (CMI).

The table below summaries our view on the test cases that need to be defined for UEIBM.

| Test Case | Nokia Proposal |
| --- | --- |
| High level: Define TCs to verify UEIBM | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2 |
| Whether to cover Mode A and Mode B? | Mode A if UE only supports Mode A. Mode A and Mode B if UE also supports Mode B. |
| Event-trigger for Intra-cell BM for SSB | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y |
| Event-trigger for Inter-cell BM for SSB | Y |
| Event-trigger for Inter-cell BM for CSI-RS | Y |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | Y |
| Beam switching delay for TCI states switching | N |
| CMI | N |

Conclusion

The following Observations and Proposals were made:

Proposal 1: RAN4 to at least define a test case for Event-2.

Proposal 2: If UE only supports Mode A, UE to be tested with Mode A.

Proposal 3: If UE supports both Mode A and Mode B, UE to be tested with both modes.

Proposal 4: Include reporting Mode A and Mode B as test configurations under each UEIBM test. No need to duplicate test cases.

Proposal 5: RAN4 shall define test cases for Event-trigger for intra-cell and inter-cell BM for CSI-RS and SSB based reporting.

Proposal 6: RAN4 to define test cases when eventInstanceCount-r19>1 for SSB and CSI-RS based reporting.

Proposal 7: Since no new TCI state switch delay requirements have been introduced for UEIBM, no need to define a new test case.

Proposal 8: RAN4 to not define a test case for the condition met indicator (CMI).

- References

- R4-2509059, WF on RRM requirements for NI_MIMO_Ph5_Part1, Samsung