3GPP TSG-RAN4 Meeting # 116bis	R4-2514069

# Prague, Czech,  13th October - 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RRM_Ph5-Perf) Draft CR on SSB based Intra-frequency measurement without MG with CSSF solution 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rel-19 introduces the optimization of CSSF outside gap in CA/DC for speeding up FR2-1 L3 measurement delay. The test cases shall take the feature into account. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clause for SSB based Inter-frequency measurement without MG with CSSF solution 3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test case on for SSB based Inter-frequency measurement without MG to verify CSSF solution 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.6.1.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Start of Change 1

#### A.6.6.1.X	SA event triggered reporting tests without gap under non-DRX (FR1 CA, FR1 EN-DC and FR1+FR2 CA) for UE supporting [FR1 only CA and FR1 only NR-DC 3-searcher capability] and/or [FR1+FR2 NR-DC (PCell is FR1 only) 3-searcher capability]

##### A.6.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2 for UE supporting [FR1 only CA and FR1 only NR-DC 3-searcher capability] and/or [FR1+FR2 NR-DC (PCell is FR1 only) 3-searcher capability].

##### A.6.6.1.X.2	Test parameters

In this test, there are three tests with different configurations, The UE is required to pass only one of the three tests that matches its supported capabilities.. The three tests are:

-	Test 1: FR1 CA configuration:  NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 on NR RF channel 2,  NR cell 3 as SCell in FR1 on NR RF channel 3, where NR RF channel 1, NR RF channel 2 and NR RF channel 3 are in differnt bands, furthermore, NR cell 4 as neighbour cell in FR1 on NR RF channel 2 same as NR cell 2.

-	Test 2: FR1 EN-DC configuration: LTE cell 1 as PCell in FR1 on LTE RF channel 1, NR cell 1 as PSCell in FR1 on NR RF channel 2, NR cell 2 as SCell in FR1 on NR RF channel 3 and NR cell 3 as SCell in FR1 on NR RF channel 4, where NR RF channel 2, NR RF channel 3  and NR RF channel 4 are in differnt bands, furthermore, NR cell 4 as neighbour cell in FR1 on NR RF channel 2 same as NR cell 2.

-	Test 3: FR1+FR2 CA configuration:  NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR2 on NR RF channel 2,  NR cell 3 as SCell in FR2 on NR RF channel 3, where NR RF channel 1, NR RF channel 2 and NR RF channel 3 are in differnt bands, furthermore, NR cell 4 as neighbour cell in FR2 on NR RF channel 2 same as NR cell 2.

In the measurement control information, SCells with only SSB based L3 measurement are configured, and a measurement object is configured for the frequency of the neigbor cell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 4.

Supported test configurations of tess are shown in table A.6.6.1.X.1.2-1.

Table A.6.6.1.X.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FR1 cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE cell: LTE FDD FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

The test parameters of tests are shown in table A.6.6.1.X.2-2, A.6.6.1.X.2-3, A.6.6.1.X.2-4  and A.6.6.1.X.2-5 below.

Table A.6.6.1.X.2-2: General test parameters for intra-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration |  |  | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 1 | Test 2 | Test 3 |  |
| NR RF Channel Number |  | Config 1 | Config 2 | Config 3 | 3 | 3 | 3 |  |
| E-UTRA RF Channel Number |  | NA | Config 2 | NA | NA | 1 | NA | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 1 | Config 2 | Config 3 | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) | LTE cell 1 (PCell), NR cell 1(PSCell), NR cell 2(SCell) , NR cell 3(SCell) | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) |  |
| Neighbour cell |  | Config 1 | Config 2 | Config 3 | NR cell 4 | NR cell 4 | NR cell 4 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | Config 2 | Config 3 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 SSB.1 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | Config 2 | Config 3 | 16 |  |  |  |
| Hysteresis | dB | Config 1 | Config 2 | Config 3 | 0 |  |  |  |
| A3-Threshold | dBm | Config 1 | Config 2 | Config 3 | -11 |  |  |  |
| CP length |  | Config 1 | Config 2 | Config 3 | Normal |  |  |  |
| TimeToTrigger | s | Config 1 | Config 2 | Config 3 | 0 |  |  |  |
| Filter coefficient |  | Config 1 | Config 2 | Config 3 | 0 |  |  | L3 filtering is not used |
| DRX |  | Config 1 | Config 2 | Config 3 | OFF |  |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | Config 2 | Config 3 | 3 s |  |  | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | Config 2 | Config 3 | 3s |  |  | Synchronous cells |
| T1 | s | Config 1 | Config 2 | Config 3 | 5 |  |  |  |
| T2 | s | Config 1 | Config 2 | Config 3 | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC |  |

Table A.6.6.2.X.2-3: Cell specific test parameters for intra-frequency event triggered reporting for FR2 without SSB time index detection, for test case 1

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | FDD |  | FDD |  |
| TDD configuration |  |  |  | Config 1 | N/A |  | N/A |  | N/A |  |
| BWchannel |  |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  |  | Config 1 | 52 |  | 52 |  | 52 |  |
| BWP BW |  |  | MHz | Config 1 | 52 |  | 52 |  | 52 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | N/A |  | N/A |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | N/A |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 1 | SR.1.1 FDD |  | - |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 1 | CR.1.1 FDD |  | - |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 1 | CCR.1.1 FDD |  | - |  | - |  |
| SMTC configuration defined |  |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 1 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 1 | -98 |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 1 | 16 |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz | Config 1 | -64.60 |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 1 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

Table A.6.6.2.X.2-4: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection, for test case 2

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  |  | Config 2 | Not Applicable |  | Not Applicable |  | Not Applicable |  |
| BWchannel |  |  | MHz | Config 2 |  |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  |  | Config 2 |  |  | 52 |  | 52 |  |
| BWP BW |  |  | MHz | Config 2 |  |  | 52 |  | 52 |  |
| BWP configuration | Initial DL BWP |  |  | Config 2 |  |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 2 |  |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 2 |  |  | SR.1.1 TDD |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 2 |  |  | CR.1.1 TDD |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  | - |  |
| SMTC configuration defined |  |  |  | Config 2 |  |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 2 |  |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 2 |  |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 2 |  |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 2 |  |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz dBm/95.04 MHz Note 5 | Config 2 |  |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 2 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

Table A.6.6.2.X.1-5: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection, for test case 3

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  |  |  | Config 3 | N/A |  | Setup 3 as specified in clause A.3.15 |  | Setup 3 as specified in clause A.3.15 |  |
|  |  |  |  |  | N/A |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  |  | Config 3 | N/A |  | Rough |  | Rough |  |
| NR RF Channel Number |  |  |  | Config 3 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 3 | FDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 3 | Not Applicable |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  |  | MHz | Config 3 | 10: NPRB,c = 52 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  |  | Config 3 | 52 |  | 66 |  | 66 |  |
| BWP BW |  |  | MHz | Config 3 | 52 |  | 66 |  | 66 |  |
| BWP configuration | Initial DL BWP |  |  | Config 3 | DLBWP.0.1 |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 3 | SR.1.1 FDD |  |  |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 3 | CR.1.1 FDD |  |  |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 3 | CCR.1.1 FDD |  |  |  | - |  |
| SMTC configuration defined |  |  |  | Config 3 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 3 | 15 |  | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 3 | -98 |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 3 | 16 |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz dBm/95.04 MHz Note 5 | Config 3 | -64.60 |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 3 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

1600 for UE supporting power class 1, or

960 for UE supporting other power class.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

1600 for UE supporting power class 1, or

960 for UE supporting other power class.

In test 3, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

1600 for UE supporting power class 1, or

960 for UE supporting other power class.

In test 1, test 2 and test 3 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# End of Change 1