3GPP TSG-RAN WG4 Meeting #116bis	R4-2514084

Prague, Czech, 13th – 17th October, 2025

Agenda item:		6.13.2.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#116 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. The core part was finalized, and it is time to implement the agreements to the performance part of the specification. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation.

# Discussion

## In-band blocking

### SBFD-capable BS in-band blocking requirement

The following WF was agreed during RAN4#116 regarding the in-band blocking requirement:

### Issue 2: In-band blocking value for FR1 WA BS - Agreement - Adopt -25 dBm as the requirement ### Issue 3: In-band blocking value for FR2-1 WA BS - Agreement - Adopt -44 dBm as a provisional requirement. The value and the methodology to derive it shall be revisited in maintenance. ### Issue 4: In-band blocking value for FR1 Micro BS - Agreement - Adopt -32 dBm as the requirement. ### Issue 5: In-band blocking requirement assumption - Agreement - Capture the following as informative NOTE, with the understanding that they are not part of RF requirements. - NOTE1: For FR1 WA BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz with 288 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements - NOTE2: For FR1 MR BS, the in-band blocking requirements are derived based on the simulation assumption of 4GHz outdoor deployment with 167 meters minimum distance coming from 100% grid shift between SBFD-capable BSs and neighbouring adjacent-channel TDD BSs. This NOTE shall not be interpreted as part of RF requirements or in the context of indoor ### Issue 6: Interference signal definition - Agreement - Table 2.2.1-1: FR1 SBFD-capable BS in-band blocking requirements BS channel bandwidth of the lowest/highest carrier received (MHz); Interfering signal mean power (dBm); Interfering signal centre frequency minimum offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz); Type of interfering signal. 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100; Wide Area BS: -25 Medium Range BS: -32 Local Area BS: -35; ±30; 20 MHz CP-OFDM NR signal 30 kHz SCS, 51 RBs.

RAN4 needs to agree on the method of how to derive the in-band blocking requirement for FR2-1. This will be discussed more in our contribution to the maintenance part. There is limited time available for the performance part. Therefore, if no agreement is reached, the first implementation to TS 38.141-2 can be made with the provisional requirement of -44dBm as the interferer power. For FR1 there is an agreement as stated in the table above and it also needs to be implemented to TS 38.141-1/2.

First implementation to TS 38.141-2 can be made with -44dBm as the interferer power.

Agreed requirements for in-band blocking to be implemented in TS 38.141-1/2.

### ACS requirement

The following was agreed during RAN4#116:

- Agreement - For WA SBFD BS type, adopt -28 dBm as the requirement. - For MR and LA SBFD BS types, adopt the following values as requirement respectively. - MR: -32 dBm - LA: -35 dBm

The agreed values for the ACS interferer power levels need to be implemented in TS 38.141-1/2.

Agreed requirements for adjacent channel selectivity to be implemented in TS 38.141-1/2.

### ICS requirement

The following was agreed during RAN4#116:

- Agreement - Existing ICS requirements can be reused with some updates. - The NOTE in ICS requirement should be updated to place the wanted signal and interference signal around the middle of uplink sub-band instead of Fc of the carrier bandwidth.

The agreement to reuse ICS requirements need to be implemented in TS 38.141-1/2. The NOTE also needs to be updated according to the agreement.

Agreed reuse of in-channel selectivity requirements to be implemented in TS 38.141-1/2 and TS 38.141-2 together with the updated NOTE.

# Conclusion

In this contribution, our views on the existing BS RF RX requirements for SBFD operation are presented. There were observations and proposals as listed below.

- First implementation to TS 38.141-2 can be made with -44dBm as the interferer power.

- Agreed requirements for in-band blocking to be implemented in TS 38.141-1/2.

- Agreed requirements for adjacent channel selectivity to be implemented in TS 38.141-1/2.

- Agreed reuse of in-channel selectivity requirements to be implemented in TS 38.141-1/2 and TS 38.141-2 together with the updated NOTE.

# References

- R4-2512680 “Way Forward for [116][307] NR_duplex_evo_BSRF”, RAN4#116, Bengaluru, India, 25th-29th August, 2025