3GPP TSG-RAN WG4 Meeting #116bis	R4-2514088

Prague, Czech, 13th – 17th October, 2025

Agenda item:		4.19.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

Way Forward was agreed in RAN4#116 [1] regarding evolution of NR duplex operation. The core part for NR duplex evolution was finished in RAN4#116 with final agreements made. There were still some items that needs a return to and this contribution shares Nokia’s views on those aspects.

# Discussion

## In-channel adjacent subband leakage for FR1 WA/MR BS, for DU/UD case

The following was agreed in RAN4#116:

Agreement: - Absolute limit for in-channel adjacent subband leakage for DU/UD case - Reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1) for bands: Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit BS category / BS class; ACLR absolute basic limit. Category A Wide Area BS; -13 dBm/MHz. Category B Wide Area BS; -15 dBm/MHz. Medium Range BS; -25 dBm/MHz. Local Area BS; -32 dBm/MHz. - The necessity of the additional 3dB relaxation of ACLR absolute basic limit for band n104 shall be studied in maintenance of Rel-19 SBFD WI

The agreement in RAN4#116 was to reuse legacy ACLR absolute basic limit as described above. However, there is a bullet stating that the necessity of additional 3dB relaxation for band n104 shall be studied in maintenance. We must note immediately that there is no additional relaxation in the legacy ACLR absolute basic limit for band n104. There is a relaxation only in the relative ACLR for legacy. As RAN4 has agreed to only use the absolute limit for in-channel adjacent subband leakage, there should not be any additional relaxation for band n104.

RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# Conclusion

In this contribution, our views on the SBFD-capable BS RF requirements are presented. The following proposals were listed:

- RAN4 not to introduce additional relaxation for the in-channel adjacent subband leakage for band n104.

# References

- R4-2512665 “Way Forward for [116][306] NR_duplex_evo_General”, RAN4#116, Bengaluru, India, 25th-29th August, 2025