# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514104

# Prague, Czech, 13th – 17th October, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM requirements for HD-FDD VSAT on Ku band

Agenda item:	7.6.4.2

Document for:	Approval

Introduction

At RAN plenary #108 meeting, the latest WI“Enhanced requirements for NR NTN and IoT NTN Phase 2”[1] was approved. In current WID, the goal is to specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies. The following description states RAN4’s work in future stage:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability. - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

Discussion

RAN4 #116bis meeting will kick off the study of HD-FDD VSAT operating in both Ka and Ku bands with FR1 and FR2 numerologies. As WID said, phase 1 work will commence immediately, and the study of phase 2 will commence upon completion of phase 1 and RAN1/RAN2 availability. In line with the latest WID regarding the RRM core parts, we would like to outline our initial views on Phase 1.

Observation 1: Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability.

Proposal 1: RAN4 shall firstly discuss RRM core requirements on Phase 1.

  - 2.1 General discussion

It is noted that phase 1 contains two sub-bullets to address distinct scenarios and pertain to different HD-FDD VSAT capabilities.

For the first sub-bullet, it says RAN4 shall introduce a single set of requirements for HD-FDD for VSAT types 2,3,5,6 with electronically steered antenna. Accordingly, there is no need to define separate core requirements for these HD-FDD VSAT with electronically steered antenna.

For the second sub-bullet, it says RAN4 will introduce requirements for HD-FDD mobile VSAT with small antennas with a new VSAT type. It is important to note that this sub-bullet only clarifies a mobile VSAT, without specifying the antenna steering mechanism. Therefore, all potential scenarios—including but not limited to communicating with LEO/GSO and LEO, and with electronic or mechanical antenna steering—should be considered. Nonetheless, it is observed that the new VSAT type has small antenna (20 x 20 cm) and is akin to automotive-mounted VSAT. The vehicle itself is in constant motion and turning. A mechanical steered antenna requires a complex stabilization and tracking system that shall compensate for both the vehicle's own movement and the satellite's movement especially for LEO. This kind of small antenna has slow response times and can easily lose the signal during rapid maneuvers, such as sharp turns. Hence, only electronic steered antenna is considered.

Observation 2: A mechanical antenna has slow response times and can easily lose the signal such as during rapid maneuvers if small antenna is used.

Proposal 2: Only electronic steered antenna shall be considered.

In view of the above, RRM requirements for the complete set of scenarios shall be considered as follows:

Case 1: A set of RRM requirements for HD-FDD on VSAT types 2,3,5,6 with electronic steered antenna.

[Case 2: RRM requirements for HD-FDD mobile VSAT featuring a small electronical steered antenna, communicating with both GSO and LEO.]

[Case 3: RRM requirements for HD-FDD mobile VSAT featuring a small electronical steered antenna, communicating with LEO only.]

Given this, it shall be a consensus that the new VSAT type is studied within the RF session first. Consequently, the RRM requirements for HD-FDD on Case 2 and Case 3 should be deferred until after that discussion.

Proposal 3: RRM requirements shall be defined for the following cases:

Case 1: A set of RRM requirements for HD-FDD on VSAT types 2,3,5,6 with electronic steered antenna.

[Case 2: RRM requirements for HD-FDD mobile VSAT featuring a small electronical steered antenna, communicating with both GSO and LEO.]

[Case 3: RRM requirements for HD-FDD mobile VSAT featuring a small electronical steered antenna, communicating with LEO only.]

Proposal 4: The discussion on RRM requirements shall await the outcomes in RF discussion regarding the new VSAT type with small antenna.

  - 2.2 RRM core requirements

The latest WID states leveraging the RedCap NTN HD-FDD solution for NTN HD-FDD VSAT where applicable. We understand this as a directive to use the established RedCap NTN HD-FDD core requirements as a starting point for defining the RRM requirements for VSAT. Thus, the first step is to systematically review the RedCap requirements to ensure consistency when applying the same logic to the VSAT types 2,3,5,6 (Case 1).

1. Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state 2. Handover interruption time 3. Random access 4. SA: RRC Connection Release with Redirection 5. Minimum requirement for L1 indication for RLM and LR 6. Scheduling availability of UE performing intra/inter measurements

The above requirements shall be considered for NTN HD-FDD VSAT types 2,3,5,6 (Case 1).

Proposal 5: The following core requirements shall be considered for HD-FDD on VSAT types 2,3,5,6 in NTN Ku band scenario with FR1 numerology:

1. Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state 2. Handover interruption time 3. Random access 4. SA: RRC Connection Release with Redirection 5. Minimum requirement for L1 indication for RLM and LR 6. Scheduling availability of UE performing intra/inter measurements

For paging reception requirements, the existing specification could be seen as below:

The FDD, HD-FDD and TDD RedCap UE shall meet all applicable requirements specified in clause 4.2.2.6. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. - The 1 Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2.2.6 under the following conditions - - at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration.

It is obvious that the paging reception requirements for 2Rx and 1Rx RedCap in HD-FDD does not distinguish the frequency range. It mainly points to UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. Consequently, we do think legacy paging reception requirements shall be reused for 2Rx and 1Rx in HD-FDD VSAT in NTN scenario.

The random access and RRC connection release with redirection requirements have the same story as paging reception requirements.

Proposal 6: The legacy paging reception requirements defined in RedCap HD-FDD in NTN scenario shall be reused for 2Rx and 1Rx in HD-FDD in NTN Ku band with FR1 numerology.

Proposal 7: The legacy random access requirements defined in RedCap HD-FDD in NTN scenario shall be reused for 2Rx and 1Rx in HD-FDD in NTN Ku band with FR1 numerology.

Proposal 8: The RRC connection release with redirection requirements defined in RedCap HD-FDD in NTN scenario shall be reused for 2Rx and 1Rx in HD-FDD in NTN Ku band with FR1 numerology.

For RACH-based handover and satellite seitching with re-synchronization, the following rules shall be added:

- For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

- For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

  - -One SSB is available during T∆

  - -One SSB is available during TIU

  - Actually, these applicability rules and requirements have been defined in Ka band NTN. In Rel-19 discussion, the frequency range for NTN has been extended and contained Ku band and Ka band. We also have one conclusion as below:

Agreement: - Clarify in clause 3.6.12 that “Unless otherwise stated, the requirements defined for FR2-NTN apply for VSAT working in FR1-NTN”.

  - Following this logic, we deem that the above applicability rule shall be reused.

Proposal 9: For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

  - -One SSB is available during T∆

  - -One SSB is available during TIU

Conclusion

In this paper we provided our views on RRM requirements on R19 NR Ku band NTN enhancement.

Observation 1: Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability.

Proposal 1: RAN4 shall firstly discuss RRM core requirements on Phase 1.

Observation 2: A mechanical antenna has slow response times and can easily lose the signal such as during rapid maneuvers if small antenna is used.

Proposal 2: Only electronic steered antenna shall be considered.

Proposal 3: RRM requirements shall be defined for the following cases:

Case 1: A set of RRM requirements for HD-FDD on VSAT types 2,3,5,6 with electronic steered antenna.

[Case 2: RRM requirements for HD-FDD mobile VSAT featuring a small electronical steered antenna, communicating with both GSO and LEO.]

[Case 3: RRM requirements for HD-FDD mobile VSAT featuring a small electronical steered antenna, communicating with LEO only.]

Proposal 4: The discussion on RRM requirements shall await the outcomes in RF discussion regarding the new VSAT type with small antenna.

Proposal 5: The following core requirements shall be considered for HD-FDD on VSAT types 2,3,5,6 in NTN Ku band scenario with FR1 numerology:

1. Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state 2. Handover interruption time 3. Random access 4. SA: RRC Connection Release with Redirection 5. Minimum requirement for L1 indication for RLM and LR 6. Scheduling availability of UE performing intra/inter measurements

Proposal 6: The legacy paging reception requirements defined in RedCap HD-FDD in NTN scenario shall be reused for 2Rx and 1Rx in HD-FDD in NTN Ku band with FR1 numerology.

Proposal 7: The legacy random access requirements defined in RedCap HD-FDD in NTN scenario shall be reused for 2Rx and 1Rx in HD-FDD in NTN Ku band with FR1 numerology.

Proposal 8: The RRC connection release with redirection requirements defined in RedCap HD-FDD in NTN scenario shall be reused for 2Rx and 1Rx in HD-FDD in NTN Ku band with FR1 numerology.

Proposal 9: For RACH-based HO and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

  - -SSB is available at the UE once every SMTC period during Tsearch

  - -One SSB is available during T∆

  - -One SSB is available during TIU

For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

  - -One SSB is available during T∆

  - -One SSB is available during TIU

- References

- RP-252958, WID: New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2.