3GPP TSG-RAN WG4 Meeting # 116bis 	    R4-2514117

Prague, CZ, 13 – 17 October 2025

Agenda Item:	6.11.5

Source: 	Ericsson

Title:	On core requirement for AI/ML based positioning

Document for:	Approval

# Introduction

This contribution discusses the core requirements for UE-based positioning case 1 in the revised WID for AI/ML for NR air interface [1].

# Discussion

UE feature list for AI/ML based positioning was discussed and agreed in RAN1#112 meeting, where the following can be observed [2].

Observation 1: Depending on the UE capability, network can configure UE to perform UE based positioning case 1, or UE based positioning case 1 + non-AI/ML based DL-TDoA positioning, or UE based positioning case 1 + non-AI/ML based DL-AoD positioning.

Observation 2: Depending on UE capability, UE can be configured to perform PRS measurement within measurement gap or outside of the measurement gap in a PRS processing window (PPW) for UE based positioning case 1.

Observation 3: Depending on the UE capability network can configure UE to perform positioning measurements for UE based positioning case 1 by aggregating PRS resources from 2 or 3 PFLs.

Based on the observations above, following are the scenarios that are valid from the signalling point of view.

- Scenario 1: UE-based positioning Case 1.

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

  - measurement with and without PRS aggregation from 2 and 3 PFLs

- Scenario 2: Simultaneous UE-based positioning Case 1 and DL-TDoA positioning

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

- measurement with and without PRS aggregation from 2 and 3 PFLs

- Scenario 3: Simultaneous UE-based positioning Case 1 and DL-AoD positioning

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

  - measurement with and without PRS aggregation from 2 and 3 PFLs

From the requirement point of view, scenario 1 where UE is configured to perform UE-based positioning 1 is the scenario of RAN4 concern. Scenario 2 and scenario 3 are valid scenarios where the UE can potentially use the UE location estimated based on DL-TDoA or DL-AoD positioning to evaluate the performance of its AI/ML model for UE location inferencing. Moreover, when UE is configured for scenario 2 or scenario 3, UE is not expected to report position reports for UE-based positioning case 1 and DL-TDoA or DL-AoD positioning. We therefore propose to focus on scenario 1 and complete the core requirement applicable to this scenario first.

The core part for the AI/ML positioning case 1 needs to be completed by RAN#110 which means RAN4 will have technically two meetings including RAN4#116bis meeting. As highlighted, scenario 1 can be realized by UE by performing PRS measurements within measurement gap, outside of the measurement gap, and by aggregating PRS resources from multiple PFLs. Defining requirements for all these different flavours of UE-based positioning case 1 will require quite an effort and may not be feasible to accomplish within the extended time frame. We therefore propose to prioritize, if not specify the core requirement only for the case where UE is configured for UE-based positioning case 1 by LMF.

Observation 4: Following are the scenarios that are valid from the signalling point of view.

- Scenario 1: UE-based positioning Case 1

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

  - measurement with and without PRS aggregation from 2 and 3 PFLs

- Scenario 2: Simultaneous UE-based positioning Case 1 and DL-TDoA positioning

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

- measurement with and without PRS aggregation from 2 and 3 PFLs

- Scenario 3: Simultaneous UE-based positioning Case 1 and DL-AoD positioning

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

  - measurement with and without PRS aggregation from 2 and 3 PFLs

Proposal 1: RAN4 to define core requirements for the scenario where the UE is only configured to perform UE based positioning case 1.

In RAN4#116 meeting reporting delay requirement for UE-based positioning case 1 was discussed. The agreement, copied below for reference, has an FFS on number of samples to be considered for core requirement.

Agreement from RAN4#116 [3]:

- Take the framework of the existing reporting delay requirements

- Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report

- Check until next meeting on the number of samples needed for the measurements

For legacy positioning measurements number of samples for positioning measurements is based on the UE capability to perform positioning measurements with reduced number of samples, whether the PRS bandwidth is within the active bandwidth part of the UE, and if the magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6dB. In our view, the same principle can also be applied to the scenario where the UE is configured to perform UE-based positioning case 1.

Proposal 2: In the core requirement for UE based positioning case 1, Nsample = 1 if UE supports reduced number of samples for PRS measurement, PRS bandwidth is within the active bandwidth part, and magnitude of difference between the serving cell’s SS-RSRP and neighbor cell’s PRS-RSRP is within 6dB.

Proposal 3: In the core requirement for UE based positioning case 1, Nsample = 2 if UE supports reduced number of samples for PRS measurement, PRS bandwidth is not within the active bandwidth part, and magnitude of difference between the serving cell’s SS-RSRP and neighbor cell’s PRS-RSRP is not within 6dB.

Proposal 4: In the core requirement for UE based positioning use case 1, Nsample = 4 if UE does not support reduced number of samples for PRS measurement.

Like Nsample, impact of Rx beam sweeping factor can also be inherited to the core requirement for UE-based positioning case 1 for FR2. For the core requirement in FR2, number of Rx beam sweeping factor can be equal to the value that UE has reported as its capability to the network. For UEs that cannot perform PRS measurement with reduced Rx beam sweeping factor the default value of 8 shall be considered.

Proposal 5: For UEs that support reduced Rx beam sweeping factor for PRS measurement in FR2, the Rx beam sweeping factor in the core requirement for UE-based positioning case 1 is equal to the value reported by UE as its capability.

Proposal 6: For UEs that do not support reduced Rx beam sweeping factor for PRS measurement in FR2, default value of 8 is considered in the core requirement for UE-based positioning case 1.

Unlike in the legacy positioning measurements, UE can perform PRS measurements for its model input with a single Rx timing error group (TEG) limiting the impact of number of Rx TEGs on the measurement period requirement for UE-based positioning case 1. Besides, the usage of Rx TEG is also transparent from the signaling point of view, thus adding limitation to the testing procedure to verify the impact of number of TEGs on the measurement delay requirement. Therefore, impact of the number of Rx TEGs on the measurement period requirement for UE-based positioning case 1 should not be considered.

Proposal 7: Impact of the number of Rx TEGs on the measurement period requirement for UE-based positioning case 1 should not be considered.

# Summary

In this contribution, issues related to the core requirement for UE-based positioning case 1 in WID are discussed. The observations and proposals below summarize the discussions presented in this paper.

Observation 1: Depending on the UE capability, network can configure UE to perform UE based positioning case 1, or UE based positioning case 1 + non-AI/ML based DL-TDoA positioning, or UE based positioning case 1 + non-AI/ML based DL-AoD positioning.

Observation 2: Depending on UE capability, UE can be configured to perform PRS measurement within measurement gap or outside of the measurement gap in a PRS processing window (PPW) for UE based positioning case 1.

Observation 3: Depending on the UE capability network can configure UE to perform positioning measurements for UE based positioning case 1 by aggregating PRS resources from 2 or 3 PFLs.

Observation 4: Following are the scenarios that are valid from the signalling point of view.

- Scenario 1: UE-based positioning Case 1

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

  - measurement with and without PRS aggregation from 2 and 3 PFLs

- Scenario 2: Simultaneous UE-based positioning Case 1 and DL-TDoA positioning

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

- measurement with and without PRS aggregation from 2 and 3 PFLs

- Scenario 3: Simultaneous UE-based positioning Case 1 and DL-AoD positioning

  - PRS measurement within measurement gap

  - PRS measurement outside of measurement gap

  - measurement with and without PRS aggregation from 2 and 3 PFLs

Proposal 1: RAN4 to define core requirements for the scenario where the UE is only configured to perform UE based positioning case 1.

Proposal 2: In the core requirement for UE based positioning case 1, Nsample = 1 if UE supports reduced number of samples for PRS measurement, PRS bandwidth is within the active bandwidth part, and magnitude of difference between the serving cell’s SS-RSRP and neighbor cell’s PRS-RSRP is within 6dB.

Proposal 3: In the core requirement for UE based positioning case 1, Nsample = 2 if UE supports reduced number of samples for PRS measurement, PRS bandwidth is not within the active bandwidth part, and magnitude of difference between the serving cell’s SS-RSRP and neighbor cell’s PRS-RSRP is not within 6dB.

Proposal 4: In the core requirement for UE based positioning use case 1, Nsample = 4 if UE does not support reduced number of samples for PRS measurement.

Proposal 5: For UEs that support reduced Rx beam sweeping factor for PRS measurement in FR2, the Rx beam sweeping factor in the core requirement for UE-based positioning case 1 is equal to the value reported by UE as its capability.

Proposal 6: For UEs that do not support reduced Rx beam sweeping factor for PRS measurement in FR2, default value of 8 is considered in the core requirement for UE-based positioning case 1.

Proposal 7: Impact of the number of Rx TEGs on the measurement period requirement for UE-based positioning case 1 should not be considered.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

- R1-2506627, Updated RAN1 UE features list for Rel-19 NR after RAN1 #122, Moderators (AT&T, NTT DOCOMO, INC.).

- R4-2511890, WF on NR_AIML_air_part1, Qualcomm Incorporated.