3GPP TSG-RAN WG4 Meeting # 116bis	    R4-2514120

Prague, CZ, 13 – 17 October 2025

Agenda Item:	7.12.2

Source: 	Ericsson

Title:	General discussion on AI/ML for mobility

Document for:	Approval

# Introduction

This contribution discusses modeling of error in measurements used by UE as an input to AI/ML model for RRM measurement prediction, and aspects related to simulation evaluation to be conducted by RAN4 during the work item phase.

# Discussion

## Error Modelling

To predict the RRM measurement, the UE needs to perform measurements for model input. The accuracy of predicted measurements depends on the accuracy of the measurements provided as an input to the AI/ML model. In RAN4#116 meeting, it was there agreed to consider the impact of baseband error and RF error in input measurements on the accuracy of the predicted measurements. The agreement is copied below for reference purposes.

Agreement from RAN4#116 [1]:

- RAN4 to capture the following to model measurement error to consider the impact on prediction accuracy:

- As a baseline:

  - For BB error, use link level simulation to generate L3-RSRP difference as baseband error.

  - For RF error model, use a truncated Gaussian distribution.

    - Further discuss in the WI phase either the same or different RF error value may be considered at least for different frequency layers and/or different RF chains.

To evaluate the impact of RF error on RRM measurement prediction accuracy a truncated Gaussian distribution with zero mean and variance corresponding to the RF calibration error (±2.5dB for FR1 and ±4dB for FR2) for RSRP measurement shall be considered. Since RF error may appear differently when UE is performing measurements in different frequency layers, different RF error value may need to be considered for measurements in different frequency layers during simulations. Although a UE may have different RF chains that it can use to perform measurements to be used as input to its AI/ML model. But it is very likely that the UE will use the same Rx chain to perform measurements on different cells for cell level L3-RSRP measurements. Therefore, in our view, the consideration of different RF error values for different RF chains does not seem reasonable for L3-RRM measurement prediction and shall not be considered in RAN4 simulations.

Observation 1: RF error can appear differently when UE is performing measurements in different frequency layers.

Observation 2: UE can use the same Rx chain to perform measurements on different cells for cell level L3-RSRP measurements.

Proposal 1: RF error in FR1 follows a Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

Proposal 2: RF error in FR2 follows a Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Proposal 3: RAN4 to not consider different RF error values for different RF chains for RRM measurement prediction simulations.

## Simulation Evaluation

Consideration of RF error in simulation is a new approach that is under discussion in RAN4 for AI/ML aided mobility. There can potentially be two different approaches to consider impact of RF error in simulations. One of the approaches is to consider impact of RF error at point C of Figure 1 and the other approach is to consider the impact of RF error at point A of Figure 1. How to consider the RF error in simulations therefore needs to be discussed and a common understanding should be established to ensure results from different companies are produced in a similar simulation setting.

Figure 1. L3-RSRP measurement model from 38.300.

Proposal 4: RAN4 to agree on one of the following options for considering RF error in simulations for AI/ML aided mobility.

- Option 1: consider impact of RF error at point C in L3-RSRP measurement model from 38.300.

- Option 2: consider the impact of RF error at point A in L3-RSRP measurement model from 38.300.

During the study item consideration of UE trajectory for AI/ML aided mobility was discussed but no conclusion could be met. It was observed in the contributions from companies that a reference UE trajectory needs to be considered to at least ensure prediction consistency in time domain and will also have an impact on test setup for FR2. These issues will have an impact on RAN4 evaluations, requirements that RAN4 aims to define, and the test setup that will be required to validate UE capability of meeting the RAN4 requirements.

Proposal 5: RAN4 to discuss whether to define a reference UE trajectory to be considered for AI/ML aided mobility simulations.

# Summary

Issues related to modeling of error in measurements used by UE as an input to AI/ML model for RRM measurement prediction, and aspects related to simulation evaluation to be conducted by RAN4 during the work item phase are discussed in this paper. The observations and proposals below summarize the discussion presented in this paper.

Observation 1: RF error can appear differently when UE is performing measurements in different frequency layers.

Observation 2: UE can use the same Rx chain to perform measurements on different cells for cell level L3-RSRP measurements.

Proposal 1: RF error in FR1 follows a Gaussian distribution with μ=0,  σ2 = 2.5 truncated at ±2.5.

Proposal 2: RF error in FR2 follows a Gaussian distribution with μ=0,  σ2 = 4 truncated at ±4.

Proposal 3: RAN4 to not consider different RF error values for different RF chains for RRM measurement prediction simulations.

Proposal 4: RAN4 to agree on one of the following options for considering RF error in simulations for AI/ML aided mobility.

- Option 1: consider impact of RF error at point C in L3-RSRP measurement model from 38.300.

- Option 2: consider the impact of RF error at point A in L3-RSRP measurement model from 38.300.

Proposal 5: RAN4 to discuss whether to define a reference UE trajectory to be considered for AI/ML aided mobility simulations.

# References

R4-2512127, WF_AIML_Mobility_Part_1, Moderator (Nokia).