# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514151

# Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Simulation results on SAN demodulation requirements for IoT-NTN

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.20.4

# Document for:	Discussion

# 1 Introduction

RAN4#116 [1] and RAN4#115 [2] meetings agreed with the way forward on Rel-19 IoT-NTN phase 3 SAN demodulation requirements, where RAN4 are going to define NPUSCH format 1 demodulation requirements with OCC2. This contribution provides our simulation results.

# 2 Discussion

In the last meeting, RAN4 discussed the detailed assumptions for IoT-NTN phase3 SAN demodulation requirements with OCC operation. Based on the previous meeting agreements, we provide the initial simulation results in the following.

The SNR corresponding to 70% maximum throughput is shown in the following.

| Number of TX antennas | Number of RX antennas | Subcarrier spacing | Number of allocated subcarriers | Propagation conditions and correlation matrix (Annex D)) | FRC | Frequency offset | TxDuration | Repetition number(NRep in 36.213 Table 16.5.1.1-3) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 | 70% | -4.2 |
| 1 | 2 | 3.75KHz | 1 | NTN TDLA100-1 | A7-1 (Note 1) | 128 Hz | 256 | 2 | 70% | -7.3 |
| 1 | 1 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 256 | 8 | 70% | -11.7 |
| 1 | 2 | 15KHz | 1 | NTN TDLA100-1 | A16-2 (Note 2) | 128 Hz | 256 | 8 | 70% | -14.2 |

# 3 Conclusion

In this contribution, we provide our initial simulation results on IoT NTN Phase3 SAN demodulation performance requirements.

# 4 References

- R4-2508756, Way Forward for [115][334] IoT_NTN_Ph3_demod, ZTE Corporation, Sanechips.

- R4-2512645 Way Forward for [116][332] IoT_NTN_Ph3_demod, ZTE Corporation, Sanechips.