3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514177

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on performance requirements for R19 A-IoT

Source: 	Huawei, HiSilicon

Agenda item:	6.14.3

Document for:	Discussion

Introduction

RRM performance requirements for Rel-19 A-IoT are discussed in RAN4#116, and the outcomes are captured in WF [1]. The following is agreed related to RRM test for A-IoT.

Issue 2-1-1: Test scenario Agreement: - The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test.

In this paper, we will provide our views on RRM test cases for R19 A-IoT.

Discussion

In RAN4#116, it is agreed that the random access and D2R transmit timing requirements will be tested in RAN4 test cases, but there are different options regarding whether to define dedicated RRM test cases or combine the test with RF/demod test.

Our view is that RAN4 should define a dedicated RRM test case for UL transmit timing, but RAN4 does not need to define dedicated RRM test case for random access.

General principle

Overall, unlike NR, the procedures and channels supported in R19 A-IoT are very limited. There is only one physical channel in DL (PRDCH) and only one in UL (PDRCH). Procedure wise, each D2R transmission is triggered by a corresponding R2D transmission, for both functions of inventory (including both CBRA and CFA) and command. This means we do not need to verify the same RRM performance for every procedure. Even in NR, the UL timing is tested with SRS and PRACH, and there is no test case to verify the transmit timing accuracy for PUSCH or PUCCH.

In addition, it should be considered that the cost of A-IoT device would be ultra-low, so the number of test cases should be minimized to keep the cost for testing low. In NR, performance test cases are defined separately in RAN4 in RF, RRM and demod sessions, and there are signaling test cases defined in RAN5. For A-IoT, we should try to avoid testing the same performance or procedure with different test cases from different RAN4 sessions and RAN5, and try to combine the test cases as much as possible, e.g. test RF and RRM requirements based on same procedure.

Random access

The core requirements for random access are focused on the procedure correctness and there is no particular performance requirement. Specifically, requirements are defined for the following procedures.

- CBRA: Device transmits Msg1 as response to paging

- CBRA: Device transmits R2D data as response to Msg2

- CBRA: Device does not transmit R2D as response to no Msg2

- CFA: Device transmits R2D data as response to paging

In NR, there is dedicated RRM test case for random access, and besides procedure correctness, transmit power and timing of PRACH are also tested. For R19 A-IoT, the UL transmission is based on backscattering and there is no power control, so transmit power accuracy cannot be tested. The transmit timing accuracy will be tested in the dedicated RRM test case.

Therefore, we do not see clear points to define dedicated RRM test case for random access. The correctness of the 4 random access procedures above can be guaranteed by RAN5 signaling test, which will provide a more complete coverage over different procedures and configurations than RRM test case.

Proposal 1: RAN4 does not define dedicated RRM test case for random access.

Transmit timing

We suggest to define a dedicated RRM test case to verify the UL transmit timing using CFA procedure. The reason to select CFA is that it has been agreed in RF session [2] to use CFA for refsens test.

Issue 3-4-1: Reference sensitivity Agreement: - EIS at peak antenna gain direction is -34dBm. Additional peak EIS levels can be added in the future if needed - EIS spherical coverage requirement is defined as sensitivity over partial sphere, i.e. XdBm at Y solid angle range partial sphere . Y is suggested to be +-45 degree with respect to the bore sight direction (or UE declared direction), X is suggested to be 5.5dB worse than peak EIS. - For testing - 90% success rate. - Use CFRA for REFSENS test procedure - Allow set time + SFO for the device response time. - Leave number of repeats to reach 90% success rate for the RAN5 to determine. - Further discuss if false alarm test can be considered in demod. - FRC is listed as below (note: it does not mean UE should be tested for all specified FRC)

RF session has also agreed the FRC, which includes the configurations, for R2D transmission, and we understand that they will also discuss and agree the FRC for R2D transmission for the refsens test in next meeting. Using CFA for RRM allows us to re-use the conclusions from RF session, and enables RAN5 to combine the RF and RRM performance test into a single test case.

Proposal 2: RAN4 to define a dedicated RRM test case for transmit timing using CFA procedure. The FRCs from RF session for refsens test can be re-used as much as possible.

Conclusions

In this paper we provided our views on RRM test cases for R19 A-IoT.

Proposal 1: RAN4 does not define dedicated RRM test case for random access.

Proposal 2: RAN4 to define a dedicated RRM test case for transmit timing using CFA procedure. The FRCs from RF session for refsens test can be re-used as much as possible.

Reference

- R4-2512134, WF on RRM for Ambient_IoT_Solutions, Moderator (CMCC)

- R4-2511859, WF on A-IOT device, CMCC