3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514178

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on RRM core requirements for Rel-19 IoT NTN

Source: 	Huawei, HiSilicon

Agenda item:	4.26.2

Document for:	Discussion

Introduction

RRM core requirements for Rel-19 IoT NTN are discussed in RAN4#116, and the outcomes are captured in WF [1]. In RAN2#131, an LS [2] is sent to RAN4 related to CQI reporting for CB-Msg3 EDT, which will impact RAN4 requirements.

In this paper, we will provide our views on remaining issues in RRM core requirements for IoT NTN.

Discussion

In [2], the following is informed.

In Release 19 IoT NTN, RAN2 is specifying support for Contention-Based Msg3 Early Data Transmission (CB-Msg3 EDT). In this scheme the UE can perform EDT without transmitting Msg1 and receiving Msg2, in other words directly transmitting Msg3 in a contention-based manner. As part of this, RAN2 has discussed whether CQI reporting in CB-Msg3 EDT procedure shall be supported or not. RAN2 has come to the following agreement, which applies to both NB-IoT and eMTC: We clarify that CQI reporting is not included in CB-Msg3 for CB-Msg3-EDT for non-anchor carrier but it can be included for anchor carrier (send LS to RAN4) RAN2 understands that RAN4 may potentially need to introduce some specification changes to support the above agreement.

In our view, the LS is related to the following requirements for NB-IoT.

#### 6.6A.2.6 MSG3-based channel quality report for UE Category NB1 The requirements in this clause shall apply for UE supporting DL channel quality reporting for UE Category NB1 as defined in TS 36.331 [2] section 5.3.3.3, 5.3.3.3a, and 5.3.7.4. …… For channel quality reporting in the anchor carrier, the reported NPDCCH repetition level shall be derived from the channel quality measured in the period T1 or T2 in the carrier where the random access response is transmitted, where - T1 is the period before NPRACH transmission used for NRSRP measurement for enhanced coverage level estimation - T2 is the period from the beginning of the random access response to the beginning of PUSCH format 1 for DL channel quality reporting. For channel quality reporting in the non-anchor carrier, the reported NPDCCH repetition level shall be derived from the channel quality measured in the period T2 in the carrier where UE monitors Random Access Response where T2 is defined above.

As there is no RAR before CB-Msg3, T2 is not applicable and UE can only measure the channel quality before CB-Msg3. However, T1 is based on PRACH which does not exist before CB-Msg3. In our view, there are 2 ways to clarify the requirements

- Option 1: extend definition of T1, e.g. T1 is the period before NPRACH or CB-Msg3 transmission

- Option 2: define separate requirements for Msg3 and CB-Msg3, and the requirements for CB-Msg3 are based on T3 which is the period before CB-Msg3 transmission.

We slightly prefer option 2 as it makes the requirements clearer.

It is noted that the updated (option 1) or new (option 2) requirements are only applicable for anchor carrier. As indicated in [2], channel quality report is not included in CB-Msg3 for non-anchor carrier.

Proposal 1: In 6.6A.2.6, define new requirements for channel quality reporting in CB-Msg3 on anchor carrier for NB-IoT, where the measurement is performed in period before CB-Msg3 transmission.

Conclusions

In this paper we provided our views on remaining issues in RRM core requirements for IoT NTN.

Proposal 1: In 6.6A.2.6, define new requirements for channel quality reporting in CB-Msg3 on anchor carrier for NB-IoT, where the measurement is performed in period before CB-Msg3 transmission.

Reference

- R4-2512148, WF on RRM requirements for IoT_NTN_Ph3, MediaTek

- R2-2506299, LS on CQI reporting for CB-Msg3 EDT, RAN2