3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514183

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on RRM impact of Rel-20 NTN enhancement

Source: 	Huawei, HiSilicon

Agenda item:	7.6.4.2

Document for:	Discussion

Introduction

In RAN#109, a new R20 WI on “Enhanced requirements for NR NTN and IoT NTN Phase 2” was approved. The WI include the following objective that involves RRM work that starts in this meeting.

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

In this paper, we will provide our views on RRM impact for supporting HD-FDD VSAT in FR1-NTN.

Discussion

The RRM impacts of HD-FDD have been discussed in R19 NR NTN WI for RedCap UE, mainly including

    - Clarification of requirement applicability in Paging reception, RA, Random access, Redirection, L1 indication for RLM and LR and scheduling restriction for L3 measurements

    - Handling of collision between CSI-RS based L1 measurement and PRS based measurement with UL transmission, based on collision handling mechanism defined by RAN1 in R19

In R19 WI of Ku band NTN, the requirements for VSAT in FR1-NTN have been defined. By default, the requirements defined in R18 for FR2-NTN (Ka band) will apply for VSAT in FR1-NTN. Most of the FR2-NTN requirements are same as those defined for FR1-NTN, including all the above listed requirements. This means the RRM impacts of HD-FDD defined for RedCap UE also apply for VSAT in FR1-NTN.

The difference between requirements for FR1-NTN and FR2-NTN is mainly the HO requirements, where the blind HO requirements for inter-satellite HO would apply. As discussed in RAN4#116 for RedCap UE, SSB is always prioritized over UL transmission, and in our view same principle should also apply for VSAT in FR1-NTN. This means there is no impact from HD-FDD on the HO requirements for VSAT in FR1-NTN.

Proposal: RAN4 to confirm that RRM impacts of HD-FDD defined for RedCap UE also apply for VSAT in FR1-NTN.

Conclusions

In this paper we provided our views on RRM impact for supporting HD-FDD VSAT in FR1-NTN.

Proposal: RAN4 to confirm that RRM impacts of HD-FDD defined for RedCap UE also apply for VSAT in FR1-NTN.

Reference

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, Moderator (RAN4 VC, Qualcomm Incorporated)